| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0264 |
| --- | --- |

| Title: | CE3-1.7: Multiple-model LM with small block size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin (Foxconn) Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Marta Karczewicz (Qualcomm Inc.) | Email: | PoHanLin@fginnov.com aramasub@qti.qualcomm.com geertv@qti.qualcomm.com |
| Source: | Foxconn, Qualcomm Inc. |  |  |

_____________________________

- Abstract

This contribution provides the simulation results of sub-test CE3-1.7 on multi-model LM (MMLM). This test enables 3 MMLM modes: MMLM-TL, MMLM-L and MMLM-T with a small block size restriction. It is reported that when compared to VTM-4.0, the proposed method has a BD-rate performance of (−0.27%, −1.56%, −2.13%) for (Y, Cb, Cr) with 100% encoding time and 100% decoding time for the AI configuration, and BD-rate performance of (−0.13%, −1.29%, −1.75%) for (Y, Cb, Cr) with 100% encoding time and 100% decoding time for the RA configuration.

Test description

This contribution reports the coding efficiency of the MMLM method in sub-test CE3-1.7 in Core Experiment (CE) 3 [1]. MMLM is similar to CCLM but applies more than 1 model to predict a chroma block. In this contribution, three MMLM modes are used: two-model equivalents corresponding to each of CCLM-TL, CCLM-L and CCLM-T modes. For the multi-model derivation, the average of the luma reference samples is used to classify all the luma reference samples into two groups. Then, a linear model is derived for each group by the maximum and the minimum value; the model parameter derivation (for each group) is the same as that used for CCLM. For signalling, one additional flag is used to indicate whether an MMLM mode is signalled, and when an MMLM mode is indicated, an index is signalled to indicate the particular MMLM mode. The number of lines used for model derivation is the same as CCLM modes in VTM 4.0, i.e., 2 lines above and 3 lines left of the current block for luma, and 1 line each above and left of the current block for chroma. And when the top boundary of the current block shares the CTU boundary, only one luma line is used from above the block.  The 3 MMLM modes are only enabled for chroma blocks that contain more than 16 samples.

Simulation results

The following experiments were conducted using the VTM-4.0 software with the VTM configuration and the JVET common test conditions [2] to collect the BD-rate, encoding time and decoding time of the proposed techniques in the All Intra and Random Access configurations. The proposed method has a BD-rate performance of (−0.27%, −1.56%, −2.13%) for (Y, Cb, Cr) with 100% encoding time and 100% decoding time for the AI configuration, and BD-rate performance of (−0.13%, −1.29%, −1.75%) for (Y, Cb, Cr) with 100% encoding time and 100% decoding time for the RA configuration.

Table 1: Simulation results of the proposed combined test CE3-1.7

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | -2.54% | -4.73% | 100% | 100% |
| Class A2 | -0.11% | -0.82% | -0.87% | 100% | 100% |
| Class B | -0.21% | -1.92% | -2.25% | 100% | 101% |
| Class C | -0.27% | -1.80% | -2.13% | 100% | 99% |
| Class E | -0.02% | -0.41% | -0.57% | 100% | 100% |
| Overall | -0.27% | -1.56% | -2.13% | 100% | 100% |
| Class D | -0.08% | -0.99% | -1.07% | 100% | 100% |
| Class F | -0.37% | -1.59% | -2.19% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -1.67% | -3.35% | 100% | 100% |
| Class A2 | -0.05% | -0.77% | -0.60% | 99% | 100% |
| Class B | -0.06% | -1.70% | -2.00% | 100% | 100% |
| Class C | -0.05% | -0.88% | -1.10% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -1.29% | -1.75% | 100% | 100% |
| Class D | -0.02% | -1.09% | -0.92% | 99% | 100% |
| Class F | -0.16% | -1.03% | -1.36% | 100% | 100% |

Conclusions

The results show that the proposed method provides significant improvement to coding efficiency in terms of luma and chroma BD-rate with negligible increase in EncT/DecT. It is suggested to adopt the proposed tool as part of VVC.

Reference

- G. Van der Auwera, L. Li, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, the 13th JVET meeting, Jan. 2019, Marrakech, MA.

- F. Bossen, J. Boyce, X. Li ,V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakech, MA.

Acknowledgements

We would like to thank Canon for cross-checking this CE test.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).