| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0266_r2 |
| --- | --- |

| Title: | CE2-related: Disabling bi-prediction or inter-prediction for small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC [1], bi-predicted 4*16, 16*4, 4*8 and 8*4 coding units (CU) and uni-predicted 4*4 CUs require much higher memory bandwidth than 8*8 bi-predicted CUs. This contribution provides the results of disabling inter-prediction for 4*4 CUs and bi-prediction for 4*N and N*4 CUs. It is reported that:

- Disable inter prediction for 4*4 CUs: there is 0.03% and 0.03% coding loss in RA and LDB Main10 configurations respectively.

- Disable inter prediction for 4*4 CUs and disable bi-prediction for 4*8 and 8*4 CUs: there is 0.08% and 0.11% coding loss in RA and LDB Main10 configurations respectively.

- Disable inter prediction for 4*4 CUs and disable bi-prediction for 4*8, 8*4, 4*16 and 16*4 CUs: there is 0.25% and 0.30% coding loss in RA and LDB Main10 configurations respectively.

# Introduction

In current VVC, bi-predicted 4*16, 16*4, 4*8 and 8*4 coding unit (CU) and uni-predicted 4*4 CU require higher memory bandwidth than 8*8 bi-predicted CU. Tool provided in JVET-M0862 [2] is used to analyze the memory bandwidth required by these CUs, and the results are summarized in Table 1 for YCbCr 4:2:0 case. Both luma and chroma components are considered in the analysis.

Compared with HEVC wherein 8*8 bi-predicted CU requires the highest memory bandwidth, the worst-case memory bandwidth of VVC is increased by around 46% and 100% for 2*1 memory pattern and 16*2 memory pattern respectively.

Table 1: Memory bandwidth comparison of different block sizes.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 29.38 | 146% | 72.00 | 200% |
| 8*4 Bi | 29.38 | 146% | 54.00 | 150% |
| 4*16 Bi | 22.69 | 112% | 54.00 | 150% |
| 16*4 Bi | 22.69 | 112% | 36.00 | 100% |
| 4*4 Uni | 21.38 | 106% | 54.00 | 150% |

# Proposed method

To suppress the problem mentioned above, this contribution test disabling bi-prediction or inter-prediction for small CUs. Specifically, we run the following tests:

- Test #1: disabling inter-prediction for 4*4 CU.

- Test #2: Test #1 + disabling bi-prediction for 4*8 and 8*4 CUs.

- Test #3: Test #1 + disabling bi-prediction for 4*8, 8*4, 4*16 and 16*4 CUs, including CUs coded in triangle mode.

In Test #2, the worst-case memory bandwidth of VVC is reduced to 112% and 150% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively. In Test #3, the worst-case memory bandwidth of VVC is reduced to that of the 8*8 bi-predicted CU.

# Working draft

## Working draft of proposed method

The working draft is described as following, which is on top of JVET-M1001-v6 [1]. The newly added parts are highlighted in yellow for test #1, yellow and green for test #2, yellow, green and turquoise for test #3, and the deleted parts of test #2 and test #3 are highlighted in yellow strikethrough.

7.3.6.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && tile_group_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( tile_group_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( tile_group_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ) ) ) && sps_ibc_enabled_flag && !( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 1 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| … |  |
| } |  |

#### 7.3.6.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 && ( cbWidth + cbHeight ) != 20 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.7.5	Coding unit semantics

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable CuPredMode[ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If pred_mode_flag is equal to 0, CuPredMode[ x ][ y ] is set equal to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1), CuPredMode[ x ][ y ] is set equal to MODE_INTRA.

When pred_mode_flag is not present, it is inferred to be equal to 1 when decoding an I tile group or when decoding a coding unit with cbWidth is equal to 4 and cbHeight is equal to 4, and equal to 0 when decoding a P or B tile group, respectively.

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred to be equal to the value of sps_ibc_enabled_flag when decoding an I tile group group or when decoding a coding unit coded in skip mode and cbWidth is equal to 4 and cbHeight is equal to 4, and 0 when decoding a P or B tile group, respectively.

When pred_mode_ibc_flag is equal to 1, the variable CuPredMode[ x ][ y ] is set to be equal to MODE_IBC for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to Table 7-9. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-9 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( cbWidth + cbHeight ) != 8 | ( cbWidth + cbHeight ) = = 8 ¦ ¦ ( cbWidth + cbHeight ) = = 12 ¦ ¦ ( cbWidth + cbHeight ) = = 20 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | n.a. |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

#### 8.5.2.1 General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

Let the variable LX be RefPicList[ X ], with X being 0 or 1, of the current picture.

For the derivation of the variables mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], refIdxL0 and refIdxL1, as well as predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the following applies:

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.5.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx.

- Otherwise, the following applies:

- For X being replaced by either 0 or 1 in the variables predFlagLX[ 0 ][0 ], mvLX[ 0 ][0 ] and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following ordered steps apply:

- The variables refIdxLX and predFlagLX[ 0 ][0 ] are derived as follows:

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xCb ][ yCb ]	(8-266)

predFlagLX[ 0 ][0 ] = 1		(8-267)

- Otherwise, the variables refIdxLX and predFlagLX[ 0 ][0 ] are specified by:

refIdxLX = −1		(8-268)

predFlagLX[ 0 ][0 ] = 0		(8-269)

- The variable mvdLX is derived as follows:

mvdLX[ 0 ] = MvdLX[ xCb ][ yCb ][ 0 ]	(8-270)

mvdLX[ 1 ] = MvdLX[ xCb ][ yCb ][ 1 ]	(8-271)

- When predFlagLX[ 0 ][ 0 ] is equal to 1, the derivation process for luma motion vector prediction in clause 8.5.2.8 is invoked with the luma coding block location ( xCb, yCb ), the coding block width cbWidth, the coding block height cbHeight and the variable refIdxLX as inputs, and the output being mvpLX.

- When predFlagLX[ 0 ][ 0 ] is equal to 1, the luma motion vector mvLX[ 0 ][ 0 ] is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 218 ) % 218	(8-272)

mvLX[ 0 ][ 0 ][ 0 ] = ( uLX[ 0 ]  >=  217 ) ? ( uLX[ 0 ] − 218 ) : uLX[ 0 ]	(8-273)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 218 ) % 218	(8-274)

mvLX[ 0 ][ 0 ][ 1 ] = ( uLX[ 1 ]  >=  217 ) ? ( uLX[ 1 ] − 218 ) : uLX[ 1 ]	(8-275)

NOTE 1– The resulting values of mvLX[ 0 ][ 0 ][ 0 ] and mvLX[ 0 ][ 0 ][ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

- The bi-prediction weight index gbiIdx is set equal to gbi_idx[ xCb ][ yCb ].

When all of the following conditions are true, refIdxL1 is set equal to −1, predFlagL1 is set equal to 0, and gbiIdx is set equal to 0:

- predFlagL0[ 0 ][ 0 ] is equal to 1.

- predFlagL1[ 0 ][ 0 ] is equal to 1.

- ( cbWidth + cbHeight = = 8 ) | | ( cbWidth + cbHeight = = 12 ) | | ( cbWidth + cbHeight = = 20 )

- cbWidth is equal to 4.

- cbHeight is equal to 4.

The updating process for the history-based motion vector predictor list as specified in clause 8.5.2.16 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and bi-prediction weight index gbiIdx.

#### 9.5.3.9	Binarization process for inter_pred_idc

Input to this process is a request for a binarization for the syntax element inter_pred_idc, the current luma coding block width cbWidth and the current luma coding block height cbHeight.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element inter_pred_idc is specified in Table 9-14.

Table 9-14 – Binarization for inter_pred_idc

| Value of inter_pred_idc | Name of inter_pred_idc | Bin string |  |
| --- | --- | --- | --- |
|  |  | ( cbWidth + cbHeight ) != 8 | ( cbWidth + cbHeight ) = = 8 ¦ ¦ ( cbWidth + cbHeight ) = = 12 ¦ ¦ ( cbWidth + cbHeight ) = = 20 |
| 0 | PRED_L0 | 00 | 0 |
| 1 | PRED_L1 | 01 | 1 |
| 2 | PRED_BI | 1 | - |

##### 9.5.4.2.1	General

Table 9 10 – Assignment of ctxInc to syntax elements with context coded bins

| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 8 && ( cbWidth + cbHeight ) != 12 && ( cbWidth + cbHeight ) != 20 ? 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ) : 4 | 4 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

## Working draft for removal of shared merge list in non-IBC mode

According to the discussion, working draft for removal of shared merge list in non-IBC mode is also provided. The deleted parts are highlighted in yellow strikethrough.

#### 7.4.7.2	Coding tree unit semantics

The CTU is the root node of the coding tree structure.

The array IsInSmr[ x ][ y ] specifying whether the sample at ( x, y ) is located inside a shared merging candidate list region, is initialized as follows for x = 0..CtbSizeY − 1 and y = 0..CtbSizeY − 1:

IsInSmr[ x ][ y ] = FALSE		(7-72)

alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] equal to 1 specifies that the adaptive loop filter is applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ). alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] equal to 0 specifies that the adaptive loop filter is not applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ).

When alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is not present, it is inferred to be equal to 0.

#### 7.4.7.4	Coding tree semantics

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-74)

SmrY[ x ][ y ] = y0		(7-75)

SmrW[ x ][ y ] = ( 1  <<  cbWidth )		(7-76)

SmrH[ x ][ y ] = ( 1  <<  cbHeight )		(7-77)

The variables allowSplitQt, allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, and allowSplitTtHor are derived as follows:

- The allowed quad split process as specified in clause 6.4.1 is invoked with the coding block size cbSize set equal to cbWidth, the location ( x0, y0 ), the and current multi-type tree depth mttDepth as inputs, and the output is assigned to allowSplitQt.

- The variables maxBtSize, maxTtSize and maxMttDepth are derived as follows:

- If treeType is equal to DUAL_TREE_CHROMA, maxBtSize, maxTtSize and maxMttDepth are set equal to MaxBtSizeC, MaxTtSizeC and MaxMttDepthC + depthOffset, respectively.

- Otherwise, maxBtSize, maxTtSize and maxMttDepth are set equal to MaxBtSizeY, MaxTtSizeY and MaxMttDepthY + depthOffset, respectively.

- The allowed binary split process as specified in clause 6.4.2 is invoked with the binary split mode SPLIT_BT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum binary tree size maxBtSize, and the current partition index partIdx as inputs, and the output is assigned to allowSplitBtVer.

- The allowed binary split process as specified in clause 6.4.2 is invoked with the binary split mode SPLIT_BT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum binary tree size maxBtSize, and the current partition index partIdx as inputs, and the output is assigned to allowSplitBtHor.

- The allowed ternary split process as specified in clause 6.4.3 is invoked with the ternary split mode SPLIT_TT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, and the maximum ternary tree size maxTtSize as inputs, and the output is assigned to allowSplitTtVer.

- The allowed ternary split process as specified in clause 6.4.3 is invoked with the ternary split mode SPLIT_TT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, and the maximum ternary tree size maxTtSize as inputs, and the output is assigned to allowSplitTtHor.

mtt_split_cu_binary_flag equal to 0 specifies that a coding unit is split into three coding units using a ternary split. mtt_split_cu_binary_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split.

When mtt_split_cu_binary_flag is not present, it is inferred as follows:

- If allowSplitBtVer is equal to FALSE and allowSplitBtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred to be equal to 0.

- Otherwise, if allowSplitTtVer is equal to FALSE and allowSplitTtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred as to be equal to 1.

- Otherwise, if allowSplitBtHor is equal to TRUE and allowSplitTtVer is equal to TRUE, the value of mtt_split_cu_binary_flag is inferred to be equal to !mtt_split_cu_vertical_flag.

- Otherwise (allowSplitBtVer is equal to TRUE and allowSplitTtHor is equal to TRUE), the value of mtt_split_cu_binary_flag is inferred to be equal to mtt_split_cu_vertical_flag.

The variable MttSplitMode[ x ][ y ][ mttDepth ] is derived from the value of mtt_split_cu_vertical_flag and from the value of mtt_split_cu_binary_flag as defined in Table 7-6 for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

Figure 7-1 – Multi-type tree spliting modes indicated by MttSplitMode (informative)

MttSplitMode[ x0 ][ y0 ][ mttDepth ] represents horizontal and vertical binary and ternary splittings of a coding unit within the multi-type tree as illustrated in Figure 7-1. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-6 – Specification of MttSplitMode[ x ][ y ][ mttDepth ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1

| MttSplitMode[ x0 ][ y0 ][ mttDepth ] | mtt_split_cu_vertical_flag | mtt_split_cu_binary_flag |
| --- | --- | --- |
| SPLIT_TT_HOR | 0 | 0 |
| SPLIT_BT_HOR | 0 | 1 |
| SPLIT_TT_VER | 1 | 0 |
| SPLIT_BT_VER | 1 | 1 |

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- One of the following conditions is true:

- mtt_split_cu_binary_flag is equal to 1 and  cbWidth * cbHeight / 2 is less than 32

- mtt_split_cu_binary_flag is equal to 0 and  cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-78)

SmrY[ x ][ y ] = y0		(7-79)

SmrW[ x ][ y ] = cbWidth		(7-80)

SmrH[ x ][ y ] = cbHeight		(7-81)

#### 7.4.7.5	Coding unit semantics

When all the following conditions are true, the history-based motion vector predictor list for the shared merging candidate list region is updated by setting NumHmvpSmrCand equal to the NumHmvpCand, and setting HmvpSmrCandList[ i ] equal to HmvpCandList[ i ] for i = 0..NumHmvpCand − 1:

- IsInSmr[ x0 ][ y0 ] is equal to TRUE

- SmrX[ x0 ][ y0 ]  is equal to x0

- SmrY[ x0 ][ y0 ]  is equal to y0

The following assignments are made for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

CbPosX[ x ][ y ] = x0		(7-82)

CbPosY[ x ][ y ] = y0		(7-83)

CbWidth[ x ][ y ] = cbWidth		(7-84)

CbHeight[ x ][ y ] = cbHeight		(7-85)

#### 8.5.2.2	Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-276)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-277)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-278)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-279)

smrNumHmvpCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-280)

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.5.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, the luma coding block height cbHeight set equal to smrWidth and smrHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-281)

predFlagL0Col = availableFlagL0Col		(8-282)

predFlagL1Col = 0		(8-283)

- When tile_group_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, the luma coding block height cbHeight set equal to smrWidth and smrHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-284)

predFlagL1Col = availableFlagL1Col		(8-285)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-286)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and smrNumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than MaxNumMergeCand and greater than 1, the following applies:

- The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index gbiIdx of candidate avgCand being added into mergeCandList is set equal to 0.

- numOrigMergeCand is set equal to numCurrMergeCand.

#### 8.5.2.6	Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

The array smrHmvpCandList and the variable smrNumHmvpCand are derived as follows:

smrHmvpCandList = isInSmr  ?  HmvpSmrCandList  :  HmvpCandList	(8-353)

smrNumHmvpCand = isInSmr  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-354)

For each candidate in smrHmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = smrHmvpCandList[ smrNumHmvpCand − hMvpIdx ] 	(8-355)

# Experimental results

The proposed method was implemented on top of reference software VTM-4.0 [3], and simulations were performed following the common test conditions [4]. The results are shown in Table 2~Table 4.

Table 2: Coding performance of Test #1.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.02% | 0.02% | 0.19% | 99% | 100% |  |  |  |  |  |
| Class B | 0.03% | 0.11% | 0.16% | 98% | 100% | 0.03% | 0.00% | 0.06% | 100% | 99% |
| Class C | 0.06% | 0.49% | 0.47% | 97% | 99% | 0.08% | 0.48% | 0.25% | 97% | 94% |
| Class E |  |  |  |  |  | -0.02% | 0.27% | -0.51% | 100% | 98% |
| Overall | 0.03% | 0.19% | 0.22% | 98% | 100% | 0.03% | 0.23% | -0.02% | 99% | 97% |
| Class D | 0.16% | 0.69% | 0.56% | 96% | 92% | 0.24% | 0.77% | 0.39% | 99% | 100% |
| Class F | 0.13% | 0.31% | 0.29% | 98% | 98% | 0.28% | 1.06% | 0.99% | 98% | 98% |

Table 3: Coding performance of Test #2.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.08% | 99% | 100% |  |  |  |  |  |
| Class A2 | 0.05% | 0.24% | 0.33% | 98% | 100% |  |  |  |  |  |
| Class B | 0.07% | 0.29% | 0.34% | 97% | 98% | 0.08% | 0.12% | 0.18% | 99% | 101% |
| Class C | 0.17% | 0.52% | 0.67% | 95% | 94% | 0.26% | 0.82% | 0.69% | 97% | 94% |
| Class E |  |  |  |  |  | -0.05% | 0.25% | -0.49% | 99% | 96% |
| Overall | 0.08% | 0.27% | 0.37% | 97% | 98% | 0.11% | 0.39% | 0.18% | 98% | 97% |
| Class D | 0.37% | 1.21% | 0.86% | 95% | 92% | 0.42% | 0.58% | 0.85% | 98% | 97% |
| Class F | 0.21% | 0.43% | 0.41% | 96% | 95% | 0.23% | -0.01% | 0.10% | 97% | 97% |

Table 4: Coding performance of Test #3.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.25% | 98% | 100% |  |  |  |  |  |
| Class A2 | 0.22% | 0.61% | 0.80% | 98% | 99% |  |  |  |  |  |
| Class B | 0.24% | 0.61% | 0.61% | 97% | 100% | 0.24% | 0.51% | 1.20% | 98% | 98% |
| Class C | 0.49% | 1.12% | 1.23% | 95% | 98% | 0.55% | 1.22% | 1.35% | 94% | 93% |
| Class E |  |  |  |  |  | 0.07% | -0.06% | -0.48% | 98% | 97% |
| Overall | 0.25% | 0.63% | 0.74% | 97% | 99% | 0.30% | 0.60% | 0.83% | 97% | 96% |
| Class D | 0.80% | 1.60% | 1.60% | 94% | 91% | 0.86% | 1.73% | 1.04% | 94% | 97% |
| Class F | 0.32% | 0.59% | 0.56% | 97% | 97% | 0.45% | 0.93% | 0.71% | 96% | 99% |

# Conclusions

This contribution tests disabling bi-prediction and inter-prediction for small CUs. In Test #2 and Test #3, the worst-case memory bandwidth of VVC is reduced dramatically and the coding performance loss is relatively small. It is recommended to adopt it into VVC.

# References

- “Versatile Video Coding (Draft 4)”, B. Bross, J. Chen, S. Liu, Doc. JVET-M1001, Jan. 2019.

- “BoG report on CE2 related contributions”, Y. He, Doc. JVET-M0862, Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).