| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0268 |
| --- | --- |

| Title: | CE2-related: Remove above-left affine inherited motion vector predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to remove the above-left affine inherited motion vector predictor. In the memory optimized implementation, the proposed method can reduce one third of the memory requirement of affine motion inheritance. The Y, U, V BD-rate changes are 0.01%, -0.01%, 0.00% in RA and -0.03%, 0.10%, -0.26% in LDB.  When the proposed method is extended to normal inter-prediction mode and intra block copy, The Y, U, V BD-rate changes are 0.01%, -0.01%, 0.00% in RA and -0.01%, 0.13%, -0.09% in LDB.

# Introduction

In the current design of affine motion prediction, the inherited affine motion predictor can be derived from left, above, or above-left neighboring coded blocks. According to JVET-L0047, the control-point motion vectors and other affine inheritance related information are stored in left-column, top-row, and top-left contexts in the memory-optimized implementation.

In this contribution, it’s proposed to remove affine inheritance from the above-left neighbor, so that 1/3 of storage, i.e. the storage of top-left context, for the affine motion inheritance can be saved. The proposed method can also be extended to remove the above-left motion predictor for normal inter-prediction mode and intra block copy.

# Test result

The proposed method is implemented on top of VTM-4.0. Two tests are conducted under random access and low delay B common test conditions. In test 1, the proposed method is applied to affine motion only, the overall Y, U, V, BD-rate changes are 0.01%, -0.01%, 0.00% in RA and -0.03%, 0.10%, -0.26% in LDB. In test 2, the proposed method is extended to all inter prediction mode and intra block copy. The detailed test results are shown in Table 1 and Table 2.

Table 1 Test 1 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | 0.03% | 99% | 99% |
| Class A2 | 0.00% | -0.03% | 0.01% | 100% | 101% |
| Class B | 0.01% | -0.05% | 0.06% | 102% | 101% |
| Class C | 0.02% | -0.03% | -0.11% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.02% | 0.17% | -0.04% | 99% | 99% |
| Class F | -0.01% | 0.07% | 0.00% | 100% | 99% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.10% | -0.51% | 100% | 102% |
| Class C | -0.04% | 0.09% | -0.18% | 100% | 99% |
| Class E | -0.08% | 0.12% | 0.06% | 100% | 102% |
| Overall | -0.03% | 0.10% | -0.26% | 100% | 101% |
| Class D | -0.01% | 0.26% | -1.06% | 100% | 96% |
| Class F | -0.06% | 0.16% | 0.56% | 100% | 100% |

Table 2 Test 2 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 0.03% | -0.03% | -0.01% | 100% | 100% |
| Class B | 0.04% | 0.00% | -0.01% | 103% | 102% |
| Class C | 0.00% | 0.02% | -0.09% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.04% | -0.08% | 100% | 101% |
| Class F | 0.00% | -0.05% | -0.11% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.13% | -0.09% | 100% | 103% |
| Class C | -0.04% | 0.17% | 0.12% | 100% | 101% |
| Class E | 0.00% | 0.53% | -0.37% | 100% | 99% |
| Overall | -0.01% | 0.13% | -0.09% | 100% | 101% |
| Class D | -0.03% | 0.51% | -0.62% | 101% | 98% |
| Class F | 0.03% | -0.11% | -0.24% | 102% | 101% |

# Conclusion

In this contribution, it’s shown that removing above-left MVP doesn’t degrade the bd-rate performance. It’s suggested to consider it for adoption in the next version of VTM.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).