| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0269 |
| --- | --- |

| Title: | CE4-4.4: Merge list construction for triangular prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The current merge list for Triangular prediction mode (TPM) requires accessing spatial and temporal blocks to obtain regular merge candidates with full pruning. The obtained regular merge candidates are further traversed 4 times to derive the uni-prediction TPM candidates, and full pruning is applied to each TPM candidate. In this contribution, the partial regular merge list (for non-sub-block and non-TPM) including spatial, temporal and HMVP candidates are reused with a limitation of pruning operations among TPM candidates. Therefore, the regular merge candidates need only to be traversed once. Simulation results reportedly show that the proposed method achieves 0.02% BD rate reduction for RA Main10 configurations and 0.08% coding loss for LDB Main10 configuration.

# Introduction

The Triangular Prediction Mode (TPM) was adopted in the 3rd VVC meeting [1]. In TPM, one merge list with TPM candidates is constructed and shared for two partitions. For each TPM candidate, it is restricted to be uni-prediction. While for the non-sub-block merge mode, a regular merge candidate list is constructed with each regular motion candidate could be either uni- or bi-prediction.

The construction process of the TPM merge list could be done with the following steps:

- Obtain regular motion candidates from 5 spatial neighboring blocks with full pruning (corresponding to block 1-5 in Figure 1)

- Obtain regular merge candidates from two temporal blocks (corresponding to block 6-7 in Figure 1).

Full pruning for the following four loops.

- Loop #1: For each regular motion candidates, if it is uni-prediction (either from List 0 or List 1), it is directly added to the merge list as an TPM candidate.

- Loop #2: For each motion candidates, if it is bi-prediction, the motion information from List 0 is added as a new TPM candidate.

- Loop #3: For each motion candidates, if it is bi-prediction, the motion information from List 1 is added as a new TPM candidate.

- Loop #4: For each motion candidates, if it is bi-prediction, the motion information of List 1 is firstly scaled to List 0 reference picture, and the average of the two MVs is added as a new TPM candidate.

- Add default motion vector candidates

Once the number of available merge candidates reaches the maximumly allowed value (e.g., 5 in current VTM-3.0), there is no need to further add the remaining merge candidates.

It should also be noted that the available regular merge candidates may be traversed four times to derive TPM candidates. In addition, whenever adding a TPM candidate to the TPM merge list, full pruning is applied. The pruning operation in the TPM candidate list is also different from that used for the regular merge list. Therefore, the complexity is relatively high.

Figure 1. Position of the neighboring blocks

# Proposed methods

To simplify the TPM merge list construction process, partial of the regular merge list construction process is reused. In addition, limited pruning is introduced when generating the TPM merge list.

The regular merge list construction process (before invoking pairwise bi-predication) is fully reused. That is, a regular motion candidate list is constructed and each regular motion candidate, after pruning if needed, is used to construct the TPM candidate list. More details of the TPM candidate list construction process is defined as follows:

- Reuse the regular merge list construction process to get valid regular merge candidates after pruning among regular merge candidates, denoted by RC0, RC1….

- For each regular merge candidate RCi, the following apply:

  - if it is uni-prediction, add it if i is no smaller than Th or not identical with the previous one TPM candidate

  - Otherwise (if bi-prediction),

    - Add one TPM candidates with list 0 motion information.

    - Add one TPM candidates with list 1 motion information if i is no smaller than Th or not identical with the previous one TPM candidate.

- If TPM list is not full, zero motion candidates are added.

In above description, the variable Th controls the number of pruning operations among TPM candidates. In this contribution, Th is set to 4.

# Summary of comparisons of three solutions

Table 1 summarizes the main differences of three solutions, as well as complexity analysis for the worst case. In addition, the complexity analysis data for the current VTM-4.0 design is also tabulated. It should be noted that for solution #1, the number of pruning operations could be further reduced if some simplifications tools are adopted for regular merge list construction process in the future.

Table 1. Analysis of pruning operations for different merge candidate category (for the worst case)

| Solutions | Total No. of Bi + Uni pruning operations | No. of TMVP | No. of loops for traversing regular merge candidates |
| --- | --- | --- | --- |
| #1 | 8 (for regular merge list) + 3 | Up to 1 | 1 |
| VTM4.0 | 10 + 46 | 2 | 4 |

# Experimental Results

The proposed method is implemented on VTM-4.0 [2]. The simulations were performed following the JVET CTC [3]. Results of Random Access (RA) configuration and Low-delay B (LB) configuration are summarized in the Table 2. More detail results can be found in the attached spreadsheets.

Table 2. Results of proposed method compared to VTM-4.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.03% | 100% | 99% |  |  |  |  |  |
| Class A2 | -0.07% | -0.20% | -0.11% | 99% | 100% |  |  |  |  |  |
| Class B | -0.02% | -0.07% | -0.10% | 100% | 100% | 0.09% | 0.02% | -0.18% | 100% | 100% |
| Class C | 0.01% | -0.07% | -0.19% | 101% | 101% | 0.16% | 0.42% | 0.18% | 100% | 100% |
| Class E |  |  |  |  |  | 0.00% | 0.18% | -0.71% | 101% | 102% |
| Overall | -0.02% | -0.09% | -0.11% | 100% | 100% | 0.09% | 0.20% | -0.19% | 100% | 100% |
| Class D | -0.01% | -0.14% | -0.02% | 102% | 100% | 0.13% | 0.36% | -0.35% | 101% | 100% |
| Class F | 0.04% | -0.03% | -0.01% | 102% | 100% | -0.13% | 0.16% | -0.06% | 103% | 100% |

# Conclusions

In this contribution, a simplification method of TPM merge list construction process is provided. With the proposed method, there are minor coding gains under the RA configurations with much lower complexity requirement. It is recommended to consider adoption of it to the next version of VTM.

# Reference

- “CE10.3.1.b: Triangular prediction unit mode”,	R.-L. Liao, C. S. Lim (Panasonic), Doc. JVET-L0124, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).