| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0272-v1 |
| --- | --- |

| Title: | CE2-5.4: Parameter-based affine model inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to store affine parameters for affine inheritance, instead of CPMVs, CU top-left coordinate and CU dimensions. With the proposed method, it is asserted that the In-CTU buffer required by affine inheritance is reduced from 7689 bits to 2112 bits (27.5% of the original size). In addition, the derivations of affine parameters and CPMVs for affine inheritance merge mode are removed. Simulation results reportedly show 0.02% and 0.00% BD-rate changing under RA and LB configurations, respectively, with almost no change on encoding/decoding time.

# Introduction

In VTM-4.0, when the affine model of the current CU is inherited from a neighbouring 4×4 block in the current CTU row, control point motion vectors (CPMVs) stored in a special CPMV buffer are accessed. The current CU can inherit the 4-parameter or the 6-parameter affine model from the neighbouring CU. This design has two problems:

- In-CTU buffer size is increased.

For a 4×4 block inside a CTU, three CPMVs for two reference lists, the width, the height and the top-left coordinate of the CU containing the 4×4 block must be stored. Therefore, 2×3 bits for width and height, 2×5 bits for the coordinate, one bit to indicate 4-parameter or 6-parameter affine, and 2×3×2×18 bits for CPMVs, are required to be stored for each 8×8 block inside a CTU. Totally, the In-CTU buffer required by affine inheritance is 33×(2×3×2×18+6+10+1)= 7689 bits.

- Redundant computation.

First, the affine parameters are unnecessarily derived several times. The affine parameters are calculated for an affine-code CU when it is coded. And those parameters are re-calculated whenever coding a proceeded block which inherits affine model from this block. The affine parameters are unnecessarily calculated twice: from the CPMVs of the neighbouring CU and from the CPMVs of the current CU for the affine-inheritance merge mode. Second, the derivation of the CPMVs of the current CU is unnecessary for the affine-inheritance merge mode. The MVs of sub-blocks should be calculated directly.

# Proposed methods

Parameter-based affine model inheritance was proposed in JVET-M0270 [1].

It is proposed to store affine parameters instead of three CPMVs, the top-left coordinate and the block dimensions. When the current CU applies affine-inheritance merge mode, affine parameters are directly copied from the neighbouring 4×4 block B to be inherited. Then the MV of each sub-block in the current CU is derived with the center position of B as the base position, and the MV of block B (denoted as MVB) as the base MV. When the current CU applies affine-inheritance AMVP mode, CPMVs of the current CU are derived also with the center position of B as the base position and MVB as the base MV. Then the derived CPMVs will serve as the affine MVPs.

Each parameter is stored as an 8-bit signed integer. So 2×4×8 = 64 bits are required to be store affine parameters in each 8×8 block inside a CTU. Therefore, the In-CTU-buffer required by affine inheritance is 33×64 = 2112 bits. One set of affine parameters are calculated at most only once, and CPMVs are not calculated with the affine inheritance merge mode.

Table 1 shows the comparison on storage and computation between VTM-4.0 and the proposed method.

Table 1: Summary of storage/computation analysis

|  | In-CTU buffer Size | Derivation of CPMVs for affine inheritance merge | Derivation of affine parameters for affine inheritance merge | Derivation of affine parameters for affine inheritance inter |
| --- | --- | --- | --- | --- |
| VTM-4.0 | 7689 bits | 1 | 2 | 2 |
| Proposed | 2112 bits | 0 | 0 | 1 |
| Proposed/VTM-4.0 | 27.5% | 0% | 0% | 50% |

# Experimental Results

The proposed method has been implemented on top of VTM-4.0. Results under RA/LB configurations are summarized in Table 2. The simulations were performed following the JVET CTC [2]. More detail results can be found in the attached spreadsheets.

Table 2: Results for the CE2-5.4

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.05% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.04% | -0.01% | 0.07% | 99% | 99% |  |  |  |  |  |
| Class B | 0.01% | -0.06% | 0.02% | 100% | 100% | -0.02% | -0.25% | -0.19% | 99% | 98% |
| Class C | 0.02% | -0.01% | -0.01% | 100% | 101% | -0.01% | 0.30% | -0.01% | 100% | 103% |
| Class E |  |  |  |  |  | 0.06% | 0.11% | -0.23% | 101% | 100% |
| Overall | 0.02% | -0.04% | 0.01% | 100% | 100% | 0.00% | 0.02% | -0.14% | 100% | 100% |
| Class D | -0.01% | -0.07% | 0.00% | 100% | 103% | -0.01% | 0.24% | -0.78% | 101% | 100% |
| Class F | 0.02% | 0.09% | 0.01% | 101% | 100% | 0.07% | -0.12% | 0.29% | 100% | 98% |

# Conclusions

With the proposed method, the In-CTU memory size is saved by 75%. Meanwhile, some sophisticated steps are removed. It is suggested adopting this technology to VVC.

# References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “CE2-related: An alternative storing method for affine inheritance,”, JVET-M0270, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).