| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0273 |
| --- | --- |

| Title: | CE2-related: Affine motion compensation using 2x2 subblock |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, it’s proposed to adaptively use 2x2 or 4x4 subblock based motion compensation for affine uni-prediction.  The adaptation is based on the affine parameters and ensure that the proposed method doesn’t increase worst case memory bandwidth. The proposed method is only applied to uni-prediction, it also doesn’t increase the worst-case computational complexity for interpolation. The test results show that the BD-rate changes are -0.34%, -0.05%, 0.02% in RA and -0.18%, -0.01%, -0.19% in LDB

# Introduction

In current VTM, the affine motion compensation is performed in 4x4 subblock basis. In JVET-M0268 [1], the interweaved prediction for affine motion compensation provides coding gains by taking advantage of more dense motion field and weighted prediction. It in this contribution, it’s shown that better gain can be achieved by using smaller subblock size (2x2) for uni-prediciton without increasing worst case bandwidth and computational complexity.

In the proposed method, the 2x2 subblock based affine motion compenstion is only applied to uni-prediction of luma samples. And the 2x2 subblock motion field is only used for motion compensation. The storage of motion vector field for motion prediction, etc., is still 4x4 subblock based. If bandwidth constrain is applied, the 2x2 subblock based affine motion compensation is disabled when the affine motion parameters don’t satisfy certain criterion.

# Derivation of 2x2 subblock motion field

The 2x2 subblock motion field is derived based on the control-point motion vectors of current block and the coordinate of each subblock, similarly to the derivation of 4x4 subblock motion field. Therefore, the motion field need to be recalculated if 2x2 subblock motion compensation is applied.

# Bandwidth constrains

Bandwidth constrain can be applied to ensure that 2x2 subblock motion compensation doesn’t increase the worst-case bandwidth. The 2x2 subblock based motion compensation is only enabled if the affine parameters satisfy the bandwidth constrain. In this contribution, the bandwidth constrains ensure that bandwidth of the 2x2 subblock motion compensation for a 8x8 block is less than or equal to that of 15x15. The 2x2 subblock is enable if the following is true:

(6*Normv1x-v0x> -9*pel &&+6*Normv1x-v0x < 3*pel) &&

(6*Normv1y-v0y > -3*pel && 6*Normv1y-v0y < 9*pel) &&

6*Normv2x-v0x> -3*pel &&6*Normv2x-v0x < 9*pel &&

6*Normv2y-v0y> -9*pel &&6*Normv2y-v0y < 3*pel &&

((6*Normv1x-v0x + 6*Normv2x-v0x> -9*pel) &&

(6*Normv1x-v0x + 6*Normv2x-v0x< 3*pel)) &&

((6*Normv1y-v0y + 6*Normv2y-v0y> -9*pel) &&

(6*Normv1y-v0y + 6*Normv2y-v0y< 3*pel))

Wherein (vix, viy), i=0,1,2 are the 3 control point motion vectors, and

Normv1x-v0x=v1x-v0x*128/w

Normv1y-v0y=v1y-v0y*128/w

Normv2x-v0x=v2x-v0x*128/h

Normv2x-v0x=v2x-v0x*128/h

w and h are the width and height of the current block, respectively.

# Complexity analysis

## motion compensation interpolation

In current VTM, the worst-case computational complexity for motion compensation is 4x4 bi-prediction. For 4x4 uni-prediction, it requires 4*(4+7) interpolations in horizontal direction and 4*4 interpolations in vertical direction. Therefore, the total number of interpolations for 4x4 bi-prediction is (4*11+4*4) *2 = 120, i.e. 7.5 interpolations / sample. For 2x2 uni-prediction, it requires 2*(2+7) interpolations in horizontal direction and 2*2 interpolations in vertical direction, 22 in total, i.e. 5.5 interpolations / sample. A 4x4 subblock using 2x2 uni-prediction needs 88 (22*4) interpolations.

Each interpolation requires 8 multiplications and 7 additions. Therefore, 4x4 bi-prediction requires 960 multiplications and 840 additions. The 2x2 uni-prediction for a 4x4 block requires 704 multiplications and 616 additions.

## Motion field calculation

The additional operations for motion field calculation is 16 (4*4) multiplications and 16 (4*4) additions per 4x4 subblock. Some optimized implementation can be applied. In the optimized method, it requires 4 multiplications and 4 additions for the MV of the first 2x2 block, and 4 additions for each of the remaining 3 2x2 blocks. The MV field calculation can remain in parallel for each 4x4 subblock. The additional operations per 4x4 subblock are 4 multiplications and 16 additions. Therefore, the total operations for 2x2 subblock uni-prediction are 708 multiplications and 632 additions.

# Test result

The test results under random access and low delay B common test conditions are shown in Table 1. The overall Y, U, V, BD-rate changes are -0.34%, -0.05%, 0.02% in RA and -0.18%, -0.01%, -0.19% in LDB. Runtime is not accurate except for Class C and D in RA.

Table 1 Test results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.07% | 0.09% | 103% | 100% |
| Class A2 | -0.57% | 0.14% | 0.07% | 105% | 102% |
| Class B | -0.48% | -0.28% | -0.03% | 105% | 103% |
| Class C | -0.17% | 0.00% | 0.00% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.05% | 0.02% | 104% | 102% |
| Class D | -0.38% | -0.16% | -0.08% | 101% | 102% |
| Class F | -0.65% | -0.33% | -0.31% | 106% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.14% | -0.05% | 103% | 105% |
| Class C | -0.18% | 0.11% | -0.10% | 103% | 102% |
| Class E | -0.20% | 0.06% | -0.53% | 105% | 102% |
| Overall | -0.18% | -0.01% | -0.19% | 104% | 103% |
| Class D | -0.19% | 0.52% | -0.24% | 102% | 102% |
| Class F | -1.02% | -0.89% | -0.84% | 102% | 103% |

# Conclusion

This contribution shows that the affine uni-prediction can be improved without increasing the worst-case bandwidth and computational complexity by adaptively choosing smaller subblock size. It’s suggested to study trade-off between subblock size and BD-rate performance for affine motion compensation in CE.

# References

# K. Zhang, L. Zhang, H. Liu, et al. “Non-CE2: Interweaved Prediction for Affine Motion Compensation”, JVET-M0268, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).