The following spec is based on version 9 of JVET-L1001.The added syntax is highlighted in yellow.

In 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_pcm_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | ue(v) |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_tile_groups | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_groups_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_tile_groups_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_tile_groups != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_tile_group | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_tile_group | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_groups_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_tile_groups_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma | ue(v) |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_gbi_enabled_flag | u(1) |
| sps_cpr_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| sps_reshaper_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In 7.3.3.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( tile_group_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if ( sps_reshaper_enabled_flag ) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if ( tile_group_reshaper_model_present_flag ) |  |
| tile_group_reshaper_model ( ) |  |
| else if ( tile_group_type != I ) { |  |
| if( tile_group_type = = B ) |  |
| reshaper_model_from_l1_flag | u(1) |
| reshapter_mode_from_refidx | ue(v) |
| } |  |
| tile_group_reshaper_enable_flag | u(1) |
| if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Add a new syntax table tile group reshaper model:

| tile_group_reshaper_model () { | Descriptor |
| --- | --- |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | ue(v) |
| for ( i = reshaper_model_min_bin_idx; i <= reshaper_model_max_bin_idx; i++ ) { |  |
| reshape_model_bin_delta_abs_CW [ i ] | u(v) |
| if ( reshaper_model_bin_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_bin_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| } |  |

In General sequence parameter set RBSP semantics, add the following semantics:

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

In tile group header syntax, add the following semantics

tile_group_reshaper_model_present_flag equal to 1 specifies tile_group_reshaper_model() is present in tile group header. tile_group_reshaper_model_present_flag equal to 0 specifies tile_group_reshaper_model() is not present in tile group header. When tile_group_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

reshaper_model_from_l1_flag is 1 if the reshaping information is inherited from a reference picture in reference list 1. Otherwise, the reshaping information is inherited from a reference picture in reference list 0. The When not present, it should be inferred to be 0. If reshaper_model_from_l1_flag is not present, it is inferred to be 0.

reshapter_mode_from_refidx indicates the reference index of the reference picture from which the reshaping information is inherited. If reshapter_mode_from_refidx is not present, it is inferred to be 0.

tile_group_reshaper_enabled_flag equal to 1 specifies that reshaper is enabled for the current tile group. tile_group_reshaper_enabled_flag equal to 0 specifies that reshaper is not enabled for the current tile group. When tile_group_reshaper_enable_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the current tile group. tile_group_reshaper_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current tile group. When tile_group_reshaper_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

Add tile_group_reshaper_model( ) syntax

reshape_model_min_bin_idx specifies the minimum bin (or piece) index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to MaxBinIdx, inclusive. The value of MaxBinIdx shall be equal to 15.

reshape_model_delta_max_bin_idx specifies the maximum allowed bin (or piece) index MaxBinIdx minus the maximum bin index to be used in the reshaper construction process. The value of reshape_model_max_bin_idx is set equal to MaxBinIdx – reshape_model_delta_max_bin_idx.

In a conforming bit-stream, reshape_model_delta_max_bin_idx should be in the range from 0 to MaxBinIdx-reshape_model_min_bin_idx.

reshaper_model_bin_delta_abs_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax reshape_model_bin_delta_abs_CW[ i ].

In a conforming bit-stream, reshaper_model_bin_delta_abs_cw_prec_minus1 must no larger than 6.

reshape_model_bin_delta_abs_CW[ i ] specifies the absolute delta codeword value for the ith bin.

reshaper_model_bin_delta_sign_CW_flag[ i ] specifies the sign of reshape_model_bin_delta_abs_CW[ i ] as follows:

–	If reshape_model_bin_delta_sign_CW_flag[ i ] is equal to 0, the corresponding variable RspDeltaCW[ i ] is a positive value.

–	Otherwise ( reshape_model_bin_delta_sign_CW_flag[ i ] is not equal to 0 ), the corresponding variable RspDeltaCW[ i ] is a negative value.

When reshape_model_bin_delta_sign_CW_flag[ i ] is not present, it is inferred to be equal to 0.

The variable RspDeltaCW[ i ] = (1  2*reshape_model_bin_delta_sign_CW [ i ]) * reshape_model_bin_delta_abs_CW [ i ];

The variable RspCW[ i ] is derived as following steps:

The variable OrgCW is set equal to (1 << BitDepthY ) / ( MaxBinIdx + 1).

–	If reshaper_model_min_bin_idx < = i <= reshaper_model_max_bin_idx

RspCW[ i ] = OrgCW + RspDeltaCW[ i ].

–	Otherwise, RspCW[ i ] = 0.

The value of RspCW [ i ] shall be in the range of 32 to 2 * OrgCW - 1 if the value of BitDepthY is equal to 10.

The variables InputPivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive are derived as follows

InputPivot[ i ] = i * OrgCW

If tile_group_reshaper_model_present_flag is equal to 0 and the current tile_group_type is I-tile_group, then the reshaping information of the current tile group is initialized as below:

RspCW[ i ]=0, for i = 0, 1,…, MaxBinIdx.

If reshapter_mode_from_refidx is present, let refTileGroup be the first tile group of the reference picture with reference index reshapter_mode_from_refidx, in the reference list (reshaper_model_from_l1_flag? 1 : 0). Then the following applies:

RspCW[ i ] are set equal to RspCW[ i ] of refTileGroup for i = 0, 1,…, MaxBinIdx,

reshape_model_min_bin_idx is set equal to reshape_model_min_bin_idx of refTileGroup.

reshape_model_max_bin_idx is set equal to reshape_model_max_bin_idx of refTileGroup.

The variable ReshapePivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive, the variable ScaleCoef[ i ] and InvScaleCoeff[ i ]with i in the range of 0 to MaxBinIdx , inclusive, are derived as follows:

shiftY = 14
ReshapePivot[ 0 ] = 0;

for( i = 0; i <= MaxBinIdx ; i++) {

if RspCW[ i ] == 0

ScaleCoef[ i ] = InvScaleCoeff[ i ]= 1 << shiftY,

else
	ReshapePivot[ i + 1 ] = ReshapePivot[ i ] + RspCW[ i ]

ScaleCoef[ i ] = ( RspCW[ i ] * (1 << shiftY) + (1 << (Log2(OrgCW) - 1))) >> (Log2(OrgCW))
	if ( RspCW[ i ] == 0 )
		InvScaleCoeff[ i ] = 0
	else
		InvScaleCoeff[ i ] = OrgCW * (1 << shiftY) / RspCW[ i ]
}

The variable ChromaScaleCoef[ i ] with i in the range of 0 to MaxBinIdx , inclusive, are derived as follows:

ChromaResidualScaleLut[64] = {16384, 16384, 16384, 16384, 16384, 16384, 16384, 8192, 8192, 8192, 8192, 5461, 5461, 5461, 5461, 4096, 4096, 4096, 4096, 3277, 3277, 3277, 3277, 2731, 2731, 2731, 2731, 2341, 2341, 2341, 2048, 2048, 2048, 1820, 1820, 1820, 1638, 1638, 1638, 1638, 1489, 1489, 1489, 1489, 1365, 1365, 1365, 1365, 1260, 1260, 1260, 1260, 1170, 1170, 1170, 1170, 1092, 1092, 1092, 1092, 1024, 1024, 1024, 1024};

shiftC = 11

–	if ( RspCW[ i ] == 0 )

ChromaScaleCoef [ i ] = (1 << shiftC)

–	 Otherwise (RspCW[ i ] != 0),

ChromaScaleCoef[ i ] = ChromaResidualScaleLut[RspCW[ i ] >> 1]

Add the following in Weighted sample prediction process for combined merge and intra prediction. The addition is marked in yellow.

      - 8.4.6.6	Weighted sample prediction process for combined merge and intra prediction

Inputs to this process are:

- the width of the current coding block cbWidth,

- the height of the current coding block cbHeight,

- two (cbWidth)x(cbHeight) arrays predSamplesInter and predSamplesIntra,

- the intra prediction mode predModeIntra,

- a variable cIdx specifying the colour component index.

Output of this process is the (cbWidth)x(cbHeight) array predSamplesComb of prediction sample values.

The variable bitDepth is derived as follows:

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

- Otherwise, bitDepth is set equal to BitDepthC.

The prediction samples predSamplesComb[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are derived as follows:

- The weight w is derived as follows:

- If predModeIntra is INTRA_ANGULAR50, w is specified in Table 8-10 with nPos equal to y and nSize equal to cbHeight.

- Otherwise, if predModeIntra is INTRA_ANGULAR18, w is specified in Table 8-10 with nPos equal to x and nSize equal to cbWidth.

- Otherwise, w is set equal to 4.

- If cIdx is equal to 0, predSamplesInter is derived as following:

- If tile_group_reshaper_enabled_flag is equal to 1,

shiftY = 14

idxY = predSamplesInter[ x ][ y ] >> Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1Y ( ReshapePivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] *( predSamplesInter[ x ][ y ] - InputPivot[ idxY ] )
	 + ( 1 << ( shiftY – 1 ) ) ) >> shiftY )								(8-xxx)

- Otherwise ( tile_group_reshaper_enabled_flag is equal to 0 )

predSamplesInter [ x ][ y ] =  predSamplesInter [ x ][ y ]

- The prediction samples predSamplesComb[ x ][ y ] are derived as follows:

predSamplesComb[ x ][ y ] = ( w * predSamplesIntra[ x ][ y ] + 	(8-740)
 	( 8 − w ) * predSamplesInter[ x ][ y ] ) >> 3 )

Table 8-10 – Specification of w as a function of the position nP and the size nS

| 0 <= nP < ( nS / 4 ) | ( nS / 4 ) <= nP < ( nS / 2 ) | ( nS / 2 ) <= nP < ( 3 *nS / 4 ) | ( 3 *nS / 4 ) <= nP < nS |
| --- | --- | --- | --- |
| 6 | 5 | 3 | 2 |

Add the following in Picture reconstruction process

### 8.5.5	Picture reconstruction process

Inputs to this process are:

–	a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

–	the variables nCurrSw and nCurrSh specifying the width and height, respectively, of the current block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nCurrSw)x(nCurrSh) array predSamples specifying the predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the residual samples of the current block.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y.

–	Otherwise, if cIdx is equal to 1, recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C.

–	Otherwise (cIdx is equal to 2), recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C.

When the value of tile_group_reshaper_enabled_flag is equal to 1, the (nCurrSw)x(nCurrSh) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as the mapping process specified in clause 8.5.5.1. Otherwise, the (nCurrSw)x(nCurrSh) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.5.5.1	Picture reconstruction with mapping process

This clause specifies picture reconstruction with mapping process. The picture reconstructon with mapping process for luma sample value is specified in 8.5.5.1.1. The picture reconstructon with mapping process for chroma sample value is specified in 8.5.5.1.2.

### 8.5.5.1.1	Picture reconstructon with mapping process for luma sample value

Inputs to this process are:

–	an (nCurrSw)x(nCurrSh) array predSamples specifying the luma predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the luma residual samples of the current block.

The output for this process are:

–	an (nCurrSw)x(nCurrSh) mapped luma prediction sample array predMapSamples,

–	an (nCurrSw)x(nCurrSh) reconstructed luma sample array recSamples.

The predMapSamples is derived as follows:

–	If ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_INTRA ) || ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_CPR) || ( CuPredMode[ xCurr ][ yCurr ]  = =  MODE_INTER && mh_intra_flag[ xCurr ][ yCurr ] )

predMapSamples[ xCurr + i ][ yCurr + j ] =  predSamples[ i ][ j ]	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise ( ( CuPredMode[ xCurr ][ yCurr ] = = MODE_INTER && !mh_intra_flag[ xCurr ][ yCurr ] )), the following applies:

shiftY = 14
idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
predMapSamples[ xCurr + i ][ yCurr + j ] =  ReshapePivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] *(predSamples[ i ][ j ] - InputPivot[ idxY ] )
	 + ( 1 << ( shiftY – 1 ) ) ) >> shiftY 	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

The recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1Y ( predMapSamples[ xCurr + i ][ yCurr + j ]+ resSamples[ i ][ j ] ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.5.5.1.2	Picture reconstructon with mapping process for chroma sample value

Inputs to this process are:

–	an (nCurrSwx2)x(nCurrShx2) array mapped predMapSamples specifying the mapped luma predicted samples of the current block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

The output for this process is reconstructed chroma sample array recSamples.

The recSamples is derived as follows:

–	If ( !tile_group_reshaper_chroma_residual_scale_flag || ( (nCurrSw)x(nCurrSh) <= 4) )

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C ( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) 	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tile_group_reshaper_chroma_residual_scale_flag && ( (nCurrSw)x(nCurrSh) > 4)), the following applies:

The variable varScale is derived as follows:

- invAvgLuma = Clip1Y( ( ij predMapSamples[ (xCurr << 1 ) + i ][ (yCurr << 1) + j ]
 + nCurrSw * nCurrSh *2) / ( nCurrSw * nCurrSh *4 ) )

- The variable idxYInv is derived by involing the identification of piece-wise function index as specfied in clause 8.5.6.2 with the input of sample value invAvgLuma.

- varScale = ChromaScaleCoef[ idxYInv ]

The recSamples is derived as follows:

–	 If tu_cbf_cIdx [ xCurr ][ yCurr ] equal to 1, the following applies:

shiftC = 11
recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1 ( predSamples[ i ][ j ] + Sign( resSamples[ i ][ j ] )
 * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << ( shiftC – 1 ) ) ) >> shiftC ) )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tu_cbf_cIdx[ xCurr ][ yCurr ] equal to 0)

recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1(predSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.5.6	Picture inverse mapping process

This clause is invoked when the value of tile_group_reshaper_enabled_flag is equal to 1. The input is reconstructed picture luma sample array SL and the output is modified reconstructed picture luma sample array S′L after inverse mapping process.

The inverse mapping process for luma sample value is specified in 8.4.6.1.

### 8.5.6.1	Picture inverse mapping process of luma sample values

Inputs to this process is a luma location ( xP, yP ) specifying the luma sample location relative to the top-left luma sample of the current picture.

Outputs of this process is a inverse mapped luma sample value invLumaSample.

The value of invLumaSample is derived by applying the following ordered steps:

- The variables idxYInv is derived by invoking the identification of piece-wise function index as specified in clause 8.5.6.2 with the input of luma sample value SL[ xP ][ yP ].

- The value of reshapeLumaSample is derived as follows:

shiftY = 14
invLumaSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] *( SL[ xP ][ yP ] - ReshapePivot[ idxYInv ] )
	 + ( 1 << ( shiftY – 1 ) ) ) >> shiftY		(8-xx)

- clipRange = ((reshape_model_min_bin_idx > 0) && (reshape_model_max_bin_idx < MaxBinIdx));

–	If clipRange is equal to 1, the following applies:

minVal = 16 << (BitDepthY – 8)
maxVal = 235<< (BitDepthY – 8)
invLumaSample = Clip3(minVal, maxVal, invLumaSample)

–	else (clipRange is equal to 0),

invLumaSample = ClipCidx1(invLumaSample),

### 8.5.6.2	Identification of piecewise function index for luma components

Inputs to this process are a luma sample value S.

Output of this process is an index idxS identifing the piece to which the sample S belongs. The variable idxS is derived as follows:

for( idxS = 0, idxFound = 0; idxS <= MaxBinIdx; idxS++ ) {
	if( (S < ReshapePivot[ idxS + 1 ] ) {
		idxFound = 1
		break
	}
}

Note, an alternative implementation to find the identification idxS is as following:

if (S < ReshapePivot[ reshape_model_min_bin_idx ])
	idxS = 0
else if (S >= ReshapePivot[ reshape_model_max_bin_idx ])
	idxS  = MaxBinIdx
else
	idxS = findIdx ( S, 0, MaxBinIdx + 1, ReshapePivot[ ] )

function idx = findIdx (val, low, high, pivot[ ]) {
	if ( high – low <= 1 )
		idx = low

else {
		mid = ( low + high) >> 1
		if (val <  pivot [mid] )
			high = mid
 		else
 			low = mid
		idx = findIdx (val, low, high, pivot[])
	}

}