| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0274-v1 |
| --- | --- |

| Title: | Non-CE: Cleanups on in-loop reshaping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, three cleanups are proposed to address the syntax design of in-loop reshaping. First, the reshaping mapping is initialized to be an identity mapping when no reshaping information is presented for an I tile group. Second, the reshaping information is inherited from a reference picture which is signaled when no reshaping information is presented for a P/B tile group. Third, two constrains on syntax elements related to in-loop reshaping is proposed for conforming bit-stream. Simulation results reportedly show 0.00%, 0.01%, 0.0x% BD-rate changing under AI, RA and LB configurations, respectively, with almost no impact on encoding and decoding time.

# Introduction

There are three problems in the syntax design of in-loop reshaping.

- The syntax design allows in-loop reshaping to be applied when no valid reshaping information has been transmitted to the decoder. The behavior of a decoder is undefined in this case.

- The current tile group may use reshaping information signaled to the decoder in another tile group, which it cannot refer to. For example, it may use reshaping information signaled in a tile group which is transmitted before the last IDR tile group.

- The behavior of a decoder may be undefined because syntax elements are allowed to be signaled as below:

  - reshape_model_max_bin_idx may be smaller than reshape_model_min_bin_idx.

  - reshaper_model_bin_delta_abs_cw_prec_minus1 may be arbitrarily large.

# Proposed methods

Three aspects to address the syntax design of in-loop reshaping are proposed in this contribution.

- The mapping between original samples and reshaped samples is initialized to be an identity mapping when the current tile group is an I tile group and no reshaping information is signaled in the tile group.

- When the current tile group is P/B tile group and no reshaping information is signaled in the tile group, a reference list and a reference index is signaled to indicate from which reference picture the reshaping information is inherited from.

- Two constrains are proposed for conforming bit-streams:

  - reshape_model_max_bin_idx must be no smaller than reshape_model_min_bin_idx.

  - reshaper_model_bin_delta_abs_cw_prec_minus1 must no larger than 6.

# Experimental Results

The proposed cleanups have been implemented on top of VTM-4.0. Results under AI/RA/LB configurations are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for the cleanups

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.01% | 0.01% | 0.01% | 99% | 100% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 96% | 0.02% | 0.02% | 0.02% | 99% | 99% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.01% | 0.01% | 0.01% | 100% | 100% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.06% | 0.06% | 0.06% | 100% | 101% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.02% | 0.02% | 0.02% | 99% | 101% |  |  |  |  |  |

# Conclusion

In this contribution, three aspects of cleanups are proposed to address the problems in the current syntax design of in-loop reshaping. Simulation results reportedly show that the coding performance change is minor. It is recommended to adopt these cleanups.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).