| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0275 |
| --- | --- |

| Title: | AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Ryan Lei Lidong Xu | Email: | jill.boyce@intel.com ryan.lei@intel.com lidong.xu@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A capability is proposed to divide a picture into two sections:  a special section, and a remainder section of the picture outside of the special section. The special section can be flexibly used for any of several encoder-selected features applied within the special section or at the boundaries between the special section and the remainder section, including intra coding, QP adaptation, or disabling of in-loop filters. The proposed section boundaries would not be associated with tiles, and the mapping would be signaled in the PPS or tile group header at an encoder-selected granularity as small as MinCbSizeY. The regions of the picture contained within the special section need not be contiguous. This general purpose solution can be used for a variety of use cases, including progressive intra refresh, 360 video coding with cube maps, and region-of-interest coding, and could potentially be extended for other use cases.

# Introduction

JVET-M0387 [1] and prior contributions have proposed normative and non-normative changes for progressive intra refresh, which include forced intra coding, QP delta signaling, and disabling of in-loop filtering. JVET-M0892 [2] and prior contributions have proposed normative changes to disable in-loop filtering a specified horizontal and vertical boundaries, motivated by reducing subjective artifacts in 360 video at cube face boundaries. Both JVET-M0387 and MVET-M0892 include disabling of in-loop filtering, but each addresses a single use case with different signaling. This contribution proposes a general purpose solution that can address both of these use cases, and can be more broadly applied for additional use cases.

This contribution proposes a mechanism for signaling the division of a picture into two sections: a special section and a remainder section. The encoder can flexibly select and signal what are the features of the special section and the boundary between the special section and remainder section. In the current contribution, several such features are proposed, each individually controlled by syntax elements in the PPS. The proposed features are:

- Forced intra coding and QP offset within the special section

- Disabling inloop filtering (deblocking, SAO, and ALF) across the boundary between the special section and remainder section

- Non-normative signaling of limiting intra and inter prediction for progressive intra refresh from “clean” regions of the current or reference picture, respectively, with the “clean regions” derived from the special section mapping

The granularity of the signaling of the mapping of the picture into the special section and remainder section is selected by the encoder, based on the granularity needed for the use cases, but no smaller than the MinCbSizeY. The section mapping is not aligned with tiles. The regions of the picture contained within the special section may be non-contiguous.

The mapping of the picture into sections differs from the signaling proposed in JVET-M0387 for progressive intra refresh and in JVET-M0892 for disabling in-loop filtering at cube face boundaries. The The high level syntax in JVET-M0387 limits an intra refresh region of a picture to be a contiguous group of either rows or columns across the entire picture. The high level syntax in JVET-M8092 indicates a number of horizontal and/or vertical boundaries across the entire picture across which in-loop filtering is disabled.

The VVC Draft 4 design does not currently provide any mechanism for disabling in-loop filters at arbitrary CU boundaries. In the VVC Draft 4 design, in-loop filters may be disabled at tile boundaries, and tile sizes are restricted to CTU granularity.

# Signaling of special section mapping

The horizontal and vertical granularity of the special section map signaling is determined by the encoder and explicitly signaled in the PPS, and optionally overwritten in the tile group header. The smallest signaling granularity is MinCbSizeY, and the largest signaling granularity is the coded picture width or height. Syntax elements signal the number of units into which the coded picture width and the coded picture height, respectively, are split into.

The proposed syntax and semantics are in section 4. Higher signaling overhead is required for the section mapping signaling when more horizontal or vertical units are used.

There is no required alignment between tiles and the section mapping. An entire CU is semantically restricted to be entirely within one section, either the special section or remainder section.

A decoder would require storage, at the MinCbSizeY granularity for entire picture, of one bit to indicate whether the block is within the special section, e.g. the block section is “active,” or in the remainder section.

A one bit flag, section_active_flag[ i ][ j ] is signaled for each i in 0..section_hor_units_minus1, j in 0 .. section_ver_units_minus1.

To map to MinCbSizeY granularity, for x in 0..PicWidthInMinCbSizeY − 1, y in 0..PicHeightInMinCbSizeY − 1,

SectionActiveMinCb[ x ][ y ] = section_active_flag

[ (x *  (section_hor_units_minus1 + 1)/ PicWidthInMinCbSizeY) ]

[ (y *  (section_ver_units_minus1 + 1)/ PicHeightInMinCbSizeY) ]

The figures below illustrate several examples of how the picture could be mapped into special sections.

The example in Figure 1 splits the picture into 4 horizontal units and 2 vertical units for section mapping, with the special section indicated in blue and the remainder section in white.

Fig. 1: Example special section mapping with 4 horizontal units and 2 vertical units

Figure 2 is an example for a 360 video cube map, with 1 horizontal unit and 2 vertical units used for section mapping. The figure shows the 6 cube faces indicated with dashed lines, with 2 cube face rows with contiguous faces. Only cube face rows are separated into sections. This mapping can be used to disable in loop filters at the cube face row boundary. CE13 Test 13.1.1 at the Macau meeting, in JVET-M0033 [3], proposed disabling of in-loop filters at cube face row boundaries, and informal expert viewing showed this improved subjective quality, as summarized in the BoG report in JVET-M0874 [4].

Fig. 2: Example special section mapping with 1 horizontal units and 2 vertical units, for 360 video cube map

Figure 3 is an example for progressive intra refresh, where a vertical column is mapped to the special section. In this example, the picture is split into 4 horizontal units and 1 vertical unit, which enables ¼ of the picture to be intra updated. For the intra refresh use case, the special section is used to indicate the intra refresh region, and the region would change with each picture, so that the all positions in the entire picture would eventually be included within the special section, spread across multiple pictures.  A more typical special section mapping for the intra refresh use case would use more horizontal units and apply intra update to a smaller fraction of the picture, i.e. 1/32 of each picture could be mapped to the special section for intra refresh, to refresh the entire picure within 32 coded pictures.

Fig. 3: Example special section mapping with 4 horizontal units and 1 vertical unit, for

progressive intra refresh

# Encoder-selectable features

The proposed general purpose “special section” capability can be flexibly used by the encoder, to address desired use cases, and potentially extended in the future for additional use cases. Several flags are proposed to be included in the PPS to indicate the features of the special section or of the boundaries between the special section and remainder section. Alternatively, those flags could be signaled in the SPS. The encoder can enable any combination of the features.

## Disabling in-loop filters across special section and remainder section boundaries

A flag is proposed to signal that for the two blocks across a horizontal edge or vertical edge, if one block is within the special section and the other block is within the remainder section, in-loop filtering be disabled across that edge. The specific semantics to disable the in-loop filters are the same as proposed in JVET-M0892 [2], and includes all of ALF, SAO and deblocking. Only the proposed high-level syntax is different.

This contribution differs from JVET-M0387 in that, when selected, in-loop filters are disabled across all boundaries between the special section and remainder section, regardless of whether the remainder section is part of a “clean” region from a previously coded picture. This allows flexible section mapping patterns, while avoiding the need to keep track of “clean” regions across pictures.

As shown in section 4, there is a semantic restriction that an entire CU must be within the same section, so all boundaries between the special section and remainder section must occur at CU boundaries.

## QP offset within special section

A flag is proposed to signal negative QP offsets for luma and chroma to be applied within the special section, which is used in addition to the signaled CU QP delta. This can be used for progressive intra refresh, or for coding a region of interest with a higher quality level.

It is expected that signaling a negative QP offsets for luma and chroma for the entire special section will save bitrate vs. signaling a QP delta individually for each CU.

## Forced intra within special section

A flag is proposed to force intra coding of CUs within the special section, without signaling of cu_skip_flag[ x0 ][ y0 ] and pred_mode_flag.

It is expected that forcing intra coding for entire special section will save bitrate by avoiding signaling of  cu_skip_flag[ x0 ][ y0 ] and pred_mode_flag at the CU level.

JVET-M0387 also included a proposal to avoid signaling of these syntax elements within a progressive intra refresh area. The combined proposal in JVET-M0387, which also contained some additional features, provided some coding efficiency gains, but did not break down the gains of the different components of the proposal.

## Prediction restriction for progressive intra refresh

Two flags are proposed to indicate that the encoder has constrained intra and inter prediction, respectively, to enable progressive intra refresh functionality. The intra prediction restriction flag indicates that for the current picture, all spatial intra prediction of CUs within the special section may not refer to any samples outside of a “clean” progressive intra refresh section. The inter prediction restriction flag restricts inter prediction to enable “clean” progressive intra refresh decoder operation.

These flags do not require any normative changes to decoder behavior. While it may be possible to signal these flags in the VUI of in an SEI message instead of in the PPS, it would be difficult to interpret their meaning without the special section mapping signaled in the PPS or tile group header.

When the inter prediction restriction flag is signaled, it shall be the same for all active PPSs in the coded video sequence. For “clean” intra and inter prediction operation in the progressive intra refresh use case, clean and dirty regions of the picture are tracked and updated for each coded video picture, in coding order. Intra prediction for clean regions in the current picture is only allowed from clean regions in the current picture. Inter prediction for clean regions in the current picture is only allowed from clean regions in the reference picture. Whenever the upper left minCB block of a picture is included within the special section, a reset takes place and all blocks within the special section of the current picture are set to be clean, and the blocks in the remainder section are reset to be dirty.  For subsequent pictures in coding order, the clean/dirty region mapping is inherited from the previously coded picture and then updated by setting the blocks within the special section of the current picture to be clean. Intra prediction in all CUs in the clean region of the current picture are restricted to be inter predicted using only samples within the clean region of the current picture, including all samples needed for filtering. Inter prediction in all CUs in the clean region of the current picture are restricted to be inter predicted using only samples within the clean region of the reference picture, including all samples needed for sub-pel interpolation.

# Syntax and semantics

## SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_section_enable_flag | u(1) |
| … |  |

sps_section_enable_flag equal to 1 indicates that a special section of coding units will be signalled for pictures referring to the SPS.

## PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. | ue(v) |
| pps_section_flag | u(1) |
| if ( pps_section_flag) { |  |
| section_disable_inloop_filters_flag | u(1) |
| section_qp_offset_flag | u(1) |
| if ( section_qp_offset_flag ) { |  |
| section_neg_qp_offset_luma | u(4) |
| section_neg_qp_offset_chroma | u(4) |
| } |  |
| section_force_intra_flag | u(1) |
| section_intra_pred_restrict_flag | u(1) |
| if (section_intra_pred_restrict_flag) |  |
| section_inter_pred_restrict_flag | u(1) |
| section_hor_units_minus1 | ue(v) |
| section_ver_units_minus1 | ue(v) |
| for ( i = 0; i <= section_hor_units_minus1; i++ ) |  |
| for ( j = 0; j<= section_ver_units_minus1; j++ ) |  |
| section_active_flag[ i ][ j ] | u(1) |
| pps_section_tile_group_mapping_present_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_section_flag equal to 1 specifies that a special section of coding units will be signalled in the PPS. pps_section_flag equal to 0 specifies that a special section of coding units will not be signalled in pictures referring to the PPS.

section_disable_inloop_filters_flag equal to 1 specifies that in-loop filters are disabled across special section boundaries in pictures referring to the PPS. section_disable_inloop_filters_flag equal to 0 specifies that in-loop filters may be performed in pictures referring to the PPS. When section_disable_inloop_filters_flag not present, it is inferred to be equal to 0.

section_qp_offset_flag equal to 1 specifies that the section_neg_qp_offset_luma and section_neg_qp_offset_chroma syntax elements are present. section_qp_offset_flag equal to 0 specifies that the section_neg_ qp_offset_luma and section_neg_ qp_offset_chroma syntax elements are not present. When section_qp_offset_flag not present, it is inferred to be equal to 0.

section_neg_ qp_offset_luma specifies a difference to be subtracted when determining the value of the luma quantization parameter Qp′Y in coding units within the signalled special section in pictures referring to the PPS. When section_neg_ qp_offset_luma is not present, it is inferred to be equal to 0.

section_neg_ qp_offset_chroma specifies a difference to be subtracted when determining the value of the chroma quantization parameters Qp′Cb and Qp′Cr in coding units within the signalled special section in pictures referring to the PPS. When section_neg_ qp_offset_chroma is not present, it is inferred to be equal to 0.

section_force_intra_flag equal to 1 specifies that coding units within the signalled special section are coded in intra prediction mode in pictures referring to the PPS. When section_force_intra_flag not present, it is inferred to be equal to 0.

section_intra_pred_restrict_flag equal to 1 specifies restrictions on intra prediction for coding units within the clean regions in the current picture, as indicated by the SectionClean[ ][ ] variable being equal to 1,  in which case intra prediction only uses residual data and decoded samples from neighboring coding blocks within the clean region.  When section_intra_pred_restrict_flag not present, it is inferred to be equal to 0. The value of section_intra _pred_restrict_flag, when present,  shall be same for all activated PPSs in the coded video sequence.

section_inter _pred_restrict_flag equal to 1 specifies restrictions on inter prediction for coding units within the clean regions in the current picture, as indicated by the SectionClean[ ][ ] variable being equal to 1, in which case inter prediction only uses samples from clean regions in the reference pictures.When section_inter_pred_restrict_flag not present, it is inferred to be equal to 0.The value of section_inter _pred_restrict_flag, when present,  shall be same for all activated PPSs in the coded video sequence.

section_hor_units_minus1 plus 1 and section_ver_units_minus1 plus 1 specify the number of section_active_flag[ i ][ j ] syntax elements that will be signalled. When section_hor_units_minus1  and section_ver_units_minus1 are not present, they are inferred to be equal to 0.

section_active_flag[ i ][ j ] equal to 1 specifies that the block (i, j) is included within the special section in picutres referring to the PPS. section_active_flag[ i ][ j ] equal to 0 specifies that the block (i, j)  is not included within the special section in pictures referring to the PPS. The variables SectionActiveMinCb[ x ][ y ] and SectionClean[ x ][ y ] are derived as follows for x in 0.. PicWidthInMinCbSizeY − 1, y in 0.. PicHeightInMinCbSizeY − 1,

SectionActiveMinCb[ x ][ y ] = section_active_flag

[ (x *  (section_hor_units_minus1 + 1)/ PicWidthInMinCbSizeY) ]

[ (y *  (section_ver_units_minus1 + 1)/ PicHeightInMinCbSizeY) ]

if (SectionActiveMinCb[ 0 ][ 0 ] )

SectionClean[ x ][ y ] = SectionActiveMinCb[ x ][ y ]

else

SectionClean[ x ][ y ] = SectionClean[ x ][ y ]  | | SectionActiveMinCb[ x ][ y ]

It is a requirement of bitstream conformance that the value of SectionActiveMinCb[ x ][ y ] be the same for all x, y within the same CU.

pps_section_tile_group_mapping_present_flag indicates that tile_group_section_hor_units_minus1 and tile_group_section_ver_units_minus1 syntax elements are present in the associated tile group headers. pps_section_tile_group_mapping_present_flag  equal to 0 indicates that these syntax elements are not present in the associated tile group headers.

## Tile group header

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_section_tile_group_mapping_present_flag ){ |  |
| tile_group_section_hor_units_minus1 | ue(v) |
| tile_group_section_ver_units_minus1 | ue(v) |
| for ( i = 0; i <= tile_group_section_hor_units_minus1; i++ ) |  |
| for ( j = 0; j<= tile_group_section_ver_units_minus1; j++ ) |  |
| tile_group_section_active_flag[ i ][ j ] | u(1) |
| } |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_section_hor_units_minus1 plus 1 and tile_group section_ver_units_minus1 plus 1 specify the number of tile_group section_active_flag[ i ][ j ] syntax elements that will be signalled. When tile_group section_hor_units_minus1 and tile_group section_ver_units_minus1 not present, they are inferred to be equal to 0. It is a requirement of bitstream conformance that the values of tile_group_section_hor_units_minus1 plus 1 and tile_group section_ver_units_minus1shall be the same for all slices of a coded picture.

tile_group section_active_flag[ i ][ j ] equal to 1 specifies that the block (i, j) is included within the special section in the current picture. tile_group_section_active_flag[ i ][ j ] equal to 0 specifies that the block (i, j)  is not included within the special section in the current tile. It is a requirement of bitstream conformance that the values of tile_group section_active_flag[ i ][ j ] shall be the same for all tile groups of a coded picture for all i, j.

The variables SectionActiveMinCb[ x ][ y ] and SectionClean[ x ][ y ] are derived as follows for x in 0.. PicWidthInMinCbSizeY − 1, y in 0.. PicHeightInMinCbSizeY − 1,

if ( pps_section_tile_group_mapping_present_flag  )

SectionActiveMinCb[ x ][ y ] = tile_group_section_active_flag

[ (x *  (section_hor_units_minus1 + 1)/ PicWidthInMinCbSizeY) ]

[ (y *  (section_ver_units_minus1 + 1)/ PicHeightInMinCbSizeY) ]

if (SectionActiveMinCb[ 0 ][ 0 ] )

SectionClean[ x ][ y ] = SectionActiveMinCb[ x ][ y ]

else

SectionClean[ x ][ y ] = SectionClean[ x ][ y ] | | SectionActiveMinCb[ x ][ y ]

It is a requirement of bitstream conformance that the value of SectionActiveMinCb[ x ][ y ] be the same for all x, y within the same CU.

## Coding unit

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( ( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag) && !(section_force_intra_flag && SectionActiveMinCb[x0>> MinCbLog2SizeY][y0>> MinCbLog2SizeY] ) ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| … |  |

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable CuPredMode[ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If pred_mode_flag is equal to 0, CuPredMode[ x ][ y ] is set equal to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1), CuPredMode[ x ][ y ] is set equal to MODE_INTRA.

When pred_mode_flag is not present, it is inferred to be equal to 1 when decoding an I tile group or ( !section_force_intra_flag && SectionActiveMinCb[x0>> MinCbLog2SizeY][y0>> MinCbLog2SizeY] )  , and equal to 0 when decoding a P or B tile group, respectively.

# Decoding process

## 8.7.1 Derivation process for quantization parameters

…

QpySecNegOffsetLuma= SectionActiveMinCb[xCb>>MinCbLog2SizeY ][ yCb>>MinCbLog2SizeY ] ? section_neg_ qp_offset_luma : 0

QpySecNegOffsetChroma = SectionActiveMinCb[xCb>>MinCbLog2SizeY ][ yCb>>MinCbLog2SizeY ] ? section_neg_ qp_offset_chroma : 0

- The predicted luma quantization parameter qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(8-1006)

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal + 64 + 2 * QpBdOffsetY )%( 64 + QpBdOffsetY ) ) − QpBdOffsetY	(8-1007)

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffsetY − QpySecNegOffsetLuma	(8-1008)

[Ed. (BB): Modify highlighted sections when tiles without tile groups are incorporated]

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + tile_group_cb_qp_offset )	(8-1009)

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + tile_group_cr_qp_offset )	(8-1010)

–	If ChromaArrayType is equal to 1, the variables qPCb and qPCr are set equal to the value of QpC as specified in Table 8-12 based on the index qPi equal to qPiCb and qPiCr, respectively.

–	Otherwise, the variables qPCb and qPCr are set equal to Min( qPi, 63 ), based on the index qPi equal to qPiCb and qPiCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC − QpySecNegOffsetChroma	(8-1011)

Qp′Cr = qPCr + QpBdOffsetC − QpySecNegOffsetChroma	(8-1012)

## 8.8.2.1 General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The vertical edges in a picture are filtered first. Then the horizontal edges in a picture are filtered with samples modified by the vertical edge filtering process as input. The vertical and horizontal edges in the CTBs of each CTU are processed separately on a coding unit basis. The vertical edges of the coding blocks in a coding unit are filtered starting with the edge on the left-hand side of the coding blocks proceeding through the edges towards the right-hand side of the coding blocks in their geometrical order. The horizontal edges of the coding blocks in a coding unit are filtered starting with the edge on the top of the coding blocks proceeding through the edges towards the bottom of the coding blocks in their geometrical order.

NOTE – Although the filtering process is specified on a picture basis in this Specification, the filtering process can be implemented on a coding unit basis with an equivalent result, provided the decoder properly accounts for the processing dependency order so as to produce the same output values.

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

–	Edges that are at the boundary of the picture,

–	Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

–	Edges that coincide with upper or left boundaries of tile groups with tile_group_loop_filter_across_tile_groups_enabled_flag equal to 0 or tile_group_deblocking_filter_disabled_flag equal to 1,

–	Edges within tile groups with tile_group_deblocking_filter_disabled_flag equal to 1,

–	Edges that do not correspond to 8x8 sample grid boundaries of the considered component,

–	Edges within chroma components for which both sides of the edge use inter prediction,

–	Edges of chroma transform blocks that are not edges of the associated transform unit.

–	Edges across the luma transform blocks of a coding unit that has an IntraSubPartitionsSplit value not equal to ISP_NO_SPLIT.

–	Edges that coincide with boundaries between the special section and remainder section when section_disable_inloop_filter_flag is equal to 1.

[Ed. (BB): Adapt syntax once tiles are integrated.]

The edge type, vertical or horizontal, is represented by the variable edgeType as specified in Table 8-16.

Table 8-16 – Name of association to edgeType

| edgeType | Name of edgeType |
| --- | --- |
| 0 (vertical edge) | EDGE_VER |
| 1 (horizontal edge) | EDGE_HOR |

When tile_group_deblocking_filter_disabled_flag of the current tile group is equal to 0, the following applies:

- The variable treeType is derived as follows:

- If tile_group_type is equal to I and qtbtt_dual_tree_intra_flag is equal to 1, treeType is set equal to DUAL_TREE_LUMA.

- Otherwise, treeType is set equal to SINGLE_TREE.

- The vertical edges are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.2.2 with the variable treeType, the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0 or treeType is equal to SINGLE_TREE, the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_VER as inputs, and the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0 or treeType is equal to SINGLE_TREE, the arrays recPictureCb and recPictureCr as outputs.

- The horizontal edge are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.2.2 with the variable treeType, the modifed reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0 or treeType is equal to SINGLE_TREE, the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_HOR as inputs, and the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0 or treeType is equal to SINGLE_TREE, the arrays recPictureCb and recPictureCr as outputs.

- When tile_group_type is equal to I and qtbtt_dual_tree_intra_flag is equal to 1, the following applies:

- The variable treeType is set equal to DUAL_TREE_CHROMA

- The vertical edges are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.2.2 with the variable treeType, the reconstructed picture prior to deblocking, i.e., the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_VER as inputs, and the modified reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr as outputs.

- The horizontal edge are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.2.2 with the variable treeType, the modifed reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_HOR as inputs, and the modified reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr as outputs.

8.8.2.2 Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the reconstructed picture prior to deblocking, i.e., the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are the modified reconstructed picture after deblocking, i.e:

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr.

For each coding unit with coding block width log2CbW, coding block height log2CbH and location of top-left sample of the coding block ( xCb, yCb ), when edgeType is equal to EDGE_VER and xCb % 8 is equal 0 or when edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The coding block width nCbW is set equal to 1  <<  log2CbW and the coding block height nCbH is set equal to 1  <<  log2CbH

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- SectionActiveMinCb[xCb>>MinCbLog2SizeY][yCb>>MinCbLog2SizeY ] != SectionActiveMinCb[xCb>>MinCbLog2SizeY ][yCb>>MinCbLog2SizeY − 1] and section_disable_inloop_filters_flag is equal to 1.

- Otherwise if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- SectionActiveMinCb[xCb>>MinCbLog2SizeY ][yCb >>MinCbLog2SizeY ] != SectionActiveMinCb[xCb >>MinCbLog2SizeY−1][ yCb >>MinCbLog2SizeY ] and section_disable_inloop_filters_flag is equal to 1.

- Otherwise, filterEdgeFlag is set equal to 1.

[Ed. (BB): Adapt syntax once tiles are integrated.]

- All elements of the two-dimensional (nCbW)x(nCbH) array edgeFlags are initialized to be equal to zero.

- The derivation process of transform block boundary specified in clause 8.8.2.3 is invoked with the location ( xB0, yB0 ) set equal to ( 0, 0 ), the block width nTbW set equal to nCbW, the block height nTbH set equal to nCbH, the variable treeType, the variable filterEdgeFlag, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The derivation process of coding subblock boundary specified in clause 8.8.2.4 is invoked with the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The picture sample array recPicture is derived as follows:

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, recPicture is set equal to the reconstructed luma picture sample array prior to deblocking recPictureL.

- Otherwise (treeType is equal to DUAL_TREE_CHROMA), recPicture is set equal to the reconstructed chroma picture sample array prior to deblocking recPictureCb.

- The derivation process of the boundary filtering strength specified in clause 8.8.2.5 is invoked with the picture sample array recPicture, the luma location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the variable edgeType, and the array edgeFlags as inputs, and an (nCbW)x(nCbH) array verBs as output.

- The edge filtering process is invoked as follows:

- If edgeType is equal to EDGE_VER, the vertical edge filtering process for a coding unit as specified in clause 8.8.2.6.1 is invoked with the variable treeType, the reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the array verBs as inputs, and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

- Otherwise if edgeType is equal to EDGE_HOR, the horizontal edge filtering process for a coding unit as specified in clause 8.8.2.6.2 is invoked with the variable treeType, the modified reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the array horBs as inputs and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

## 8.8.3.2 CTB modification process

Inputs to this process are:

–	the picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying the colour component index,

–	a pair of variables ( rx, ry ) specifying the CTB location,

–	the CTB width nCtbSw and height nCtbSh.

Output of this process is a modified picture sample array saoPicture for the colour component cIdx.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The location ( xCtb, yCtb ), specifying the top-left sample of the current CTB for the colour component cIdx relative to the top-left sample of the current picture component cIdx, is derived as follows:

( xCtb, yCtb ) = ( rx * nCtbSw, ry * nCtbSh )		(8-1125)

The sample locations inside the current CTB are derived as follows:

( xSi, ySj ) = ( xCtb + i, yCtb + j )		(8-1126)

( xYi, yYj ) = ( cIdx  = =  0 ) ? ( xSi, ySj ) : ( xSi * SubWidthC, ySj * SubHeightC )	(8-1127)

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, depending on the values of pcm_loop_filter_disabled_flag, pcm_flag[ xYi ][ yYj ] and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xSi ][ ySj ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xSi ][ ySj ] is not modified:

- pcm_loop_filter_disabled_flag and pcm_flag[ xYi ][ yYj ] are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 0.

[Ed. (BB): Modify highlighted sections prending on future decision transform/quantizaion bypass.]

–	Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k = 0..1 are specified in Table 8-18 based on SaoEoClass[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows:

- The modified sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) are derived as follows:

( xSik′, ySjk′ ) = ( xSi + hPos[ k ], ySj + vPos[ k ] )	(8-1128)

( xYik′, yYjk′ ) = ( cIdx  = =  0 ) ? ( xSik′, ySjk′ ) : ( xSik′ * SubWidthC, ySjk′ * SubHeightC )	(8-1129)

- If one or more of the following conditions for all sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) with k = 0..1 are true, edgeIdx is set equal to 0:

- The sample at location ( xSik′, ySjk′ ) is outside the picture boundaries.

- The sample at location ( xSik′, ySjk′ ) belongs to a different tile group and one of the following two conditions is true:

- MinTbAddrZs[ xYik′  >>  MinTbLog2SizeY ][ yYjk′  >>  MinTbLog2SizeY ] is less than MinTbAddrZs[ xYi  >>  MinTbLog2SizeY ][ yYj  >>  MinTbLog2SizeY ] and tile_group_loop_filter_across_tile_groups_enabled_flag in the tile group which the sample recPicture[ xSi ][ ySj ] belongs to is equal to 0.

- MinTbAddrZs[ xYi  >>  MinTbLog2SizeY ][ yYj  >>  MinTbLog2SizeY ] is less than MinTbAddrZs[ xYik′  >>  MinTbLog2SizeY ][ yYjk′  >>  MinTbLog2SizeY ] and tile_group_loop_filter_across_tile_groups_enabled_flag in the tile group which the sample recPicture[ xSik′ ][ ySjk′ ] belongs to is equal to 0.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different tile.

- section_disable_inloop_filters_flag is equal to 1 and SectionActiveMinCb[xSik′ >>MinCbLog2SizeY ][ ySjk′ >>MinCbLog2SizeY ]!= SectionActiveMinCb[xYik>>MinCbLog2SizeY ][ yYjk′ >>MinCbLog2SizeY ]

[Ed. (BB): Modify highlighted sections when tiles without tile groups are incorporated]

- Otherwise, edgeIdx is derived as follows:

- The following applies:

edgeIdx = 2 + Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 0 ] ][ ySj + vPos[ 0 ] ] ) +
		Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 1 ] ][ ySj + vPos[ 1 ] ] )	(8-1130)

- When edgeIdx is equal to 0, 1, or 2, edgeIdx is modified as follows:

edgeIdx = ( edgeIdx  = =  2 ) ? 0 : ( edgeIdx + 1 )	(8-1131)

- The modified picture sample array saoPicture[ xSi ][ ySj ] is derived as follows:

saoPicture[ xSi ][ ySj ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, recPicture[ xSi ][ ySj ] +
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-1132)

- Otherwise (SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1), the following ordered steps apply:

- The variable bandShift is set equal to bitDepth − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The list bandTable is defined with 32 elements and all elements are initially set equal to 0. Then, four of its elements (indicating the starting position of bands for explicit offsets) are modified as follows:

for( k = 0; k < 4; k++ )
	bandTable[ ( k + saoLeftClass ) & 31 ] = k + 1	(8-1133)

- The variable bandIdx is set equal to bandTable[ recPicture[ xSi ][ ySj ]  >>  bandShift ].

- The modified picture sample array saoPicture[ xSi ][ ySj ] is derived as follows:

saoPicture[ xSi ][ ySj ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, recPicture[ xSi ][ ySj ] +
			SaoOffsetVal[ cIdx ][ rx ][ ry ][ bandIdx ] )	(8-1134)

Table 8-18 – Specification of hPos and vPos according to the sample adaptive offset class

| SaoEoClass[ cIdx ][ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| hPos[ 0 ] | −1 | 0 | −1 | 1 |
| hPos[ 1 ] | 1 | 0 | 1 | −1 |
| vPos[ 0 ] | 0 | −1 | −1 | −1 |
| vPos[ 1 ] | 0 | 1 | 1 | 1 |

### 8.8.4 Adaptive loop filter process

To be provided once specification text is available to handle disabling ALF across tile boundaries .

# Signalling bit count examples

Bit counts are provided for additional bits in the PPS for the examples in the figures.

## Example 1: Regions of Interest with QP offset, from Figure 1

pps_section_flag = 1

section_disable_inloop_filters_flag = 1

section_qp_offset_flag = 1

section_neg_ qp_offset_luma = 12

section_neg_ qp_offset_chroma = 12

section_force_intra_flag = 0

section_intra_pred_restrict_flag = 0

section_hor_units_minus1 = 3

section_ver_units_minus1 = 1

section_active_flag[ 0 ][ 0 ] = 1

section_active_flag[ 0 ][ 1 ] = 0

section_active_flag[ 0 ][ 2 ] = 0

section_active_flag[ 0 ][ 3 ] = 0

section_active_flag[ 1 ][ 0 ] = 0

section_active_flag[ 1 ][ 1 ] = 0

section_active_flag[ 1 ][ 2 ] = 1

section_active_flag[ 1 ][ 3 ] = 0

pps_section_tile_group_mapping_present_flag = 0

Total bitcount in PPS:  30 bits

## Example 2: for 360 cube map, with disabling of inloop filter across face row, from Figure 2

pps_section_flag = 1

section_disable_inloop_filters_flag = 1

section_qp_offset_flag = 0

section_force_intra_flag = 0

section_intra_pred_restrict_flag = 0

section_hor_units_minus1 = 0

section_ver_units_minus1 = 1

section_active_flag[ 0 ][ 0 ] = 1

section_active_flag[ 1 ][ 0 ] = 0

pps_section_tile_group_mapping_present_flag = 0

Total bitcount in PPS: 12 bits

## Example 3: for progressive intra refresh, from Figure 3

pps_section_flag = 1

section_disable_inloop_filters_flag = 1

section_qp_offset_flag = 1

section_neg_ qp_offset_luma = 6

section_neg_ qp_offset_chroma = 6

section_force_intra_flag = 1

section_intra_pred_restrict_flag = 1

section_inter_pred_restrict_flag = 1

section_hor_units_minus1 = 3

section_ver_units_minus1 = 0

section_active_flag[ 0 ][ 0 ] = 0

section_active_flag[ 0 ][ 1 ] = 1

section_active_flag[ 0 ][ 2 ] = 0

section_active_flag[ 0 ][ 3 ] = 0

pps_section_tile_group_mapping_present_flag = 0

Total bitcount in PPS: 25 bits

(For a more typical intra refresh arrangement with narrower intra refresh column, and updates of full intra frame in 32 frames, would have 32 horizontal units, and total additional PPS bit count would be 57 bits.)

# References

[1] Jean-Marc Thiesse, Didier Nicholson, David Gommelet, JVET-M0387, “AHG14: Updates on Intra Refresh Proposal,” Jan 2019.

[2] Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju, Philippe Hanhart, Yuwen He, JVET-M0892, “CE-13 related: Loop filter disabled across virtual boundaries,” Jan 2019.

[3] Philippe Hanhart, Jian-Liang Lin, Chirag Pujara, JVET-M0033, “CE13: Summary report on coding tools for 360° omnidirectional video,” Jan 2019.

[4] Jill Boyce, JVET-M0874, “BoG report on CE13 and CE13 related 360° video coding,” Jan 2019.

[5] Benjamin Bross, Jianle Chen, Shan Liu, JVET-M1001-v3, “Versatile Video Coding (Draft 4)”

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).