| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0276 |
| --- | --- |

| Title: | AHG15: On interoperability point signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

Five new constraint flags are proposed, primarily for tools adoptions at the prior meeting, and small modifications of existing constraint flags to align with renaming or syntax reordering in the VVC Draft 4 specification. It is also proposed to have the presence of tool enable flags in the SPS, PPS, and tile group header be conditioned on the value of the corresponding constraint flags in the general_constraint_info( ) syntax structure. Additionally, it proposed to add a syntax element in the profile_tier_level( ) syntax structure for sub-profile indication.

# Proposal summary

The following new constraint flags are proposed:

- no_partition_constraints_override _constraint_flag

- no_dmvr_constraint_flag

- no_sbt_constraint_flag

- no_fpel_mmvd_constraint_flag

- no_transform_skip_constraint_flag

Some existing constraint flags are renamed or reordered, in order to align with VVC Draft 4, as shown in Section 2.

It is also proposed to not signal the syntax element for a tool enable flag if the use of the tool is already forbidden by the corresponding constraint flag. When an enable flag syntax element is not present, its value is inferred to be equal to 0. This enables a savings in bitrate overhead for the flags, and avoids the possibility of conflicting signaling in non-conforming bitstreams.

It is noted that some enable flags are signaled in the PPS, while the constraint flags are signaled in the general_constraint_info( ) syntax structure of the SPS. JVET-M0133 has proposed to allow a relaxation of the general design principle followed for AVC and HEVC that avoided parsing dependency of the PPS on the SPS.

A sub-profile indicator syntax element is proposed to be added to the profile_tier_level( ) syntax structure, with the semantics below.

general_sub_profile_idc indicates interoperability metadata registered as specified by Recommendation ITU-T T.35, the contents of which are not specified in this Specification.

It is suggested that Recommendation ITU-T T.35 be used as a registration authority, although this aspect may require discussion at the parent body level.

# Syntax and semantics changes

Syntax elements fully in yellow are newly proposed syntax elements. Syntax elements in blue indicate renamed or relocated syntax elements. Additional conditions are shown with change marks, but not highlighted.

#### General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_pcm_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_gbi_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| no_triangle_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_qp_delta_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| // ADD reserved bits for future extensions |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_max_dec_pic_buffering_minus1 | ue(v) |
| long_term_ref_pics_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < !rpl1_same_as_rpl0_flag ? 2 : 1; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| if ( !no_qtbtt_dual_tree_intra_constraint_flag ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| if ( !no_partition_constraints_override _constraint_flag ) |  |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_tile_group_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_tile_groups != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_tile_group | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_tile_group | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_tile_group_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma | ue(v) |
| } |  |
| } |  |
| if ( !no_sao_constraint_flag ) |  |
| sps_sao_enabled_flag | u(1) |
| if ( !no_alf_constraint_flag ) |  |
| sps_alf_enabled_flag | u(1) |
| if ( !no_pcm_constraint_flag ) |  |
| sps_pcm_enabled_flag | u(1) |
| if( sps_pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| if ( !no_ref_wraparound_constraint_flag ) |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| if ( !no_temporal_mvp_constraint_flag ) |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag && !no_sbtmvp_constraint_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if ( !no_amvr_constraint_flag ) |  |
| sps_amvr_enabled_flag | u(1) |
| if ( !no_bdof_constraint_flag ) |  |
| sps_bdof_enabled_flag | u(1) |
| if ( !no_affine_ constraint_flag ) |  |
| sps_affine_amvr_enabled_flag | u(1) |
| if ( !no_dmvr_constraint_flag ) |  |
| sps_dmvr_enabled_flag | u(1) |
| if ( !no_cclm_constraint_flag ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| if ( !no_mts_constraint_flag ) |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| if ( !no_sbt_constraint_flag ) |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| if ( !no_affine_motion_constraint_flag ) |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| if ( !no_gbi_constraint_flag ) |  |
| sps_gbi_enabled_flag | u(1) |
| if ( !no_ibc_constraint_flag ) |  |
| sps_ibc_enabled_flag | u(1) |
| if ( !no_ciip_constraint_flag ) |  |
| sps_ciip_enabled_flag | u(1) |
| if ( !no_fpel_mmvd_constraint_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if ( !no_triangle_constraint_flag ) |  |
| sps_triangle_enabled_flag | u(1) |
| if ( !no_ladf_constraint_flag ) |  |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### General profile, tier, and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_sub_profile_idc | u(24) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| if( no_transform_skip_constraint_flag ) |  |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| if( no_qp_delta_constraint_flag ) |  |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_tile_group_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( tile_group_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| tile_group_aps_id | u(5) |
| } |  |
| if ( !no_dep_quant_constraint_flag ) |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag && ! no_sign_data_hiding_constraint_flag) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### Sequence parameter set RBSP semantics

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process. When not present, the value of sps_sao_enabled_flag is inferred to be equal to 0.

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled. When not present, the value of sps_alf_enabled_flag is inferred to be equal to 0.

sps_pcm_enabled_flag equal to 0 specifies that PCM-related syntax (pcm_sample_bit_depth_luma_minus1, pcm_sample_‌bit_depth_chroma_minus1, log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_‌coding_block_size, pcm_loop_filter_disabled_flag, pcm_flag, pcm_alignment_zero_bit syntax elements and pcm_sample( ) syntax structure) is not present in the CVS. When not present, the value of sps_pcm_enabled_flag is inferred to be equal to 0.

NOTE 3 – When MinCbLog2SizeY is equal to 6 and sps_pcm_enabled_flag is equal to 1, PCM sample data-related syntax (pcm_flag, pcm_alignment_zero_bit syntax elements and pcm_sample( ) syntax structure) is not present in the CVS, because the maximum size of coding blocks that can convey PCM sample data-related syntax is restricted to be less than or equal to Min( CtbLog2SizeY, 5 ). Hence, MinCbLog2SizeY equal to 6 with sps_pcm_enabled_flag equal to 1 is not an appropriate setting to convey PCM sample data in the CVS.

pcm_sample_bit_depth_luma_minus1 specifies the number of bits used to represent each of PCM sample values of the luma component as follows:

PcmBitDepthY = pcm_sample_bit_depth_luma_minus1 + 1	(7-29)

The value of PcmBitDepthY shall be less than or equal to the value of BitDepthY.

pcm_sample_bit_depth_chroma_minus1 specifies the number of bits used to represent each of PCM sample values of the chroma components as follows:

PcmBitDepthC = pcm_sample_bit_depth_chroma_minus1 + 1	(7-30)

The value of PcmBitDepthC shall be less than or equal to the value of BitDepthC. When ChromaArrayType is equal to 0, pcm_sample_bit_depth_chroma_minus1 is not used in the decoding process and decoders shall ignore its value.

log2_min_pcm_luma_coding_block_size_minus3 plus 3 specifies the minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MinIpcmCbSizeY is set equal to log2_min_pcm_luma_coding_block_size_minus3 + 3. The value of Log2MinIpcmCbSizeY shall be in the range of Min( MinCbLog2SizeY, 5 ) to Min( CtbLog2SizeY, 5 ), inclusive.

log2_diff_max_min_pcm_luma_coding_block_size specifies the difference between the maximum and minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MaxIpcmCbSizeY is set equal to log2_diff_max_min_pcm_luma_coding_block_size + Log2MinIpcmCbSizeY. The value of Log2MaxIpcmCbSizeY shall be less than or equal to Min( CtbLog2SizeY, 5 ).

pcm_loop_filter_disabled_flag specifies whether the loop filter process is disabled on reconstructed samples in a coding unit with pcm_flag equal to 1 as follows:

–	If pcm_loop_filter_disabled_flag is equal to 1, the deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are disabled.

–	Otherwise (pcm_loop_filter_disabled_flag value is equal to 0), the deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are not disabled.

When pcm_loop_filter_disabled_flag is not present, it is inferred to be equal to 0.

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When not present, the value of sps_ref_wraparound_enabled_flag is inferred to be equal to 0.

sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of 0 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

sps_temporal_mvp_enabled_flag equal to 1 specifies that tile_group_temporal_mvp_enabled_flag is present in the tile group headers of tile groups with tile_group_type not equal to I in the CVS. sps_temporal_mvp_enabled_flag equal to 0 specifies that tile_group_temporal_mvp_enabled_flag is not present in tile group headers and that temporal motion vector predictors are not used in the CVS. When not present, the value of sps_temporal_mvp_enabled_flag is inferred to be equal to 0.

sps_sbtmvp_enabled_flag equal to 1 specifies that subblock-based temporal motion vector predictors may be used in decoding of pictures with all tile groups having tile_group_type not equal to I in the CVS. sps_sbtmvp_enabled_flag equal to 0 specifies that subblock-based temporal motion vector predictors are not used in the CVS. When sps_sbtmvp_enabled_flag is not present, it is inferred to be equal to 0.

sps_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is used in motion vector coding. amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding. When not present, the value of sps_amvr_enabled_flag is inferred to be equal to 0.

sps_bdof_enabled_flag equal to 0 specifies that the bidirectional optical flow inter prediction is disabled. sps_bdof_enabled_flag equal to 1 specifies that the bidirectional optical flow inter prediction is enabled. When not present, the value of sps_bdof_enabled_flag is inferred to be equal to 0.

sps_affine_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is used in motion vector coding of affine inter mode. sps_affine_amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding of affine inter mode. When not present, the value of X sps_affine_amvr_enabled_flag is inferred to be equal to 0.

sps_dmvr_enabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is enabled. sps_dmvr_enabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is disabled. When not present, the value of sps_dmvr_enabled_flag is inferred to be equal to 0.

sps_cclm_enabled_flag equal to 0 specifies that the cross-component linear model intra prediction from luma component to chroma component is disabled. sps_cclm_enabled_flag equal to 1 specifies that the cross-component linear model intra prediction from luma component to chroma componenent is enabled. When not present, the value of sps_cclm_enabled_flag is inferred to be equal to 0.

sps_cclm_colocated_chroma_flag equal to 1 specifies that the top-left downsampled luma sample in cross-component linear model intra prediction is collocated with the top-left luma sample. sps_cclm_colocated_chroma_flag equal to 0 specifies that the top-left downsampled luma sample in cross-component linear model intra prediction is horizontally co-sited with the top-left luma sample but vertically shifted by 0.5 units of luma samples relatively to the top-left luma sample.

sps_mts_enabled_flag equal to 1 specifies that sps_explicit_mts_intra_enabled_flag is present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is present in the sequence parameter set RBSP syntax. sps_mts_enabled_flag equal to 0 specifies that sps_explicit_mts_intra_enabled_flag is not present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is not present in the sequence parameter set RBSP syntax. When not present, the value of sps_mts_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_intra_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_explicit_mts_intra_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for intra coding units. When not present, the value of sps_explicit_mts_intra_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_inter_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_explicit_mts_inter_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. When not present, the value of sps_explicit_mts_inter_enabled_flag is inferred to be equal to 0.

sps_sbt_enabled_flag equal to 0 specifies that subblock transform for inter-predicted CUs is disabled. sps_sbt_enabled_flag equal to 1 specifies that subblock transform for inter-predicteds CU is enabled. When not present, the value of sps_sbt_enabled_flag is inferred to be equal to 0.

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples.

MaxSbtSize = sps_sbt_max_size_64_flag  ?  64  :  32	(7-31)

sps_affine_enabled_flag specifies whether affine model based motion compensation can be used for inter prediction. If sps_affine_enabled_flag is equal to 0, the syntax shall be constrained such that no affine model based motion compensation is used in the CVS, and inter_affine_flag and cu_affine_type_flag are not present in coding unit syntax of the CVS. Otherwise (sps_affine_enabled_flag is equal to 1), affine model based motion compensation can be used in the CVS. When not present, the value of sps_affine_enabled_flag is inferred to be equal to 0.

sps_affine_type_flag specifies whether 6-parameter affine model based motion compensation can be used for inter prediction. If sps_affine_type_flag is equal to 0, the syntax shall be constrained such that no 6-parameter affine model based motion compensation is used in the CVS, and cu_affine_type_flag is not present in coding unit syntax in the CVS. Otherwise (sps_affine_type_flag is equal to 1), 6-parameter affine model based motion compensation can be used in the CVS. When not present, the value of sps_affine_type_flag is inferred to be equal to 0.

sps_gbi_enabled_flag specifies whether bi-prediction with CU weights can be used for inter prediction. If sps_gbi_enabled_flag is equal to 0, the syntax shall be constrained such that no bi-prediction with CU weights is used in the CVS, and gbi_idx is not present in coding unit syntax of the CVS. Otherwise (sps_gbi_enabled_flag is equal to 1), bi-prediction with CU weights can be used in the CVS. When not present, the value of sps_gbi_enabled_flag is inferred to be equal to 0.

sps_ibc_enabled_flag equal to 1 specifies that current picture referencing may be used in decoding of pictures in the CVS. sps_ibc_enabled_flag equal to 0 specifies that current picture referencing is not used in the CVS. When sps_ibc_enabled_flag is not present, it is inferred to be equal to 0.When not present, the value of sps_ibc_enabled_flag is inferred to be equal to 0.

sps_ciip_enabled_flag specifies that ciip_flag may be present in the coding unit syntax for inter coding units. sps_ciip_enabled_flag equal to 0 specifies that ciip_flag is not present in the coding unit syntax for inter coding units. When not present, the value of sps_ciip_enabled_flag is inferred to be equal to 0.

sps_fpel_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference is using integer sample precision. sps_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use fractional sample precision. When not present, the value of sps_fpel_mmvd_enabled_flag is inferred to be equal to 0.

sps_triangle_enabled_flag specifies whether triangular shape based motion compensation can be used for inter prediction. sps_triangle_enabled_flag equal to 0 specifies that the syntax shall be constrained such that no triangular shape based motion compensation is used in the CVS, and merge_triangle_flag, merge_triangle_split_dir, merge_triangle_idx0, and merge_triangle_idx1 are not present in coding unit syntax of the CVS. sps_triangle_enabled_flag equal to 1 specifies that triangular shape based motion compensation can be used in the CVS. When not present, the value of sps_triangle_enabled_flag is inferred to be equal to 0.

sps_ladf_enabled_flag equal to 1, specifies that sps_num_ladf_intervals_minus2, sps_ladf_lowest_interval_qp_offset, sps_ladf_qp_offset[ i ], and sps_ladf_delta_threshold_minus1[ i ] are present in the SPS. When not present, the value of sps_ladf_enabled_flag is inferred to be equal to 0.

#### General profile, tier, and level semantics

When the profile_tier_level( ) structure is included in an SPS, the BitstreamInScope is the CVS for which the SPS is the active SPS.

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex A.

general_profile_idc indicates a profile to which BitstreamInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

general_sub_profile_idc indicates interoperability metadata registered as specified by X Recommendation ITU-T T.35, the contents of which are not specified in this Specification.

general_level_idc indicates a level to which BitstreamInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_level_idc other than those specified in Annex A. Other values of general_level_idc are reserved for future use by ITU-T | ISO/IEC.

NOTE 1 – A greater value of general_level_idc indicates a higher level.

NOTE 2 – When BitstreamInScope conforms to multiple profiles, general_profile_idc should indicate the profile that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification).

#### General constraint information semantics

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in BitstreamInScope should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures in BitstreamInScope should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in BitstreamInScope should be interpreted as unknown or unspecified.

–	Otherwise (general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1), the source scan type of each picture in BitstreamInScope should be interpreted as unknown or unspecified.

[Ed. If picture timing SEI message is specified for VVC, general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1 can be specified to indicate the source scan type of each picture at the picture level using the syntax element source_scan_type in a picture timing SEI message.]

NOTE 1 – Decoders may ignore the values of general_progressive_source_flag and general_interlaced_source_flag. Moreover, the actual source scan type of the pictures is outside the scope of this Specification and the method by which the encoder selects the values of general_progressive_source_flag and general_interlaced_source_flag is unspecified.

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying. [Ed. Consider a better naming of this constraint flag, e.g., display_suitability_flag or something like that.]

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages. [Ed. This NOTE should be updated at a later phase of VVC standardization based on which SEI messages are adopted into the VVC design.]

general_frame_only_constraint_flag equal to 1 specifies that BitstreamInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that BitstreamInScope conveys pictures that may or may not represent frames.

NOTE 3 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with it.

intra_only_constraint_flag equal to 1 specifies that  tile_group_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose a constraint.

max_bitdepth_constraint_idc specifies that  bit_depth_luma_minus8 and bit_depth_chroma_minus8 shall be in the range of 0 to  max_bitdepth_constraint_idc, inclusive.

max_chroma_format_constraint_idc  specifies that  chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc, inclusive.

frame_only_constraint_flag equal to 1 specifies that  the CVS conveys pictures that represent  frames. frame_only_constraint_flag equal to 0 does not impose a constraint.

no_qtbtt_dual_tree_intra constraint_flag equal to 1 specifies that  qtbtt_dual_tree_intra_flag is not present. no_qtbtt_dual_tree_intra constraint_flag equal to 0 specifies that qtbtt_dual_tree_intra_flag is present.

no_partition_constraints_override_constraint_flag equal to 1 specifies that  no_partition_constraints_override_enabled_flag is not present. no_partition_constraints_override constraint_flag equal to 0 specifies that no_partition_constraints_override_enabled_flag is present.

no_partition_constraints_override constraint_flag equal to 1 specifies that no_partition_constraints_override enabled_flag is not present. no_partition_constraints_override constraint_flag equal to 0 specifies that no_partition_constraints_override enabled_flag is present.

no_sao_constraint_flag equal to 1 specifies that sps_sao_enabled_flag is not present. no_sao_constraint_flag equal to 0 specifies that sps_sao_enabled_flag is present.

no_alf_constraint_flag equal to 1 specifies that sps_alf_enabled_flag is present.  no_alf_constraint_flag equal to 0 specifies that sps_alf_enabled_flag is present.

no_pcm_constraint_flag equal to 1 specifies that sps_pcm_enabled_flag is not present.  no_pcm_constraint_flag equal to 0 specifies that sps_pcm_enabled_flag is present.

no_ref_wraparound_constraint_flag equal to 1 specifies that  sps_ref_wraparound_enabled_flag is not present. no_ref_wraparound_constraint_flag equal to 0 specifies that sps_ref_wraparound_enabled_flag is present.

no_temporal_mvp_constraint_flag equal to 1 specifies that  sps_temporal_mvp_enabled_flag is not present. no_temporal_mvp_constraint_flag equal to 0 specifies that sps_temporal_mvp_enabled_flag is present.

no_sbtmvp_constraint_flag equal to 1 specifies that  sps_sbtmvp_enabled_flag is not present. no_sbtmvp_constraint_flag equal to 0 specifies that sps_sbtmvp_enabled_flag is present.

no_amvr_constraint_flag equal to 1 specifies that  sps_amvr_enabled_flag is not present. no_amvr_constraint_flag equal to 0 specifies that sps_amvr_enabled_flag is present.

no_bdof_constraint_flag equal to 1 specifies that  sps_bdof_enabled_flag is not present. no_bdof_constraint_flag equal to 0 specifies that sps_bdof_enabled_flag is present.

no_dmvr_constraint_flag equal to 1 specifies that  sps_dmvr_enabled_flag is not present. no_dmvr_constraint_flag equal to 0 specifies that sps_dmvr_enabled_flag is present.

no_cclm_constraint_flag equal to 1 specifies that  sps_cclm_enabled_flag is not present. no_cclm_constraint_flag equal to 0 specifies that sps_cclm_enabled_flag is present.

no_mts_constraint_flag equal to 1 specifies that  sps_mts_enabled_flag is not present. no_mts_constraint_flag equal to 0 specifies that X sps_mts_enabled_flag is present.

no_sbt_constraint_flag equal to 1 specifies that  sps_sbt_enabled_flag is not present. no_sbt_constraint_flag equal to 0 specifies that X sps_sbt_enabled_flag is present.

no_affine_motion_constraint_flag equal to 1 specifies that  sps_affine_enabled_flag  is not present. no_affine_motion_constraint_flag equal to 0 specifies that sps_affine_enabled_flag  X is present.

no_gbi_constraint_flag equal to 1 specifies that  sps_gbi_enabled_flag is not present. no_gbi_constraint_flag equal to 0 specifies that sps_gbi_enabled_flag is present.

no_ibc_constraint_flag equal to 1 specifies that sps_ibc_enabled_flag is not present. no_ibc_constraint_flag equal to 0 specifies that sps_ibc_enabled_flag is present.

no_ciip_constraint_flag equal to 1 specifies that  sps_ciip_enabled_flag is not present. no_cipp_constraint_flag equal to 0 specifies that sps_ciip_enabled_flag is present.

no_triangle_constraint_flag equal to 1 specifies that  sps_triangle_enabled_flag is not present. no_triangle_constraint_flag equal to 0 specifies that sps_triangle_enabled_flag is present.

no_ladf_constraint_flag equal to 1 specifies that  sps_ladf_enabled_flag  is not present. no_ladf_constraint_flag equal to 0 specifies that sps_ladf_enabled_flag  is present.

no_transform_skip_constraint_flag equal to 1 specifies that  sps_transfrom_skip_enabled_flag  is not present. no_transform_skip_constraint_flag equal to 0 specifics that transform_skip_flag is present.

no_qp_delta_constraint_flag equal to 1 specifies that  cu_qp_delta_enabled_flag is not present. no_qp_delta_constraint_flag equal to 0 specifies that cu_qp_delta_enabled_flag is present.

no_dep_quant_constraint_flag equal to 1 specifies that  dep_quant_enabled_flag is not present. no_dep_quant_constraint_flag equal to 0 specifies that dep_quant_enabled_flag is present.

no_sign_data_hiding_constraint_flag equal to 1 specifies that  sign_data_hiding_enabled_flag is not present. no_sign_data_hiding_constraint_flag equal to 0 specifies that sign_data_hiding_enabled_flag is present.

gci_alignment_zero_bits shall be equal to 0.

#### Picture parameter set RBSP semantics

…

transform_skip_enabled_flag equal to 1 specifies that transform_skip_flag may be present in the transform unit syntax. transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the transform unit syntax. When not present, the value of transform_skip_enabled_flag is inferred to be equal to 0.

log2_transform_skip_max_size_minus2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3.

When not present, the value of log2_transform_skip_max_size_minus2 is inferred to be equal to 0.

The variable MaxTsSize is set equal to 1 << (log2_transform_skip_max_size_minus2 + 2).

cu_qp_delta_enabled_flag equal to 1 specifies that the cu_qp_delta_subdiv syntax element is present in the PPS and that cu_qp_delta_abs may be present in the transform unit syntax. cu_qp_delta_enabled_flag equal to 0 specifies that the cu_qp_delta_subdiv syntax element is not present in the PPS and that cu_qp_delta_abs is not present in the transform unit syntax. When not present, the value of cu_qp_delta_enabled_flag is inferred to be equal to 0.

…

# Patent rights declaration(s)

Intel does not have any current or pending patent rights relating to the technology described in this contribution.