| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0277-v1 |
| --- | --- |

| Title: | Non-CE4: Parallel-friendly pairwise average merge candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM-4.0, the pair-wise average merge candidate can only be derived after the merge candidate list prior to it has been constructed. In this contribution, two solutions are proposed to make the pairwise average merge candidate more friendly for parallel coding. In solution 1, the pairwise average merge candidate is always derived with motion information from the left and above neighbouring blocks. In solution 2, the pairwise average merge candidate is always derived with motion information from the left and above neighbouring blocks, and temporal motion vector prediction. Simulation results reportedly show 0.02% and -0.01% BD-rate changing under RA and LB configurations, respectively, in solution 1; 0.00% and -0.01% BD-rate changing in solution 2.

# Introduction

In VTM-4.0, the pairwise average merge candidate is built as the average of the first two candidate in the merge candidate list. In the worst case, it cannot be built until the merge candidate list prior to it has been constructed, which is not preferable in the parallel-processing design.

# Proposed methods

Two solutions are proposed to build the pairwise average candidate in a more parallel-friendly way.

## Solution 1

The pairwise average merge candidate is always built with motion information from the left and above neighbouring blocks. If only one set of motion information is available from these two, that set of motion information is copied to the pair-wise average candidate, with MVs right-shifted by 1.

## Solution 2

The pair-wise average merge candidate is always built with motion information from the left and above neighbouring blocks, and TMVP. The first two available sets from these three are averaged to build the pair-wise average candidate. If only one set of motion information is available from these three, that motion information is copied to the pairwise average candidate, with MVs right-shifted by 1.

# Experimental Results

The proposed methods have been implemented on top of VTM-4.0. Results of solution 1 and solution 2 under /RA/LB configurations are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results of solution 1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.04% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.02% | 0.00% | -0.03% | 100% | 99% |  |  |  |  |  |
| Class B | 0.03% | -0.01% | 0.00% | 99% | 99% | -0.01% | -0.14% | -0.40% | 99% | 99% |
| Class C | 0.03% | 0.01% | -0.09% | 98% | 95% | -0.04% | 0.37% | -0.04% | 101% | 103% |
| Class E |  |  |  |  |  | 0.03% | 0.00% | -0.15% | 101% | 98% |
| Overall | 0.02% | 0.00% | -0.02% | 99% | 98% | -0.01% | 0.07% | -0.22% | 100% | 100% |
| Class D | 0.03% | 0.02% | 0.10% | 98% | 93% | -0.07% | 0.13% | -0.66% | 100% | 99% |
| Class F | -0.02% | 0.07% | -0.02% | 98% | 98% | -0.02% | -0.19% | 0.55% | 98% | 97% |

Table 2: Results of solution 2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | 0.09% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | 0.02% | -0.03% | 100% | 99% |  |  |  |  |  |
| Class B | 0.02% | -0.10% | 0.04% | 99% | 99% | -0.01% | -0.09% | 0.10% | 99% | 100% |
| Class C | 0.02% | 0.01% | -0.10% | 99% | 98% | -0.02% | 0.27% | -0.10% | 100% | 103% |
| Class E |  |  |  |  |  | 0.00% | 0.35% | 0.17% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.00% | 100% | 99% | -0.01% | 0.14% | 0.05% | 100% | 101% |
| Class D | 0.00% | -0.02% | 0.06% | 99% | 100% | -0.03% | 0.43% | -0.81% | 99% | 98% |
| Class F | 0.00% | -0.01% | -0.02% | 98% | 101% | -0.11% | -0.21% | -0.12% | 98% | 98% |

# Conclusion

Both the two solutions can make the pairwise average merge candidate derivation process more friendly to parallel processing while keeping the coding performance almost unchanged. It is recommended to adopt one of the two solutions into VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).