| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0278-v4 |
| --- | --- |

| Title: | AHG17/AHG15: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

This contribution discusses how the VVC high level syntax can address some of the issues raised in M46578, “On the decoding interface for Immersive Media.” The proposals can be summarized as follows:

- Include bitstream and decoder support for multiple independent layers in first version of VVC

- 7-bit nuh_layer_id syntax element in the two-byte NAL Unit header

- Introduce a Video Parameter Set (VPS), possibly combined with the DPS, with the following features:

  - Indication of the maximum number of layers

  - Flag to indicate that all layers are independent, using “reserved” name

  - Tool constraint flags in the general_constraint_info( ) syntax structure signalled once in the VPS and applied to all layers in the bitstream, rather than signalled per layer in the profile_tier_level( ) syntax structure

  - The –v2 version of this document adds a loop of syntax elements vps_included_layer_id[i]

- The –v2 version of this document updates specification text for the sub-bitstream extraction process to extract a single layer, LIdTarget.

- The –v3 version fixes an “off by one” bug in the VPS syntax loop of syntax elements vps_included_layer_id[i]

- The –v3 version includes a modification to the general decoding process, with LIdTarget set by external means, or derived to a default value of vps_included_layer_id[0] when external means not available

The –v4 version provides modified syntax based on discussion in the BoG in section 3.

# Proposal description

MPEG document M46578, “On the decoding interface for Immersive Media,” was circulated on the JVET reflector, to be made available to non-MPEG members. M46578 suggests capabilities for JVET high level syntax to support client-controlled access to immersive media content, which are formalized in MPEG Document N18134, “MPEG requirements on immersive media delivery and access.”

This contribution discusses how to address some of those requirements in VVC high level syntax.

MPEG N18134 contains the following wording:

In the context of these requirements, an “Object” is defined as a visual element that is independently decodable and represented by its own fully conformant bitstream. This is a draft definition that implies that there is no “subbitstream” or something similar.

HEVC has a Video Parameter Set, which allows multiple bitstream layers to be included within the same bitstream, and distinguishes between a Coded Layer-wise Video Sequence (CLVS) and a Coded Video Sequence (CVS). HEVC has a two byte NAL unit header, which includes a nuh_layer_id syntax element.

In this contribution, an independent layer is considered to be an independently decodable and fully conformant sub-bitstream, similar to an HEVC CLVS for either a base layer or an independent non-base layer (which is similar to an HEVC base layer, but with a non-zero value of nuh_layer_id.)  Bitstreams and decoders are proposed to support multiple independent layers in VVC’s first version. Non-zero values of nuh_layer_id would be supported, either as the only layer in a bitstream or if multiple layers are contained within the same bitstream.

An 7-bit nuh_layer_id syntax element is proposed, which allows up to 127 layers (0 .. 126) within a bitstream, with the possibility of using the highest value (127) to indicate a longer NAL unit header length to indicate additional layers. HEVC contains a note to indicate a possible future expansion based on its highest value of 63. (HEVC allocated 6 bits to nuh_layer_id.) A similar note is proposed to be added in VVC.

It is proposed to define a Video Parameter Set (VPS), which is similar to but simpler than HEVC’s parameter set. The VPS could be combined with the Decoding Parameter Set proposed in JVET-M0101 and recommended by the HLS BoG, as summarized in JVET-M0816.  A syntax element is proposed to be included in the VPS that indicates the maximum number of layers in the bitstream. The VPS is proposed to contain a flag to indicate whether all layers are independent. In the initial version of this specification, this flag will be required to be set, as inter-layer predictive coding is not yet supported, so it can be given a “reserved” name.

VVC Draft 4 contains tool constraint flags and syntax elements in the general_constraint_info( ) syntax structure, which is part of the profile_tier_level( ) syntax structure. It is proposed to include the general_constraint_info( ) syntax structure once in the VPS, to be applied to all layers in the bitstream, rather than including it in the profile_tier_level( ) syntax structure, which would be sent for each independent layer. It is noted that JVET-N0276 separately proposes the inclusion of a  sub_profile_idc syntax element in the profile_tier_level( ) syntax structure, which would be sent for each independent layer.

The –v2 version of this document provides updated specification text for the sub-bitstream extraction process to incorporate a targerLayerId value.

# Syntax and semantics

## NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| nuh_layer_id | u(7) |
| } |  |

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id shall be in the range of 0 to 126, inclusive. The value of 127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 for nuh_layer_id in a NAL unit, and decoders conforming to a profile specified in Annex A.

NOTE 1 – The value of 127 for nuh_layer_id may be used to indicate an extended layer identifier in a future extension of this Specification.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture is the value of the nuh_layer_id of the VCL NAL units of the coded picture.

## Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_ parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_1bit | u(1) |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_parameter_set_id identifies the VPS for reference by other syntax elements.

vps_max_layers_minus1 plus 1 specifies the maximum allowed number of layers in each CVS referring to the VPS. vps_max_layers_minus1 shall be less than 127 in bitstreams conforming to this version of this Specification. The value of 63 for vps_max_layers_minus1 is reserved for future use by ITU-T | ISO/IEC. Although the value of vps_max_layers_minus1 is required to be less than 127 in this version of this Specification, decoders shall allow the value of vps_max_layers_minus1 equal to 127 to appear in the syntax.

NOTE 2 – The value of 127 for vps_max_layers_minus1 may be used to indicate an extended number of layers in a future extension where more than 127 layers in a bitstream need to be supported.

The variable MaxLayersMinus1 is set equal to Min( 126, vps_max_layers_minus1 ).

vps_included_layer_id[ i ] specifies the nuh_layer_id value of the i-th included layer.

vps_reserved_zero_1bit shall be equal to 0 in bitstreams conforming to this version of this Specification. The value 1 for vps_reserved_zero_1bit is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_bit.

NOTE 3 – The value of 1 for vps_reserved_zero_1bit may be used to indicate support for inter-layer prediction in a future extension.

## General profile, tier, and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

## Sub-bitstream extraction process

Inputs to this process are a bitstream and a target highest TemporalId value tIdTarget and a target nuh_layer_id value LIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, tIdTarget equal to any value in the range of 0 to 6, inclusive, and LIdTarget equal to any value in the range of 0 to 126, inclusive, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget and nuh_layer_id equal to LIdTarget..

NOTE – A conforming bitstream contains one or more coded tile group NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	Remove all NAL units with nuh_layer_id not equal to LIdTarget.

## General decoding process

    - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

The variable LIdTarget, which identifies the target layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set LIdTarget, LIdTarget is set by the external means.

–	Otherwise, LIdTarget is set equal to vps_included_layer_id[ 0 ].

If vps_max_layers_minus1 is greater than 0, the sub-bitstream extraction process as specified in clause 10 is applied with the bitstream with LIdTarget and 7 as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

At the beginning of decoding a CVS, after activating the SPS RBSP that is active for the entire CVS and before decoding any VCL NAL units of the CVS, clause 8.1.2 is invoked with the CVS as input.

# Updated design syntax

# Updated design syntax is provided to reflect the BoG recommendation.

# In this revised design, the VPS would be expected to be replicated for each layer (e.g. for each value of nuh_layer_id), and have the same contents. No parameter set sharing across layers is supported. All activated parameter sets must have the same nuh_layer_id value as the VCL NAL units that reference them.

# An HRD for this design would operate on a single layer basis.

# In this updated design, presence of general_constraint_info( ) in the VPS is optional.

# The individual constraints in the general_constraint_info( ) syntax structure in the SPS may be equal to or more restrictive than the coresponding constraints in the VPS.

# Other mechanisms could be further studied to enable parameter set sharing.

## NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| nuh_layer_id | u(7) |
| } |  |

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id shall be in the range of 0 to 126, inclusive. The value of 127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 for nuh_layer_id in a NAL unit, and decoders conforming to a profile specified in Annex A.

NOTE 1 – The value of 127 for nuh_layer_id may be used to indicate an extended layer identifier in a future extension of this Specification.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture is the value of the nuh_layer_id of the VCL NAL units of the coded picture.

## Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_ parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_1bit | u(1) |
| vps_general_constraint_info_present_flag | u(1) |
| if (vps_general_constraint_info_present_flag) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_parameter_set_id identifies the VPS for reference by other syntax elements.

All video_parameter_set_rbsp( ) syntax structures with the same value of  vps_parameter_set_id shall contain the same parameters.

vps_max_layers_minus1 plus 1 specifies the maximum allowed number of layers in each CVS referring to the VPS. vps_max_layers_minus1 shall be less than 127 in bitstreams conforming to this version of this Specification. The value of 63 for vps_max_layers_minus1 is reserved for future use by ITU-T | ISO/IEC. Although the value of vps_max_layers_minus1 is required to be less than 127 in this version of this Specification, decoders shall allow the value of vps_max_layers_minus1 equal to 127 to appear in the syntax.

NOTE 2 – The value of 127 for vps_max_layers_minus1 may be used to indicate an extended number of layers in a future extension where more than 127 layers in a bitstream need to be supported.

The variable MaxLayersMinus1 is set equal to Min( 126, vps_max_layers_minus1 ).

vps_included_layer_id[ i ] specifies the nuh_layer_id value of the i-th included layer.

vps_reserved_zero_1bit shall be equal to 0 in bitstreams conforming to this version of this Specification. The value 1 for vps_reserved_zero_1bit is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_bit.

NOTE 3 – The value of 1 for vps_reserved_zero_1bit may be used to indicate support for inter-layer prediction in a future extension.

vps_general_constraint_info_present_flag equal to 1 indicates that the general_constraint_info( ) syntax structure is present in the VPS syntax structure.

## General profile, tier, and level syntax

## In this updated proposal (unlike the in the initial proposal described in Section 2), there is no change to the profile_tier_level(  ) syntax structure.

## One example is provided of how all of the constraint flag syntax element semantics will be updated.

## no_sao_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_sao_enabled_flag shall be equal to 0. no_sao_constraint_flag equal to 0 does not impose a constraint. The value of no_sao_constraint_flag signalled in the SPS for BitstreamInScope shall be greater than or equal to value of no_sao_constraint_flag signalled in the VPS for a CVS contained within BitstreamInScope.

## Sub-bitstream extraction process

## Same as in section 2.4.

## General decoding process

## Same as in section 2.5.

# Patent rights declaration(s)

Intel does not have any current or pending patent rights relating to the technology described in this contribution.