| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0280 |
| --- | --- |

| Title: | CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Bross T. Nguyen H. Schwarz D. Marpe T. Wiegand Einsteinufer 37 10587 Berlin Germany | Tel: Email: | - benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results for the experiments CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b that were conducted in the context of CE8 on Screen Content Coding Tools. Specifically, the performance of the residual coding for the transform skip mode proposed in JVET-M0464 is evaluated together with the restriction on the number of context coded bins. The luma BD-rate results for the CTC (IBC=1) are as follows for CE8-3a, CE8-3b, CE8-4a, and CE8-4b in class F and TGM, respectively:

Class F: 	−1.83 % (AI), −0.96 % (RA), −1.22 % (LB); 
TGM: 		−1.85 % (AI), −0.90 % (RA), −0.78 % (LB);

Class F: 	−2.67 % (AI), −1.77 % (RA), −1.59 % (LB);
TGM: 		−4.00 % (AI), −2.58 % (RA), −2.54 % (LB);

Class F: 	−2.57 % (AI), −1.74 % (RA), −1.39 % (LB);
TGM: 		−3.87 % (AI), −2.67 % (RA), −2.33 % (LB);

Class F: 	−2.79 % (AI), −1.93 % (RA), −1.59 % (LB);
TGM: 		−4.37 % (AI), −2.96 % (RA), −2.52 % (LB);

# Introduction

The conducted experiments evaluate the performance of the residual coding proposed in JVET-M0464 in combination with the restriction of the number of context coded bins provided in JVET-M0464 (CE8-4.3a and CE8-4.3b) and JVET-M0449 (CE8-4.4a and CE8-4.4b). Additionally, the configuration without context coded bins limitation is also provided and serves as the reference for comparison. Note that for each sub-experiment in CE8, two different configurations (IBC on and off) were requested with each consists of two operation ranges (CTC and low QP), resulting in a total number of anchors vs tested combinations equal to four for each experiment. Since the compression efficiency for the natural content sequences are almost neutral or slightly improved and the run times for both the encoder and the decoder are around 100%, those numbers were skipped in the presentation within this document. The detailed results can be found in the included Excel sheets provided together with this document.

# Observations

For the approach provided in JVET-M0464, the restriction to three context coded bins is sufficient for the CTC operation range. The performance for the restriction provided in JVET-M0449 performs better given the same number of context coded bins. Especially for the low QP operation range the JVET-M0449 approach retains compression efficiency improvements. However, the results without restriction cannot be achieved in the low QP operation range.

There exists an anomaly for the experiment CE8-4.4b (IBC=1 configuration) in the low QP operation range in the Random Access temporal configuration. The BD-rate result for the TGM sequences is even higher relative to the no restriction case. By analyzing the detailed results, the anomaly is due to the result for the  Desktop sequence generated by using the QP equal to 7.

# Results for IBC=0 configuration

## CE8-4.3: no restrictions on context coded bins

Class F: 	−5.59 % (AI), −3.79 % (RA), −2.47 % (LB); 
TGM: 		−14.6 % (AI), −7.54 % (RA), −3.70 % (LB);

## CE8-4.3a: M0464 restriction with max. two bins

Class F: 	−3.47 % (AI), −1.96 % (RA), −1.76 % (LB); 
TGM: 		−8.75 % (AI), −3.81 % (RA), −2.52 % (LB);

## CE8-4.3b: M0464 restriction with max. three bins

Class F: 	−5.52 % (AI), −3.41 % (RA), −2.39 % (LB); 
TGM: 		−13.7 % (AI), −6.81 % (RA), −3.48 % (LB);

## CE8-4.4a: M0449 restriction with max. two bins

Class F: 	−4.70 % (AI), −2.94 % (RA), −2.06 % (LB); 
TGM: 		−12.6 % (AI), −6.27 % (RA), −3.29 % (LB);

## CE8-4.4b: M0449 restriction with max. three bins

Class F: 	−5.48 % (AI), −3.65 % (RA), −2.59 % (LB); 
TGM: 		−14.4 % (AI), −7.27 % (RA), −3.59 % (LB);

## CE8-4.3: no restrictions on context coded bins for the low QP range

Class F: 	−1.20 % (AI), −1.19 % (RA), −0.87 % (LB); 
TGM: 		−6.10 % (AI), −6.08 % (RA), −3.04 % (LB);

## CE8-4.3a: M0464 restriction with max. two bins for the low QP range

Class F: 	+0.74 % (AI), +0.86 % (RA), +0.85 % (LB); 
TGM: 		+2.74 % (AI), +3.70 % (RA), +4.33 % (LB);

## CE8-4.3b: M0464 restriction with max. three bins for the low QP range

Class F: 	−0.21 % (AI), −0.08 % (RA), −0.12 % (LB); 
TGM: 		−2.82 % (AI), −1.36 % (RA), +0.21 % (LB);

## CE8-4.4a: M0449 restriction with max. two bins for the low QP range

Class F: 	−0.30 % (AI), −0.31 % (RA), −0.17 % (LB); 
TGM: 		−2.46 % (AI), −1.99 % (RA), −0.38 % (LB);

## CE8-4.4b: M0449 restriction with max. three bins for the low QP range

Class F: 	−0.75 % (AI), −0.70 % (RA), −0.57 % (LB); 
TGM: 		−4.74 % (AI), −3.50 % (RA), −0.76 % (LB);

# Results for IBC=1 configuration

## CE8-4.3: no restrictions on context coded bins

Class F: 	−2.81 % (AI), −1.98 % (RA), −1.61 % (LB); 
TGM: 		−4.46 % (AI), −3.05 % (RA), −2.43 % (LB);

## CE8-4.3a: M0464 restriction with max. two bins

Class F: 	−1.83 % (AI), −0.96 % (RA), −1.22 % (LB); 
TGM: 		−1.85 % (AI), −0.90 % (RA), −1.78 % (LB);

## CE8-4.3b: M0464 restriction with max. three bins

Class F: 	−2.67 % (AI), −1.77 % (RA), −1.59 % (LB); 
TGM: 		−4.00 % (AI), −2.58 % (RA), −2.54 % (LB);

## CE8-4.4a: M0449 restriction with max. two bins

Class F: 	−2.57 % (AI), −1.74 % (RA), −1.39 % (LB); 
TGM: 		−3.87 % (AI), −2.67 % (RA), −2.33 % (LB);

## CE8-4.4b: M0449 restriction with max. three bins

Class F: 	−2.79 % (AI), −1.93 % (RA), −1.59 % (LB); 
TGM: 		−4.37 % (AI), −2.96 % (RA), −2.52 % (LB);

## CE8-4.3: no restrictions on context coded bins for the low QP range

Class F: 	−1.02 % (AI), −1.03 % (RA), −0.55 % (LB); 
TGM: 		−4.05 % (AI), −5.05 % (RA), −2.87 % (LB);

## CE8-4.3a: M0464 restriction with max. two bins for the low QP range

Class F: 	+1.06 % (AI), +0.97 % (RA), +1.05 % (LB); 
TGM: 		+4.05 % (AI), +2.24 % (RA), +3.91 % (LB);

## CE8-4.3b: M0464 restriction with max. three bins for the low QP range

Class F: 	−0.14 % (AI), −0.20 % (RA), +0.09 % (LB); 
TGM: 		−0.85 % (AI), −3.70 % (RA), −0.08 % (LB);

## CE8-4.4a: M0449 restriction with max. two bins for the low QP range

Class F: 	−0.34 % (AI), −0.45 % (RA), −0.12 % (LB); 
TGM: 		−1.61 % (AI), −5.57 % (RA), −0.60 % (LB);

## CE8-4.4b: M0449 restriction with max. three bins for the low QP range

Class F: 	−0.70 % (AI), −0.82 % (RA), −0.37 % (LB); 
TGM: 		−2.96 % (AI), −9.82 % (RA), −1.97 % (LB);

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).