| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0281-v1 |
| --- | --- |

| Title: | CE7-related: Constraint on the number of regular bins per subblock |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 4, the constraint on the maximum allowed number of context-coded bins is defined for the 1st subblock coding pass. The total number of the context-coded bins may exceed the constraint value by 1 for coding a subblock when the syntax element coded_sub_block_flag is present in the current subblock. In this contribution, we propose to enforce the constraint value on the maximum allowed number of regular bins for coding a subblock, further counting the regular bin for coding coded_sub_block_flag. The regular bin budget for the 1st subblock coding pass is decremented by 1 when coded_sub_block_flag is present in the current subblock. The proposed method reportedly generates 0.00%, 0.00, and -0.02% luma BD-rates for the AI, RA and LB settings, respectively, under the JVET CTCs.

# Introduction

In VVC Draft 4, the constraint on the maximum allowed number of context-coded bins is defined for the 1st subblock coding pass, comprising coding syntax elements sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag. The maximum allowed number of context-coded bins is equal to 32 for a 4x4 subblock and equal to 8 for a 2x2 subblock. It aims at improving the worst-case bitstream parsing throughtput rate per transform sub-block. However, the number of the context-coded bins may exceed the defined constraint value by 1 in a sub-block when the syntax element coded_sub_block_flag is present in the current subblock.

# Proposed method

In this contribution, we propose to enforce the constraint on the maximum allowed number of context-coded bins for coding each subblock, further counting the regular bin for coding coded_sub_block_flag. The maximum allowed number of regular bins is equal to 32 for a 4x4 subblock and equal to 8 for a 2x2 subblock. The regular bin budget for the 1st subblock coding pass is decremented by 1 when coded_sub_block_flag is present in the current subblock. The regular bin budget for the 1st subblock coding pass is the same as the constraint value when coded_sub_block_flag is not present in the current subblock, corresponding to the 1st subblcok or the last significant subblock.

# Experimental results

The proposed method was integrated into the VTM4.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM4.0 reference software. Table 1 summarizes the experimental results conducted under the CTCs specified in [3]. It is shown that the proposed method does not exhibit significant difference in BD-rate and run time compared with the anchor results. The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results using the proposed method

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 102% | 106% |
| Class A2 | 0.01% | 0.17% | -0.03% | 102% | 103% |
| Class B | 0.01% | 0.05% | 0.00% | 100% | 100% |
| Class C | 0.01% | -0.03% | -0.07% | 101% | 100% |
| Class E | -0.01% | 0.06% | 0.05% | 101% | 101% |
| Overall | 0.00% | 0.04% | -0.01% | 101% | 102% |
| Class D | 0.02% | -0.03% | -0.12% | 102% | 101% |
| Class F | 0.01% | -0.03% | -0.05% | 99% | 98% |
|  |  | Random access Main10 |  |  |  |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.10% | 102% | 101% |
| Class A2 | -0.01% | 0.11% | 0.08% | 100% | 102% |
| Class B | -0.01% | 0.08% | -0.14% | 101% | 103% |
| Class C | 0.02% | 0.04% | 0.06% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.00% | 100% | 102% |
| Class D | 0.02% | -0.06% | -0.24% | 99% | 101% |
| Class F | -0.02% | -0.03% | 0.09% | 99% | 108% |
|  |  | Low delay B Main10 |  |  |  |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.35% | -0.21% | 102% | 104% |
| Class C | -0.06% | 0.31% | -0.19% | 101% | 100% |
| Class E | 0.03% | 0.08% | 1.93% | 102% | 100% |
| Overall | -0.02% | -0.02% | 0.33% | 102% | 102% |
| Class D | 0.01% | 0.38% | 0.08% | 104% | 101% |
| Class F | -0.16% | 0.01% | 0.33% | 100% | 101% |

# Conclusions

In this contribution, we propose to enforce the constraint on the maximum allowed number of context-coded bins for each subblock. The regular bin budget for the 1st subblock coding pass is decremented by 1 when coded_sub_block_flag is present in the current sub-block. It can be guaranteed that no more than 32 regular bins are consumed for coding each 4x4 subblock and no more than 8 regular bins are consumed for coding each 4x4 subblock. Experimental results show the proposed method generates minor impacts on BD bitrate results.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 4),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- The software VTM4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).