| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0283-v1 |
| --- | --- |

| Title: | CE1-5.4: mutual exclusion of LIC and BPWA |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Fabrice Urban, Tangi Poirier | Email: | philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.urban@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to reduce pipeline for LIC by disabling BPWA when LIC is enabled at CU level.

The proposed process has been implemented both in the JVET VTM4.0 on top of bi-prediction LIC (CE1-5.3). The reported simulation results in Y,Cb,Cr in RA and LDB show an average BD-rate gain of:

CE1-5.4a:	-0.52%, -0.17%, -0.19%		(RA configuration)

-0.56%, -0.35%, -0.65% 	(LDB configuration)

CE1-5.4b:	-0.43%, -0.38%, -0.38%		(RA configuration)

-0.49%, -0.29%, -0.40% 	(LDB configuration).

# Introduction

During the last JVET Meeting (Marrakech, MA, 9–18 Jan. 2019), to minimize implementation complexity of LIC and post-reconstruction filtering, it was recommended to follow some constraints when applying to current CU:

  - Not using reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

  - Disable for CIIP blocks

  - No temporal inheritance of LIC flag

  - No pruning based on LIC flag in merging candidate list generation

  - Disable LIC for all subblock modes

  - The VPDU issue should be solved (e.g., disabling for 128xN or by other method which avoids violating VPDU constraint).

  - Bi-Prediction issue should be solved (LIC should not have to be applied twice, i.e. for each L0/L1). LIC is applied after bi-predictive combination.

These limitations have been applied to LIC.

# Proposed pipeline reduction for LIC

This test is implemented on top of CE1-test-1.5.3 that proposes to apply LIC after bi-prediction is derived. When LIC and BPWA with non-default weights are enabled together, two successive weighting and clipping stages are cascaded, what may be sub-optimal. It may be also counter-productive to apply additional BPWA non-default weights on top of LIC weights that were specially computed to compensate for illumination variation.

To overcome this inconsistency and to alleviate the inter pipeline when LIC is enabled, it is proposed:

- Mutual exclusion of LIC and BPWA.

- LIC-flag is decoded first. If LIC-flag is true, then GBi-index is not decoded and inferred to defaults.

Figure 2: proposed reduced LIC pipeline in inter (top).

Besides alleviating the decoder pipeline worst case, the proposed modification allows reducing the encoder modes combinatory.

The Table 1 summarizes the results of the proposed reduced pipeline for LIC implemented on VTM-4.0 (CE1-5.4a) with CTC, on random access (RA) and low delay B (LDB).

The Table 2 summarizes the results of the proposed reduced pipeline for LIC implemented on VTM-4.0 (CE1-5.4b) with CTC, on random access (RA) and low delay B (LDB), where neighbor samples of intra, CIIP and IBC coded blocks are excluded, i.e. only inter reconstructed samples are used.

Table 1: BD-rate gains of the proposed reduced pipeline for LIC (CE1-5.4a) compared to VTM4.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | 0.15% | 0.17% | 142% | 104% |
| Class A2 | -0.35% | -0.28% | -0.33% | 130% | 108% |
| Class B | -0.56% | -0.20% | -0.18% | 133% | 109% |
| Class C | -0.38% | -0.26% | -0.37% | 137% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.17% | -0.19% | 135% | 107% |
| Class D | -0.15% | -0.06% | -0.08% | 134% | 108% |
| Class F | -1.00% | -0.65% | -0.75% | 124% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.79% | -0.69% | -0.78% | 149% | 114% |
| Class C | -0.68% | -0.39% | -0.69% | 147% | 104% |
| Class E | -0.03% | 0.25% | -0.36% | 124% | 107% |
| Overall | -0.56% | -0.35% | -0.65% | 142% | 109% |
| Class D | -0.18% | 0.04% | -0.51% | 139% | 106% |
| Class F | -3.14% | -2.19% | -2.25% | 135% | 105% |

Table 2: BD-rate gains of the proposed reduced pipeline for LIC (CE1-5.4b) compared to VTM4.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -0.42% | -0.52% | 149% | 108% |
| Class A2 | -0.29% | -0.35% | -0.36% | 134% | 110% |
| Class B | -0.41% | -0.35% | -0.24% | 134% | 106% |
| Class C | -0.35% | -0.41% | -0.47% | 141% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.38% | -0.38% | 139% | 108% |
| Class D | -0.14% | -0.22% | -0.23% | 138% | 109% |
| Class F | -0.84% | -0.56% | -0.60% | 130% | 108% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.70% | -0.49% | -0.42% | 152% | 113% |
| Class C | -0.62% | -0.40% | -0.66% | 152% | 106% |
| Class E | 0.06% | 0.19% | 0.00% | 123% | 106% |
| Overall | -0.49% | -0.29% | -0.40% | 144% | 109% |
| Class D | -0.22% | -0.36% | -0.78% | 142% | 108% |
| Class F | -2.78% | -2.95% | -3.35% | 139% | 107% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).