| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0284 |
| --- | --- |

| Title: | AHG17: On ALF and Reshaper Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email | 1-858-635-5300 seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

Abstract

# This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters. In the latest VVC WD, the ALF parameters are signaled in the APS while the reshaper model is signaled in the tile group header. This contribution proposes to signal in-loop reshaper in the APS along with the ALF parameters to avoid redundant transmission of the reshaper model parameters. It is considered beneficial to signal both these parameters together instead of creating individual parameters sets as it aims to reduce redundancy by group tools with similar functionality together.

- Introduction

In the current design of Versatile Video Coding(VVC), the Adaptation Parameter Set (APS) has been adopted for the carriage of the Adaptive Loop Filter (ALF) parameters. This serves to reduce the redundant and unnecessary transmissions of the ALF parameters. The use of the ALF tool is enabled in the Sequence Parameter Set (SPS) Similar to the ALF, the in-loop reshaper tool is also enabled in the SPS. However, the use of the in-loop reshaper is controlled in the tile group header by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag. If the tile_group_reshaper_model_present_flag is enabled in the tile group header, then it indicates that a reshaping model for that tile group is present. The tile_group_reshaper_enable_flag is further parsed to decipher if the reshaper model is used in the tile group header or not. As these two flags are signaled independently of one another.

- Proposal

We propose to minimize the redundant transmissions of the in-loop reshaper by using the APS for the carriage of the ALF parameters as well as the reshaper model parameters. An advantage of using the APS for the carriage of ALF and in-loop reshaper model is that it can be sent out-of-band. This could enable optimal delivery of VVC coded video especially for streaming applications. In another example, in the ISO base media file format (ISOBMFF) environment, APS can be carried in a time-synchronized track. Therefore, the tile groups, which may contain a group of complete tiles may be carried in thier own file format tracks,e.g., named tile tracks and the APSs can be carried in the time synchronized track. Subsequently, these tracks can be carried in a DASH respresentation, and for decoding and rendering of a subset of the tile tacks, the DASH representations containing the subset of the tile tracks as well as the DASH respresentation containing the APSs may be requested by the client, segment by segment, thereby reducing the redundancy of the ALF and reshaper model parameters [1].

Table 1shows the ALF and Reshaper in the APS.

| adaptation_parameter_set_rbsp(){ | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | ue(v) |
| alf_data() |  |
| reshaper_model( ) |  |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The Table 1 above shows the in-loop reshaper parameters being parsed in the APS. Here, the syntax reshaper_model() represents the syntax elements present within the tile_group_reshaper_model().

Table 1 shows the Tile Group Header Syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| … |  |
| if (sps_alf_enabled_flag) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if(tile_group_alf_enabled_flag){ |  |
| tile_group_aps_id_alf | u(5) |
| } |  |
| } |  |
| if (sps_reshaper_enabled_flag) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if(tile_group_reshaper_model_present_flag){ |  |
| tile_group_aps_id_reshaper | u(5) |
| } |  |
| tile_group_reshaper_enable_flag | u(1) |
| if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

tile_group_aps_id_alf specifies the adaptation_parameter_set_id of the APS that the tile group refers to access the ALF data. The TemporalId of the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id_alf shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple APSs with the same value of adaptation_parameter_set_id shall have the same content.

tile_group_aps_id_reshaper specifies the adaptation_parameter_set_id of the APS that the tile group refers to access the reshaper data. The TemporalId of the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id_reshaper shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple APSs with the same value of adaptation_parameter_set_id shall have the same content.

- Reference

[1] Y-K Wang,  H. FNU, J. Chen, “AHG17: On header parameter set (HPS).” Document of Joint Video Experts Team (JVET), JVET-M0132, 13th Meeting: Marrakesh, MA, 9-18 Jan. 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).