| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0285 |
| --- | --- |

| Title: | CE4-2.2 : Pairwise extension with STMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Le Léannec, A. Robert, T. Poirier 975 Avenue des Champs Blancs 35576 Cesson-Sévigné FRANCE | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.comantoine.robert@technicolor.com, tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of CE4-2.2. Pairwise average merge candidate is extended with one spatial-temporal motion vector predictor (STMVP) candidate added to the regular merge list construction. No merge candidate pruning, or motion vector scaling is added to VTM-4.0. Based on the last meeting comments, the construction of the merge list no more depends on the frame dependencies.

It is reported that the BD-rate gain in RA is 0.13%. In LDB, the coding efficiency is the same as VTM-4.0 despite the use of STMVP.

# Proposed technique

The proposed added merge candidate consists in an average between the temporal TMVP merge candidate, and one or two spatial ones.

The STMVP motion vector predictor is compute as follows:

- The use of STMVP is signaled by a slice header flag, similarly as TMVP.

- The temporal MV predictor used to compute the STMVP motion vector is the classical TMVP merge candidate

- The spatial candidates used are the first and second candidates contained in the merge list at the time the STMVP candidate is being computed.

- A spatial candidate is considered as available if its reference picture index is 0. Thus, compared to pairwise, the availability test is changed from (refIdx != NOT_VALID) to (refIdx == 0).

- For each reference picture list L0 and L1, the following applies:

  - If 3 candidates are available, then the STMVP candidate is available and is computed as:

    - MVSTMVP=3×MVS1+3×MVS2+2×TMVP≫3

  - If one spatial candidate MVS and the TMVP candidate are available, then:

    - MVSTMVP=MVS+TMVP≫1

The STMVP candidate, if available, is placed before the B2 candidate in the merge candidate list. The list of potential merge candidates is thus the following one:

- A1, B1, B0, A0, STMVP, B2, TMVP, HMVP, Pairwise, Zero

# Experimental results

Table 1 shows the performances of the proposed STMVP technique over VTM-4.0. A BD-rate gain of 0.13% in RA is observed.

Complementary results are provided by Table 2. They show the tool-off result of the proposed extension of pairwise average merge candidate, when compared to VTM-4.0 with no pairwise candidate use. It is shown that the overall loss increases from 0.19% (AHG13 result of pairwise) to 0.31% in RA.

Table 1: performance of the proposed spatial temporal motion vector prediction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.12% | -0.11% | 101% | 100% |
| Class A2 | -0.15% | -0.04% | -0.05% | 101% | 100% |
| Class B | -0.07% | -0.06% | -0.09% | 101% | 102% |
| Class C | -0.13% | -0.17% | -0.15% | 101% | 101% |
| Overall | -0.13% | -0.10% | -0.10% | 101% | 101% |
| Class D | -0.19% | -0.16% | -0.19% | 100% | 100% |
| Class F (optional) | 0.02% | 0.06% | 0.02% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | 0.15% | 0.25% | 101% | 101% |
| Class C | -0.03% | 0.22% | 0.13% | 102% | 102% |
| Class E | 0.03% | 0.40% | -0.31% | 101% | 105% |
| Overall | 0.00% | 0.23% | 0.07% | 102% | 103% |
| Class D | -0.01% | -0.20% | -0.22% | 101% | 103% |
| Class F (optional) | -0.04% | 0.08% | -0.33% | 101% | 102% |

Table 2: tool-off performance: VTM-4.0 without pairwise compared to the proposed PMC extension

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.38% | 0.09% | 0.27% | #NUM! | #NUM! |
| Class A2 | 0.45% | 0.20% | 0.22% | #NUM! | #NUM! |
| Class B | 0.23% | 0.07% | 0.29% | #NUM! | #NUM! |
| Class C | 0.27% | 0.20% | 0.14% | #NUM! | #NUM! |
| Overall | 0.31% | 0.14% | 0.23% | #NUM! | #NUM! |
| Class D | 0.34% | 0.31% | 0.16% | #NUM! | #NUM! |
| Class F | 0.04% | -0.09% | -0.14% | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.03% | -0.40% | -0.49% | #NUM! | #NUM! |
| Class C | 0.06% | 0.02% | -0.17% | #NUM! | #NUM! |
| Class E | -0.11% | 0.00% | -0.12% | #NUM! | #NUM! |
| Overall | -0.02% | -0.16% | -0.29% | #NUM! | #NUM! |
| Class D | 0.00% | -0.06% | 0.13% | #NUM! | #NUM! |
| Class F | -0.09% | 0.14% | -0.14% | #NUM! | #NUM! |

# Complexity analysis

Table 3: complexity analysis of proposed STMVP technique over VTM-4.0

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | +1 | +0 | +0 | +0 | +0 | +0 | Availability test changed from (refIdx != NOT_VALID) to (refIdx==0) compared to pairwise |

# Conclusion

This contribution proposes a spatial-temporal merge mode to extend the pairwise average merge candidate. It reportedly brings 0.13% gain in coding efficiency in RA and does not impact the coding efficiency of LDB.  It is suggested to adopt it into the next version of the VTM.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).