### Raw byte sequence payloads, trailing bits and byte alignment syntax

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_max_dec_pic_buffering_minus1 | ue(v) |
| long_term_ref_pics_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < !rpl1_same_as_rpl0_flag ? 2 : 1; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j ) |  |
| } |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_tile_group | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_tile_group_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_tile_groups != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_tile_group | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_tile_group | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_tile_group_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_tile_group_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_tile_group_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_tile_group_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_tile_group_chroma | ue(v) |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_pcm_enabled_flag | u(1) |
| if( sps_pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_spatial_temporal_mvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_gbi_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| sps_lmcs_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.3.4 Tile group header syntax

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( tile_group_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( sps_spatial_temporal_mvp_enabled_flag ) |  |
| tile_group_spatial_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| tile_group_aps_id | u(5) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if ( sps_lmcs_enabled_flag ) { |  |
| tile_group_lmcs_model_present_flag | u(1) |
| if ( tile_group_lmcs_model_present_flag ) |  |
| lmcs_data( ) |  |
| tile_group_lmcs_enabled_flag | u(1) |
| if ( tile_group_lmcs_enabled_flag && !( qtbtt_dual_tree_intra_flag && tile_group_type = = I ) ) |  |
| tile_group_chroma_residual_scale_flag | u(1) |
| } |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

…

### 7.4.3 Raw byte sequence payloads, trailing bits and byte alignment semantics

#### 7.4.3.1 Sequence parameter set RBSP semantics

…

sps_temporal_mvp_enabled_flag equal to 1 specifies that tile_group_temporal_mvp_enabled_flag is present in the tile group headers of tile groups with tile_group_type not equal to I in the CVS. sps_temporal_mvp_enabled_flag equal to 0 specifies that tile_group_temporal_mvp_enabled_flag is not present in tile group headers and that temporal motion vector predictors are not used in the CVS.

sps_spatial_temporal_mvp_enabled_flag equal to 1 specifies that tile_group_spatial_temporal_mvp_enabled_flag is present in the tile group headers of tile groups with tile_group_type not equal to I in the CVS. sps_spatial_temporal_mvp_enabled_flag equal to 0 specifies that tile_group_spatial_temporal_mvp_enabled_flag is not present in tile group headers and that temporal motion vector predictors are not used in the CVS.

sps_sbtmvp_enabled_flag equal to 1 specifies that subblock-based temporal motion vector predictors may be used in decoding of pictures with all tile groups having tile_group_type not equal to I in the CVS. sps_sbtmvp_enabled_flag equal to 0 specifies that subblock-based temporal motion vector predictors are not used in the CVS. When sps_sbtmvp_enabled_flag is not present, it is inferred to be equal to 0.

…

### 7.4.5 Tile group header semantics

#### 7.4.5.1 General tile group header semantics

…

tile_group_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction. If tile_group_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no temporal motion vector predictor is used in decoding of the current picture. Otherwise (tile_group_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of tile_group_temporal_mvp_enabled_flag is inferred to be equal to 0.

tile_group_spatial_temporal_mvp_enabled_flag specifies whether spatial temporal motion vector predictors can be used for inter prediction. If tile_group_spatial_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no STMVP vector predictor is used in decoding of the current picture. Otherwise (tile_group_spatiial_temporal_mvp_enabled_flag is equal to 1), STMVP vector predictors may be used in decoding of the current picture. When not present, the value of tile_group_spatial_temporal_mvp_enabled_flag is inferred to be equal to 0.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to 0 for compIdx = 0..1. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed.

…

….

#### 8.5.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-276)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-277)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-278)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-279)

smrNumHmvpCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-280)

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.5.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, the luma coding block height cbHeight set equal to smrWidth and smrHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-281)

predFlagL0Col = availableFlagL0Col		(8-282)

predFlagL1Col = 0		(8-283)

- When tile_group_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, the luma coding block height cbHeight set equal to smrWidth and smrHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-284)

predFlagL1Col = availableFlagL1Col		(8-285)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-286)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0

- The derivation process for spatial temporal luma motion vector prediction as specified in in clause 8.5.2.X is invoked with the availability flag availableFlagCol, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and Col, and the output being the availability flags availableFlagSTMVP, the reference indices refIdxLXSTMVP, the prediction list utilization flags predFlagLXSTMVP, the bi-prediction weight index gbiIdxSTMVP and the motion vectors mvLXSTMVP, with X being 0 or 1.

- The merging candidate list, mergeCandList, is constructed as follows:

if( availableFlagSTMVP)
	mergeCandList[ i++ ] = STMVP

if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and smrNumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than MaxNumMergeCand and greater than 1, the following applies:

- The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index gbiIdx of candidate avgCand being added into mergeCandList is set equal to 0.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for zero motion vector merging candidates specified in clause 8.5.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-287)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-288)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-289)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-290)

gbiIdx = gbiIdxN		(8-291)

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.5.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-292)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-293)

…

#### 8.5.2.X Derivation process for spatial temporal merging candidate

Inputs to this process are

- the availability flag availableFlagCol,

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of candidate N in mergeCandList and Col,

- the prediction list utilization flags predFlagL0N and predFlagL1N of candidate N in mergeCandList and Col,

- the motion vectors sample mvL0N and mvL1N of candidate N in mergeCandList and Col,

Outputs of this process are (with X being 0 or 1, respectively):

- the availability flag availableFlagSTMVP,

- the reference indices refIdxL0STMVP and refIdxL1STMVP,

- the prediction list utilization flags predFlagL0STMVP and predFlagL1STMVP,

- the bi-prediction weight index gbiIdxSTMVP,

- the motion vector array mvLXSTMVP.

If one or more of the following conditions is true, availableFlagSTMVP is set equal to 0, both components of mvLXSTMVP are set equal to 0, refIdxLXSTMVP is set equal to −1 and predFlagLXSTMVP is set equal to 0, with X being 0 or 1:

  - availableFlagCol is equal to 0.

  - If tile_group_spatial_temporal_mvp_enabled_flag is equal to 0, both components of mvLXSTMVP are set equal to 0 and availableFlagSTMVP is set equal to 0.

Otherwise the following ordered steps applies:

- The following assignments are made, with p0Cand being the candidate at position 0 and p1Cand being the candidate at position 1 in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ 0 ]	(8-252)

p1Cand = mergeCandList[ 1 ]	(8-253)

- The variables refIdxLXSTMVP, predFlagLXSTMVP and mvLXSTMVP with X being 0 or 1 are derived in the following steps:

  - If predFlagLXp0Cand is equal to 1, refIdxLXp0Cand is equal to 0, predFlagLXp1Cand is equal to 1 and refIdxLXp1Cand is equal to 0, the variables refIdxLXSTMVP, predFlagLXSTMVP, mvLXSTMVP[ 0 ], and mvLXSTMVP[ 1 ] are derived as follows:

refIdxLXSTMVP = 0	(8-254)

predFlagLXSTMVP = 1	(8-255)

mvLXSTMVP[ 0 ] = (3 *mvLXp0Cand[ 0 ] + 3 *mvLXp1Cand[ 0 ] + 2* mvLXCol[ 0 ]) / 8	(8-256)

mvLXSTMVP[ 1 ] = (3 *mvLXp0Cand[ 1 ] + 3 *mvLXp1Cand[ 1 ] + 2* mvLXCol[ 1 ]) / 8	(8-257)

  - Otherwise, if predFlagLXp0Cand is equal to 1, refIdxLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXSTMVP, predFlagLXSTMVP, mvLXSTMVP[ 0 ], mvLXSTMVP[ 1 ] are derived as follows:

refIdxLXSTMVP = 0	(8-260)

predFlagLXSTMVP = 1	(8-261)

mvLXSTMVP[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXCol[ 0 ]  ) / 2	(8-262)

mvLXSTMVP[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXCol[ 1 ] ) / 2	(8-263)

  - Otherwise, if predFlagLXp0Cand is equal to 0 predFlagLXp1Cand is equal to 1 and refIdxLXp1Cand is equal to 0, the variables refIdxLXSTMVP, predFlagLXSTMVP, mvLXSTMVP[ 0 ], mvLXSTMVP[ 1 ] are derived as follows:

refIdxLXSTMVP = 0	(8-264)

predFlagLXSTMVP = 1	(8-265)

mvLXSTMVP[ 0 ] = ( mvLXp1Cand[ 0 ] + mvLXCol[ 0 ] ) / 2	(8-266)

mvLXSTMVP[ 1 ] = ( mvLXp1Cand[ 1 ] + mvLXCol[ 1 ] ) / 2	(8-267).

- Otherwise, both components of mvLXSTMVP are set equal to 0, refIdxLXSTMVP is set equal to −1 and predFlagLXSTMVP is set equal to 0, with X being 0 or 1:

- The variable availableFlagSTMVP is derived as follows.

- availableFlagSTMVP = availableFlagL0STMVP  | |  availableFlagL1STMVP

- The bi-prediction weight index gbiIdxSTMVP is set equal to 0.