| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0286 |
| --- | --- |

| Title: | CE4-related: Simplified coding of the BPWA index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, Y. Chen, T. Poirier, A. Robert 975 Avenue des Champs Blancs 35576 Cesson-Sévigné FRANCE | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the coding of the BPWA index (aka GBi index). In VTM-4.0, this index is coded as a series of 2 to 4 bins, which are all context-based coded. Here, only the first bin, which indicates the default bi-prediction weights (1/2, 1/2), is context-based coded. The other bins are bypass-coded.

The proposed simplification reportedly brings 0.00% luma BD-rate in RA and 0.00% luma BD-rate in LDB, over VTM-4.0 anchor.

It is suggested to adopt the proposed simplification into next VTM-5.0.

# Introduction

In VTM-4.0, the BPWA index, also called GBi index here, is coded by a flag, followed by a Truncated Rice bin string. The flag indicates if the default GBi weight 4 is used for a given CU, corresponding to usual bi-prediction with equal weights (1/2, 1/2).

If not, then a TR (Truncated Rice) binarized bin string is used to indicate the index of the non-default weight used in the set {5, 3, 10, -2} or {5, 3}, depending on if current picture is of low-delay type or not. In VTM-4.0, each bin of this TR bin string is context-based coded with a dedicated CABAC context.

In this contribution, the binarization of the GBi index is unchanged. Additionally, it is proposed to code each bin of the above-mentioned TR bin string in bypass mode, instead of regular CABAC mode.

Therefore, the number of context-based coded bins is reduced to one bin for the signaling of the BPWA index.

# Proposed modification to VVC draft 4

This section shows the normative modifications brought to the VVC draft 4 JVET-M1001 [1]. Basically, the table 9-15 of JVET-M1001 is modified, as shown hereafter.

Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| … | … | … | … | … | … | … |
| gbi_idx[ ][ ] NoBackwardPredFlag = = 0 | 0 | 1 bypass | na | na | na | na |
| gbi_idx[ ][ ] NoBackwardPredFlag = = 1 | 0 | 1 bypass | 2 bypass | 3 bypass | na | na |
| … | … | … | … | … | … | … |
| … | … | … | … | … | … | … |

# Experimental Results

The performance obtained with the proposed simplification over VTM-4.0 anchor are reported by Table 1 below. As can be seen, no BD-rate loss is obtained with the proposed simplification, both in RA and LDB.

Table 1: RA and LDB results of the proposed simplified BPWA index coding

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.01% | 103% | 102% |
| Class A2 | -0.01% | -0.02% | 0.06% | 102% | 103% |
| Class B | 0.01% | -0.02% | 0.02% | 100% | 99% |
| Class C | 0.00% | -0.05% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.02% | 101% | 101% |
| Class D | 0.01% | -0.04% | -0.06% | 110% | 101% |
| Class F (optional) | 0.00% | -0.02% | -0.02% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.14% | 0.05% | 106% | 89% |
| Class C | -0.03% | 0.23% | 0.05% | 102% | 95% |
| Class E | 0.01% | 0.30% | -0.33% | 102% | 99% |
| Overall | 0.00% | 0.09% | -0.05% | 103% | 93% |
| Class D | 0.00% | 0.11% | -0.31% | 102% | 97% |
| Class F (optional) | -0.03% | 0.07% | 0.14% | 101% | 93% |

# Conclusion

This contribution proposes to simplify the coding of the BPWA index, by coding only the first bin in context-based mode, and by coding other bins in bypass mode. It is reported that no loss in coding efficiency is obtained with the proposed simplification.

It is suggested to adopt the proposed simplification into future VTM-5.0.

# Cited documents

[1] “Versatile Video Coding (Draft 4)”, Benjamin Bross, Jianle Chen, Shan Liu, JVET-M1001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).