| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0287 |
| --- | --- |

| Title: | CE4-related: simplified coding of the MVP index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, Y. Chen, T. Poirier, A. Robert 975 Avenue des Champs Blancs 35576 Cesson-Sévigné FRANCE | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the coding of the motion vector prediction index in AMVP mode. This syntax element, called mvp_l0_flag and mvp_l1_flag in JVET-M1001, is context-based coded in VTM-4.0. This contribution proposes to code it in bypass mode.

The proposed simplification reportedly brings +0.02% Luma BD-rate loss in RA and 0.0x% Luma BD-rate in LDB, over VTM-4.0 anchor.

It is suggested to adopt the proposed simplification into next VTM-5.0.

# Introduction

In VTM-4.0, the syntax elements mvp_l0_flag and mvp_l1_flag are coded in regular CABAC mode, with a same context. Some off-line analysis has shown that the average entropy of this binary symbol is very close to 1 bit of information, indicating that arithmetic coding of this bin is likely not to give to much benefit in terms of coding efficiency.

Therefore, this contribution proposes to code these two syntax elements in bypass mode.

# Experimental Results

The performance of the proposed simplification are shown by the following Table 1. As can be seen, 0.02% BD-rate loss is obtained in RA in Luma, and 0.0x% in LDB.

Table 1: performance obtained with the proposed simplified MVP index coding

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.11% | 0.04% | 100% | 99% |
| Class A2 | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class B | 0.03% | 0.00% | 0.02% | 101% | 100% |
| Class C | 0.02% | 0.01% | -0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | -0.01% | 101% | 100% |
| Class D | 0.00% | -0.06% | -0.08% | 100% | 101% |
| Class F (optional) | 0.09% | 0.04% | -0.07% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.16% | -0.13% | 104% | 101% |
| Class E | 0.06% | -0.14% | 0.33% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.53% | -0.55% | 99% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Proposed modification to VVC draft 4

This section shows the normative modifications brought to the VVC draft 4 JVET-M1001 [1]. Basically, the table 9-15 of JVET-M1001 is modified, as shown hereafter.

Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| … | … | … | … | … | … | … |
| mvp_l0_flag[ ][ ] | 0 bypass | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 bypass | na | na | na | na | na |
| … | … | … | … | … | … | … |
| … | … | … | … | … | … | … |

# Combination with the simplified GBI index of JVET-N0286

Table 2 shows the performances obtained with the combination of the simplification proposed in this contribution, together with the simplified GBi index coding proposed in JVET-N0286. 0.02% loss is reported in RA (Luma), and 0.0x% BD-rate change is obtained in LDB.

Table 2: performance obtained by combining the proposed simplified MVP index coding with the simplified GBi Index coding of JVET-N0286

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | 0.01% | 102% | 101% |
| Class A2 | 0.00% | -0.03% | 0.08% | 103% | 101% |
| Class B | 0.02% | 0.01% | 0.00% | 101% | 101% |
| Class C | 0.04% | 0.03% | -0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | -0.02% | 101% | 101% |
| Class D | 0.01% | -0.03% | -0.06% | 100% | 101% |
| Class F (optional) | 0.09% | 0.05% | -0.10% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | 0.19% | 0.02% | 102% | 99% |
| Class E | 0.11% | 0.29% | 0.03% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | -0.01% | -0.32% | 103% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes to simplify the coding of the Motion Vector predictor index in AMVP mode, by coding the concerned bin in bypass mode.

It is reported that 0.02% BD-rate loss in Luma is obtained with the proposed simplification and no loss is obtained in LDB.

When combined with the simplified BPWA or GBI index as proposed in JVET-Nxxxx, the overall simplification also give 0.02% BD-rate loss in Luma in RA, and 0.0x% BD-rate change in LDB.

It is suggested to adopt the proposed simplification into future VTM-5.0.

# Cited documents

[1] “Versatile Video Coding (Draft 4)”, Benjamin Bross, Jianle Chen, Shan Liu, JVET-M1001, oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).