| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0289-V3 |
| --- | --- |

| Title: | CE8-Related : On MVD Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Seethal Paluri Seung-Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution proposes unified MVD coding to address the different characteristics of IBC MVD (BVD, Block Vector Difference). Method 1 is a MVD coding based on VTM MVD binarization with modified context model. Method 2 proposes more context coded bins and modified context modeling. Specifically, abs_mvd_prefix is introduced, which contains gt0, gt1, gt3, gt7, and gt15 and is context coded. Method 1 reportedly provides 0.24%, 0.26 % and 0.06 % luma BD-rates improvement for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions. Method 2 reportedly provides 1.03%, 0.88% and 0.71% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions.

# Introduction

In HEVC and VTM4.0, MVD is coded by MVDx and MVDy component independently. Each MVD component is coded by greather_than_zero (GT0), greather_than_one (GT1), and the remaining absolute value is coded by Exponential Golomb code with order 1 (EG1).

# Problem Statement

Since the adoption of intra block copy (IBC), the existing MVD coding is also used to code MVD and BVD. However, the statistics of IBC MVD is very different from that of non-IBC MVD. In addition, the statistics between MVDx and MVDy is also different each other in IBC case as shown in Fig. 1.

Here is the stats for absolute value of MVD of IBC case:

Also Fig 1 is the stats for the bin value up to 50:

Fig 1: Absolute value of MVD x and y distribution for IBC case

In summary in IBC case the data is less skewed , is asymetric between x and y, also it is bounded to a certain value, which is smaller than normal MVD.

# Proposed Method

In this contribution we propose two unfied MVD coding methods as follows:

- Method 1: MVD coding based on current VTM MVD binarization with modified context model.

- Method 2: More context coded bins and modified context modeling

Method 1: MVD coding used for both IBC and non-IBC MVD coding without any additional context coded bin

Considering different characteristics between MVDx and MVDy , in this method for all the MVD coding cases, flag for greater than zero for component x and component y use different context models, i.e., abs_mvd_greater0_flag[ 0 ] and abs_mvd_greater0_flag[ 1 ] use different context model. Similarly flag for greater than one for component x and component y use different context models, i.e., abs_mvd_greater1_flag[ 0 ] and abs_mvd_greater1_flag[ 1 ] use different context models as explanined in the table below.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_greater0_flag[ compIdx ] | compIdx==0 ? 0 : 1 | na | na | na | na | na |
| abs_mvd_greater1_flag[compIdx ] | compIdx==0 ? 0 : 1 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |

Method 2: MVD coding with abs_mvd_prefix (gt0, gt1, gt3, gt7, and gt15)

In this method considering different charachtristics between MVDx and MVDy, as well as existence of bigger values for MVD (also less skewed data) in IBC case, we propose to have gt_x flags up to 15.

In particular in test b first a flag abs_mvd_greater0_flag[] sent to determine if absolute value of MVDi is greater than 0 or not.

If the flag abs_mvd_greater0_flag[] is 1, then another flag abs_mvd_greater1_flag[]  is coded to determine if the absolute value of MVDi is greater than 1 or not.

If the flag abs_mvd_greater1_flag[] is 1, then another flag abs_mvd_greater3_flag[] is coded to determine if the absolute value of MVDi is greater than 3 or not.

If the flag abs_mvd_greater3_flag[] is 0, then a flag coded to determine if the absolute value of MVDi is 2 or 3. If the flag abs_mvd_greater3_flag[] is 1, then another flag abs_mvd_greater7_flag[]  is coded to determine if the absolute value of MVDi is greater than 7 or not.

If the flag abs_mvd_greater7_flag[] is 0, then a FLC used to determine if the absolute value of MVDi is 4 to 7. If the flag abs_mvd_greater7_flag[] is 1, then another flag abs_mvd_greater15_flag[]  is coded to determine if the absolute value of MVDi is greater than 15 or not.

If the flag abs_mvd_greater15_flag[] is 0, then a FLC used to determine if the absolute value of MVDi is 8 to 15. If the flag abs_mvd_greater15_flag[] is 1, then remaining level of absolute value of MVDi  (absolute value of MVDi  minus 16) is coded using Exponential Golomb of order 4.

Finally for all the cases with absolute value of MVDi greater than 0, a sign flag mvd_sign_flag[]  is coded.

This method is actually similar to the current MVD coding in VTM4, however the EG prefixes till value 15 are coded with context coded bin instead of the bypass bin.

So for the simplification we combine all the abs_mvd_greaterX_flag[] together as abs_mvd_prefix[] and binarized using Truncated Unary and context coded.

| abs_mvd_prefix |  | gt0 | gt1 | gt3 | gt7 | gt15 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 |  |  |  |  |
| 1 |  | 1 | 0 |  |  |  |
| 2 |  | 1 | 1 | 0 |  |  |
| 3 |  | 1 | 1 | 1 | 0 |  |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 5 |  | 1 | 1 | 1 | 1 | 1 |

For the case of abs_mvd_prefix[] bigger than 1 and smaller than 5 (abs mvd above 1, below 16), FLC code abs_mvd_suffix_below16[] is used to code the rest. For the case of abs_mvd_prefix[] equals 5 (abs mvd greater than or equal 16) Exponential Golomb code of order 4 is used to code the remaining.

| Abs Value Level | abs_mvd_prefix | FLC |
| --- | --- | --- |
| 0 | 0 | N/A |
| 1 | 1 | N/A |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 3 | 00 |
| 5 | 3 | 01 |
| 6 | 3 | 10 |
| 7 | 3 | 11 |
| 8 | 4 | 000 |
| 9 | 4 | 001 |
| 10 | 4 | 010 |
| 11 | 4 | 011 |
| 12 | 4 | 100 |
| 13 | 4 | 101 |
| 14 | 4 | 110 |
| 15 | 4 | 111 |
| > 15 | 5 | No FLC/ Use EG4 for the remaining |

Same as test a, different context model could be used for component x and y. For simplicity and reducing number of the context model only for abs_mvd_greater0_flag[] x and y use different context model.

# Experimental results

The proposed method has been implemented based on VTM 4.0 reference software according to common test conditions defined in JVET-M1010 [1] and Core Experiment description in JVET-M1028 [2].

Results for MVD.a (CTC Condition):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -0.10% | -0.05% | -0.18% | 100% | 100% |
| Class SCC 1080p | -0.24% | -0.27% | -0.25% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | -0.06% | 100% | 100% |
| Class A2 | -0.02% | 0.09% | -0.01% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.02% | 100% | 100% |
| Class C | 0.01% | -0.08% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class D | 0.02% | 0.11% | -0.10% | 100% | 101% |
| Class F (optional) | -0.11% | -0.16% | -0.10% | 100% | 101% |
| Class SCC 1080p | -0.26% | -0.19% | -0.20% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.03% | 0.24% | -0.04% | 100% | 101% |
| Class E | -0.03% | -0.02% | -0.59% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | -0.07% | -0.39% | 100% | 100% |
| Class F (optional) | -0.19% | -0.31% | -0.12% | #NUM! | #NUM! |
| Class SCC 1080p | -0.06% | -0.18% | -0.12% | 100% | 100% |

Results for MVD.b (CTC Condition):

Coding time not reliable (different servers..)

# Conclusion

In the contribution modified MVD coding proposed, which improves the BD rete performance. It is recommended that the proposed MVD be adopted in the next VTM.

# Spec Text

Below is the spec text for test b:

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_prefix[ 0 ] | ae(v) |
| if((abs_mvd_prefix[ 0 ]>1) && (abs_mvd_prefix[ 0 ]<5)) |  |
| abs_mvd_suffix_below16 [ 0 ] | ae(v) |
| else if(abs_mvd_prefix[ 0 ]==5) |  |
| abs_mvd_minus16[ 0 ] | ae(v) |
| if(abs_mvd_prefix[ 0 ]>0) |  |
| mvd_sign_flag[ 0 ] | ae(v) |
| abs_mvd_prefix[ 1 ] | ae(v) |
| if((abs_mvd_prefix[ 1 ]>1) && (abs_mvd_prefix[ 1 ]<5)) |  |
| abs_mvd_suffix_below16 [ 1 ] | ae(v) |
| else if(abs_mvd_prefix[ 1 ]==5) |  |
| abs_mvd_minus16[ 1 ] | ae(v) |
| if(abs_mvd_prefix[ 1 ]>0) |  |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |

#### Motion vector difference semantics

abs_mvd_prefix [ compIdx ] specifies the prefix of absolute value of a motion vector component difference, where compIdx represents direction x or y.

abs_mvd_suffix_below16 [ compIdx ] specifies the suffix of mvd below 16.

When abs_mvd_suffix_below16 [ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus16[ compIdx ] plus 16 specifies the absolute value of a motion vector component difference.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_prefix [ ] | TR | cMax = 5 |
|  | abs_mvd_suffix_below16 [ ] | FL | cMax = 1 << ( abs_mvd_prefix [ ]-1) - 1 |
|  | abs_mvd_minus16[ ] | EG4 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_prefix [ compIdx ] | compIdx==0 ? 0 : 1 | 0 | 0 | 0 | 0 | na |
| abs_mvd_suffix_below16[compIdx] | bypass | bypass | bypass | na | na | na |
| abs_mvd_minus16[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |

# Reference

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

- X. Xu et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-M1028, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).