| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0290 |
| --- | --- |

| Title: | AHG17: Conditional Signaling of ALF and In-Loop Reshaper Model. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email | 1-858-635-5300 seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |  |
| Source: | LG Electronics USA |  |  |

Abstract

# This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters, while still being present conditionally in the tile group header. This is advantageous because, it can provide the flexibility of partial updates of the ALF parameters and the in-loop reshaper model either through the APS or through the tile group header itself. A new flag proposed in the tile group header is used to control if the tile group should refer to an APS or would carry its own ALF and in-loop reshaper model.

- Introduction

In the current design of Versatile Video Coding(VVC), the Adaptation Parameter Set (APS) has been adopted for the carriage of the Adaptive Loop Filter (ALF) parameters. This serves to reduce the redundant and unnecessary transmissions of the ALF parameters. The use of the ALF tool is enabled in the Sequence Parameter Set (SPS) Similar to the ALF, the in-loop reshaper tool is also enabled in the SPS. However, the use of the in-loop reshaper is controlled in the tile group header by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag. If the tile_group_reshaper_model_present_flag is enabled in the tile group header, then it indicates that a reshaping model for that tile group is present. The tile_group_reshaper_enable_flag is further parsed to decipher if the reshaper model is used in the tile group header or not. As these two flags are signaled independently of one another.

- Proposal

We propose to provide a granularity in the flexibility offered when the ALF and in-loop reshaper model parameters can be present either in the APS or be conditionally present in the tile group header. A flag i.e., tile_group_alf_usage_flag is used to indicate if the ALF data is to parsed from an APS or from within the tile group header. Similarily, the flag tile_group_reshaper_usage_flag is used to indicate if the reshaper model is to be parsed from an APS or from within a tile group header.

This may be useful for the support of real-time ultra low latency applications with sub-picture-interval delay. An example would be the remote controlling of a drone. Alternatively, another example would be wireless screen sharing. Such examples may require that each picture be encoded into multiple VCL NAL units, where each VCL is transmitted immediately after its encoded has been completed. Similar to the tranmission of the VCL units after encoding, the decoder also expects to decode the VCL units upon receiving them. Therefore, the added flexibility of having the ALF data and the in-loop reshaper present conditionally within the tile group header may be beneficial.

Table 1 shows the ALF and Reshaper in the APS.

| adaptation_parameter_set_rbsp(){ | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | ue(v) |
| alf_data() |  |
| reshaper_model( ) |  |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The Table 1 above shows the in-loop reshaper parameters being parsed in the APS. Here, the syntax reshaper_model() represents the syntax elements present within the tile_group_reshaper_model().

Table 2 shows the Tile Group Header Syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| … |  |
| if (sps_alf_enabled_flag) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if(tile_group_alf_enabled_flag){ |  |
| tile_group_alf_usage_flag | u(1) |
| if(tile_group_alf_usage_flag){ |  |
| alf_data() |  |
| } else { |  |
| tile_group_aps_id_alf | u(5) |
| } |  |
| } |  |
| } |  |
| if (sps_reshaper_enabled_flag) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if(tile_group_reshaper_model_present_flag){ |  |
| tile_group_resahper_usage_flag | u(1) |
| if(tile_group_reshaper_usage_flag){ |  |
| tile_group_reshaper_model() |  |
| } else { |  |
| tile_group_aps_id_reshaper | u(5) |
| } |  |
| } |  |
| tile_group_reshaper_enable_flag | u(1) |
| if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

tile_group_alf_usage_flag equal to 0 specifies that the APS id is specified by tile_group_aps_id_alf. tile_group_alf_usage_flag equal to 1 specifies that the APS id is not used and the alf_data() is parsed in the current tile group header.

tile_group_reshaper_usage_flag equal to 0 specifies that the APS id is specified by tile_group_aps_id_reshaper. tile_group_alf_usage_flag equal to 1 specifies that the APS id is not used and the tile_group_reshaper_model() is parsed in the current tile group header.

tile_group_aps_id_alf specifies the adaptation_parameter_set_id of the APS that the tile group refers to access the ALF data. The TemporalId of the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id_alf shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple APSs with the same value of adaptation_parameter_set_id shall have the same content.

tile_group_aps_id_reshaper specifies the adaptation_parameter_set_id of the APS that the tile group refers to access the reshaper data. The TemporalId of the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id_reshaper shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple APSs with the same value of adaptation_parameter_set_id shall have the same content.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).