| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0291-v1 |
| --- | --- |

| Title: | CE9: Results of DMVR related Tests CE9-1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

The bit-depth used for performing the SAD computations required for DMVR are maintained at a fixed intermediate bit-depth of 10-bits independent of the input bit-depth in the proposed draft text. In JVET-L0174, a reduced fixed bit-depth of 8-bits had been proposed to further reduce the storage needs and to reduce the SAD computation complexity. In this core experiment, the results in VTM4.0 for restricting the bilinear interpolated samples to 8-bits are presented. The results show that there is a negligible impact on BD-RATE (in fact, a small gain of 0.01%). Results are also shown for one variant wherein the intermediate bit-depth after horizontal interpolation in a 2-D separable interpolation case is also restricted to 8-bit. The encoding and decoding times can be reduced by leveraging 8-bit SIMD instruction support for SAD on most architectures. For hardware implementation, the reduced bit-depth reduces the complexity of the SAD execution unit. Hence, it is requested that the proposed bit depth restriction be adopted.

# Introduction

The bilinear interpolation output in the currently adopted DMVR is at a fixed bit-depth of 10-bits. DMVR requires 5 line buffers per reference each containing (sbWidth + 4) samples, where sbWidth is the sub-block width of a sub-block within the current coding unit. If the bit-depth is reduced to 8-bits, the line buffer storage needs can be reduced by 20%. Also, 8-bit SAD computation is supported well in many SIMD instruction sets on popular architectures. In hardware, 8-bit SAD computation reduces the overall gate count requirement for DMVR.

In JVET-L0174, results for 8-bits output were first presented. Core experiment 9-1.2 presents the same results in the context of the VTM4.0 codebase.

# Proposed method

The proposed method computes the 10-bit fixed bit-depth output of bilinear interpolation as in the current specification text and then further shifts it right by 2-bits without rounding to obtain a fixed 8-bit depth value. The SAD cost computations for each search point in the search space are computed using these 8-bit predicted sample values.

The changes to the specification draft are shown below.

##### 8.5.3.2.2 Luma sample bilinear interpolation process

The variables shift1, shift2, shift3, and offset1 are derived as follows:

shift1 = BitDepthY − 4		(8-452)

offset1 = 1 << ( shift1 − 3 )		(8-453)

shift2 = 6					(8-454)

-

-

-

shift3 = BitDepthY − 8		(8-456)

- If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt0 ][ yInt0 ]  >> shift3 	(8-461)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =   i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt0 ]   >>  shift1	(8-462)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=01fL[ yFracL ][ i ] * refPicLXL[ xInt0 ][ yInti ]    >>  shift1	(8-463)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..1, is derived as follows:

temp[ n ] = (  i=01fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]  + offset1 )  >>  (shift1-2)	(8-464)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =   i=01fL[ yFracL ][ i ] * temp[ i ]   >>  shift2	(8-465)

# Results

The following table Table 3.1 shows the CTC results against VTM-4.0 when only the final 10-bit values are shifted right by another 2-bits without any rounding. It can be seen that the bit-depth restriction to 8-bits has no noticeable impact on the BD-RATE. The encoding and decoding times can be reduced further by writing 8-bit SIMD intrinsics which is not done in the CE submission.

Table 3.1: Results with only final 10-bit output restricted to 8-bits

The following Table 3.2 summarizes the results when the intermediate output of horizontal interpolation in separable 2-D bilinear interpolation is also restricted to 8-bit depth. The suggested specification draft text change corresponds to this. Again, the results indicate no impact on BD-RATE with the bit-depth restriction.

Table 3.2: Results with intermediate horizontal interpolation output in 2-D interpolation also restricted to 8-bits (Supplemental data)

# Conclusion

CE9-1.2 proposes to restrict the bit-depth used for DMVR SAD computations to 8-bits. This has the advantage of reducing the line buffer storage requirements for DMVR from the current 10-bit samples to 8-bit samples. Also, 8-bit SAD computations are well supported in the SIMD instruction set of many architectures. It also reduced the gate-count for DMVR in hardware implementations. The results indicate that there is no BD-RATE impact with the proposed bit-depth restriction. Hence, it is requested that the proposed change be adopted into the next VTM.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).