| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0292-v2 |
| --- | --- |

| Title: | CE9-related: Reference sample access constraints at VPDU level for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

DMVR, as adopted, currently constrains reference sample pre-fetch at the CU level to access not more than the worst-case bi-prediction reference sample access dimensions for that CU size. Virtual pipeline data unit (VPDU) is defined in VVC as the unit at which the decoder’s processing pipeline is expected to work. However, for CU sizes larger than VPDU size (such as, 128x128, 128x64, and 64x128 CU sizes), one CU spans more than one VPDU. In this proposal, reference sample pre-fetch is constrained by treating each VPDU like a coding unit. Hence, each VPDU is constrained to access not more than the worst-case bi-prediction reference sample dimensions for the VPDU size. This eliminates the need to access additional reference samples to account for the amount of displacement due to refinement in the horizontal and vertical directions. Results show that enforcing this constraint has negligible BD-RATE impact and negligible change in EncT or DecT ratios. It is requested that such a VPDU-level reference sample pre-fetch constraint be adopted to ensure that external memory accesses at a VPDU-level do not depend on the refinement range of DMVR.

# Introduction

Decoder motion vector refinement was adopted into VVC at the 13th JVET meeting. Reference sample access in each reference picture was constrained to be based on the unrefined motion vector to not exceed (CU_width + 7) x (CU_height + 7) luma reference samples and to not exceed (CU_width/2 + 3) x (CU_height/2 + 3) chroma reference samples per component (for 4:2:0 chroma format). Whenever a sample value outside this set of sample positions is required for motion compensated interpolation using the refined motion vector obtained through DMVR, the value of the closest sample positions within these constrained block of samples are used.

Virtual pipeline data unit (VPDU) is a block of luma samples and corresponding chroma samples within a coding tree unit (CTU) on which each pipeline stage of a multi-stage pixel-processing decoding pipeline performs the operations for that stage. A VPDU can contain multiple coding units within and it is also possible for one coding unit to span multiple VPDUs. The currently adopted VPDU in VVC allows a 64x64 luma block of samples and the corresponding blocks of chroma samples to be processed in a stage of a pipeline independent of any dependencies on future VPDUs in the order of scan.

As per the current restrictions for VPDU, coding unit sizes of 128x128 luma samples (and corresponding chroma samples) comprising 4 VPDUs, and coding unit sizes of 128x64 or 64x128 luma samples (and corresponding chroma samples) comprising 2 VPDUs are allowed. In a VPDU in such CUs, only one horizontal boundary (left or right) and one vertical boundary (top or bottom) correspond to a coding unit boundary and on these boundaries the reference sample access is constrained. On the other two boundaries, reference sample access depends on the refinement range.

In this proposal, it is proposed to constrain a VPDU of size 64x64 to not access more than (64 + 7) x (64 + 7) luma samples and (32 + 3) x (32 + 3) chroma sample per component in each reference.

# Proposed method

In the proposed method, when a CU contains more than 1 VPDU, each VPDU is treated like a CU when it comes to reference sample access for motion compensation. Hence, instead of accessing (64 + 9) x (64 + 9) luma reference samples in each reference, each VPDU in such CUs access only (64 + 7) x (64 + 7) luma reference samples in each reference with respect to the unrefined motion vector. Similarly, the (4:2:0) chroma reference sample accessed is constrained from (32 + 5) x (32 + 5) samples per reference in each component to access (32 + 3) x (32 + 3) reference samples per reference in each component.

# Results

Following are the results over VTM4.0 when the proposed constraints were applied to 128x128, 128x64, and 64x128 CU sizes.

It can be seen that the constraint had a negligible impact on the BD-RATE as well as on the encoding or decoding times.

# Conclusion

In CUs that span multiple VPDUs, the proposed method constrains the reference sample access for each VPDU as though each VPDU were a coding unit. This ensures number of reference samples that need to be accessed at the VPDU level is independent of the DMVR refinement range. The results indicate that there is negligible impact on BD-RATE and encoding or decoding times with the proposed change. It is requested that the proposed method be adopted in the next version of VTM.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).