| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0293 |
| --- | --- |

| Title: | AHG17: Reshaper Reset Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email | 1-858-635-5300 seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

Abstract

This contribution proposes a design for the reset of the In-Loop reshaper model at Intra Random Access Point (IRAP) picture. The In-Loop reshaper model is conditionally present in the tile group header. In the existing design, the reshaper parameters are controlled by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag, which indicate whether the reshaper model is present in the current tile group and whether it is to be used in the current tile group. These two flags work independently of one another. During an intra refresh, the model may not be present, however the model use may be enabled for the current and subsequent tile groups. This may not be a desirable option at a random access point and may cause dependency across random access point. Therefore, it is proposed to reset the reshaper to a default reshaper at IRAP picture if reshaper is not present. The default reshaper can be linear mapping or it can be signaled in the Sequence Parameter Set.

# Background

The In-Loop reshaper has been adopted into Versatile Video Coding (VVC) standard at the 13th meeting in Marrakesh. The in-loop reshaper is used to map an input luma to an output coded luma value. The use of the in-loop reshaper in the tile group header is controlled by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag. If the tile_group_reshaper_model_present_flag is enabled in the tile group header, then it indicates that a reshaping model for the current tile group is present. The tile_group_reshaper_enable_flag is further parsed to decipher if the reshaper model is to be used in the tile group header or not. As these two flags are signaled independently of one another. In current VTM, if reshaper model is not present, while reshaper model is enabled, it will keep using previous reshaper. It can be inferred from the syntax the conditions for the usage of the reshaper can occur as shown below in Table 1.

Table 1 shows the combinations of the reshaper use for an IRAP picture.

| Model Present | Model Enabled | Syntax Permits | Feasible for IRAP picture |
| --- | --- | --- | --- |
| Yes | Yes | Yes | Yes |
| Yes | No | Yes | Yes |
| No | Yes | Yes | No |
| No | No | Yes | Yes |

In Table 1, it can be seen that of the four combinations, one combination in particular i.e., model not present tile group header but needs to be used in the tile group is permitted by the syntax but should not be feasible in the case of an IRAP picture. In such a case, the reshaper should not have any dependency on any previous reshaper. As the existing structure does not restrict the usage of a reshaper when the model is not present for the intra coded tile group, it may be beneficial to define a default reshaper in the Sequence Parameter Set (SPS) in order to restrict the usage of the reshaper at IRAP pciture.

# Proposal

We propose to resolve this issue by resetting to a default reshaper at IRAP pciture before receiving tile group reshaper infomation. The default reshaper may be a linear mapping or it may be signaled in SPS as shown in Table 2 below.

Table 2. Shows an excerpt from the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_reshaper_enabled_flag | u(1) |
| if(sps_reshaper_enabled_flag) |  |
| reshaper_model ( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Here the reshaper_model() shown above in Table 2 contains the parameters for the in-loop reshaper model.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements. The value of sps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).