| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0295-v1 |
| --- | --- |

| Title: | CE9-related: Using extended samples during DMVR SAD cost evaluations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

DMVR, as adopted at the last meeting, performs refinement on each sub-block within a coding unit independent of the other sub-blocks in that coding unit. Given that the sub-block boundaries are currently not deblocked based on the refined MVs across the sub-block edge, independent movement of each sub-block can result in certain boundary discontinuities. In this proposal, use of extended reference samples beyond the dimensions of the current sub-block is proposed for DMVR cost evaluations in order to improve the coding efficiency of DMVR. Specifically, extensions of 1 or 2 samples on all sides is proposed. The worst-case computational complexity increase due to the proposed changes are analyzed and the extension is accordingly suppressed for two sub-block sizes. The results under CTC indicate that using extended samples can offer a BDRATE gain of ~0.1% with an average decoding time increase of ~2%. It is requested that use of extended samples be considered for further study in core experiments.

# Introduction

In the adopted version of DMVR, refinement is performed at a sub-block level within a coding unit in order to reduce the on-chip memory needs for DMVR. However, independent refinements for sub-blocks can result in some sub-block boundary discontinuities that can result in some compression loss or can result in some visual artifacts when residuals are not coded. De-blocking across sub-block edges based on boundary strength computed using the refined MVs of the sub-blocks was not adopted at the last meeting due to hardware design constraints.

In this proposal, in order to reduce the sub-block boundary level discontinuities and improve the coding efficiency, use of extended samples is proposed while computing the matching costs during the refinement process.

Complexity increase due to the need for producing more bilinear interpolated sample for refinement and due to the larger block sizes used for the matching cost computations are analyzed. Based on the analysis, some recommendations for when to use the extended samples is made. Results are presented under CTC to demonstrate the improved coding gains and the complexity increase seen with the proposed method.

# Proposed Method

In the proposed method, for a sub-block of dimensions sbW x sbH, one-sided refinement range of S pixels in both the horizontal and vertical directions, predicted samples of dimension (sbW + 2*S + 2*Ex) x (sbH + 2*S + 2*Ey) in each reference are obtained using the initial unrefined MV pair, where Ex and Ey correspond to the one-sided extended samples to be used in the horizontal and vertical directions while computing the SAD based costs at each search position in the refinement range. When the unrefined MV is a fractional sample position, bilinear interpolation is employed to obtain the predicted samples.

For a given delta MV DMV in the refinement range, extended blocks of dimension (sbW + 2*Ex) x (sbH + 2*Ey) in each predicted sample block are extracted. For example, for a DMV of (0,0), blocks of the stated dimension are extracted starting from the position (Sx, Sy) in the two predicted sample blocks. For a DMV of (1,1), blocks of the stated dimension are extracted from (Sx + 1, Sy + 1) in the first reference L0 and from (Sx – 1, Sy – 1) in the second reference L1. SAD costs are computed between the two extracted extended blocks at each refinement point within the refinement range. The order of cost comparisons to select the final delta MV is the same process as used in the reference DMVR method that does not use extended samples.

sbW

Sx

Ex

Sx

Ex

sbH

Sy

Ey

Sy

Ey

interpWidth = (sbW + 2*Sx + 2*Ex)

Figure 1: Predicted samples to be produced with desired extension (shown for Ex = Ey = 1)

costWidth = (sbW + 2*Ex)

costHeight = (sbH + 2*Ey)

Figure 2: Extended samples used for SAD cost computation

Figure 1 illustrates the predicted samples to be produced with the desired extension and Figure 2 illustrates the extended block used for matching cost evaluation. Though Fig. 2 does not indicate the vertical decimation by a factor of 2 while evaluating SAD costs, the results reported in this contribution employ SAD evaluation with a vertical decimation by a factor of 2. When extension amount is by 1 sample, due to this vertical decimation, extension row in either the top or bottom will not be used in the cost calculation.

# Complexity analysis

The use of extended samples increases the size of the line buffers or the prediction buffers used during refinement. The line buffers increase in width by 2*Ex samples. The prediction buffer increases in height by 2*Ey samples.

Though the software implementations can perform the bilinear interpolation at a coding unit level rather than at a sub-block level, hardware implementations may not be able to re-use prediction samples computed for one sub-block for another neighbor sub-block. The increase in bilinear interpolation operations at a sub-block level is captured in the attached spreadsheet across the different sub-block sizes used by DMVR and summarized in the table below as the % increase in operation count.

| Extension related details | Worst-case bilinear interpolation operation count % increase | Worst-case SAD operation count % increase |
| --- | --- | --- |
| Extension enabled for all sub-block sizes (Ex = Ey = 1) | 80 | 69 |
| Extension disabled for 4x16 and 8x8 sub-block sizes (Ex = Ey = 1) | 21 | 41 |
| Extension enabled for all sub-block sizes (Ex = Ey = 2) | 126 | 150 |
| Extension disabled for 4x16 and 8x8 sub-block sizes (Ex = Ey = 2) | 47 | 88 |

Table 3.1 Summary of operation count increase related to extended sample based matching

# Results

Table 4.1 shows the results with Ex = Ey = 2 against VTM4.0 under CTC. It can be seen that the use of extended samples results in a BDRATE gain of 0.11% with an average decoding time increase of 2%.

Table 4.1 Results with Ex = Ey = 2

Table 4.2 shows the results with Ex = Ey = 1 against VTM4.0 under CTC. The BDRATE gain in this case is 0.07% with a decoding time increase of ~2%.

Table 4.2 Results with Ex = Ey = 1

The results with the extension disabled for 4x16 and 8x8 sub-block sizes are underway and shall be updated in subsequent versions of this document.

# Conclusions

In this proposal, a method for using extended samples beyond a sub-block size of samples for matching cost evaluation in DMVR is proposed. The extension offers a BDRATE gain of 0.11%. Both the bilinear interpolation complexity and SAD computation complexity increase with the proposed method. In order to cap the worst-case complexity, it is suggested that the extended samples based matching cost be disabled for 4x16 and 8x8 sub-block sizes. It is requested that this method be taken up for further study in a core experiment.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).