| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0296-v2 |
| --- | --- |

| Title: | CE9-related: DMVR costs based early termination for BDOF in CE9-2.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

In CE9-2.6, BDOF early termination at both CU and 4x4 level are disabled. In addition, BDOF performs an implicit split of a coding unit into sub-blocks not exceeding a width or height of 16 luma samples in order to reduce the internal memory needs related to BDOF and to harmonize the pipeline for processing across DMVR and BDOF. However, disabling of the early termination increases the average decoding time by ~4%. In this proposal, it is proposed to employ an early termination check for BDOF in CUs that apply both DMVR and BDOF by thresholding the matching cost from DMVR corresponding to the refined motion vector against a pre-determined threshold. The results indicate that there is a negligible (< 0.01%) BD-RATE impact associated with the re-introduction of early termination when compared to CE9-2.6. The decoding time shows a 3% reduction relative to CE9-2.6. It is possible to gate the BDOF related computations in hardware implementations to achieve average power savings with the proposed method. It is requested that the proposed DMVR matching cost based early termination for BDOF be considered for further study in core experiments.

# Introduction

Current BDOF performs a CU-level SAD-based early termination check and a 4x4 block-level SAD-based early termination check. Many contributions at the last meeting have highlighted the problems with the early exit check spanning across VPDUs. Given that DMVR is adopted with forced partitioning of coding units into sub-blocks, CE9-2.4 proposes to force partition a coding unit on similar lines into sub-blocks. In addition, CE9-2.6 combines this aspect with disabling of early termination checks for BDOF by combining CE9-2.4 with CE9-2.1. However, disabling early termination checks increases the average decoding time.

In this proposal, it is proposed to re-use the matching cost computed for DMVR to perform early termination check for BDOF whenever both DMVR and BDOF are applicable for a current coding unit. The results are presented on top of CE9-2.6. Thus, the advantage of not requiring any additional SAD computation unit in BDOF is retained, while offering an average decoding time reduction in software implementation and average power savings in hardware implementations.

# Proposed Method

In DMVR, SAD based matching cost is evaluated at different integer distance search points within a refinement range from the initial motion vector pair obtained from the merge index signaled in the bitstream. If the lowest matching cost does not fall on the boundary of the refinement range, the following parametric error surface based equation is solved to compute a sub-pixel refinement motion vector.

E(x,y) = A*(x – x0) 2 + B(y – y0)2 + C
In this case, given the evaluated matching costs E(x,y) at (x,y) positions of (0,0), (-1,0), (0,-1), (1,0), and (0,1) and the parametric equation for E(x,y), the 5 unknowns A, B, C, x0, y0 can be solved as follows:

A = (E(-1,0) + E(1,0) – 2*E(0,0))/2

B = (E(0,-1) + E(0,1) – 2*E(0,0))/2

x0 = (E(-1,0) – E(1,0))/(2*(E(-1,0) + E(1,0) – 2*E(0,0)))

y0 = (E(0,-1) – E(0,1))/(2*(E(0,-1) + E(0,1) – 2*E(0,0)))

C = DerivedCost=E0,0- x0*E-1,0-E1,04- y0*(E0,-1-E0,1)4

In the proposed method, the derived cost C is used as the estimate for the matching cost at the final refined motion vector and compared against a pre-determined threshold, and BDOF process is skipped for the sub-block if the cost is less than the threshold. In the results presented, the threshold used is the same threshold value that is currently used in the adopted BDOF. Since the equation only involves a division by 4, no real division is required for computing the derived cost.

In the event that the position with lowest matching cost falls on the boundary of the refinement range, the parametric error surface fit is not applied and the lowest evaluated matching cost itself is compared against a pre-determined threshold, and BDOF process is skipped for the sub-block if the cost is less than the threshold.

The early termination check at both CU and 4x4 levels are disabled for CUs for which only BDOF is applied and DMVR is not applied in the proposed method.

# Results

The results of the proposed DMVR matching cost based early termination for BDOF was implemented over CE9-2.6 and the results are also presented under CTC against the results of CE9-2.6 in Table 3.1 below. It can be seen that the proposed method has no BD-RATE impact and reduces the average decoding time by 4% when compared to CE9-2.6.

Table 3.1 Results of the proposed early termination check against CE9-2.6

In general, the % of samples that apply both DMVR and BDOF is considerably higher than the % of sample that apply only BDOF. By eliminating the CU-level and 4x4 level early exit check and replacing with a sub-block level early termination check only for CUs that apply both DMVR and BDOF, the average decoding time reduction achieved is very close to the decoding time reduction due to both early termination checks in the adopted BDOF.

# Conclusions

The proposed DMVR matching costs based early termination for BDOF at a sub-block level eliminates the need for any dedicated SAD computation unit within BDOF. It facilitates gating BDOF related operations in hardware implementations and offers a significant reduction in average decoding time with negligible BDRATE impact. It is requested that the proposed method be considered along with the method of CE9-2.6 for further study in core experiments.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).