| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0297 |
| --- | --- |

| Title: | AHG17: Reshaper Model Signaling for Intra Coded Tile Group |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email | 1-858-635-5300 seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

Abstract

This contribution proposes to signal the use of an In-loop reshaper model for an intra coded tile group. In particular it imposes the condition that a reshaper model be present in the intra coded tile group header and allows the reshaper model to be optionally present in other tile group headers.

# Background

The In-Loop reshaper has been adopted into Versatile Video Coding (VVC) standard at the 13th meeting in Marrakesh. It is used to map an input luma to an output coded luma value. The use of the in-loop reshaper in the tile group header is controlled by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag. If the tile_group_reshaper_model_present_flag is enabled in the tile group header, then it indicates that a reshaping model for the current tile group is present. The tile_group_reshaper_enable_flag is further parsed to decipher if the reshaper model is to be used in the tile group header or not.

- Proposal

We propose to add flexibility to the in-loop reshaper design in the tile group header by making an allowance for the model to always be present when the tile group is intra coded. This ensures a refresh mechanism for the model for the coded video sequence.  The syntax as signaled in the tile group header is shown in Table 1 below. A use case to motivate the proposed design is to allow its use in ultra low delay applications which may want to display each VCL NAL unit as soon as it is decoded, with minimal parsing dependency.

Table 1 Shows an excerpt from the tile group header.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| … |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if ( sps_reshaper_enabled_flag ) { |  |
| if ( tile_group_type != I ) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if (tile_group_reshaper_model_present_flag) |  |
| tile_group_reshaper_model ( ) |  |
| tile_group_reshaper_enable_flag | u(1) |
| if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_type specifies the coding type of the tile group according to Table 2.

Table 2 – Name association to tile_group_type

| tile_group_type | Name of tile_group_type |
| --- | --- |
| 0 | B (B tile group) |
| 1 | P (P tile group) |
| 2 | I (I tile group) |

When nal_unit_type is equal to IRAP_NUT, i.e., the picture is an IRAP picture, tile_group_type shall be equal to 2.

tile_group_reshaper_model_present_flag equal to 1 specifies tile_group_reshaper_model() is present in tile group header.

tile_group_reshaper_model_present_flag equal to 0 specifies tile_group_reshaper_model() is not present in tile group header.

When tile_group_reshaper_model_present_flag is not present, it is specified as follows.

-        When tile_group_type is equal to 2(I tile group), tile_group_reshaper_model_present_flag is inferred to be equal to 1.

-        Otherwise, tile_group_reshaper_model_present_flag is inferred to be equal to 0.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).