| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0298 |
| --- | --- |

| Title: | CE10: CIIP using explicit signaling of weights (CE10-1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andre Seixas Dias Gosala Kulupana Saverio Blasi The Lighthouse White City Place 201 Wood Lane, W12 7TQ, London, UK | Tel: Email: | andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk saverio.blasi@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents the results of test CE10-1.2, where a modification of the combined inter and intra prediction in VTM 4.0 is proposed. In test CE10-1.2, an Intra prediction can be combined with a merge Inter prediction using a weighted average. To allow this weighted combination of Intra and Inter predictions, additional syntax elements are signalled indicating if unequal weights are used and if so, which pair of weights should be applied. Only Planar mode is allowed for Intra prediction when used in combination with an Inter prediction. Average BD-rates of -0.04% (Y), 0.04% (U) and 0.05% (V) are reported for RA with 100% encoding time and 99% decoding time with respect to VTM 4.0. For LD, the reported BD-rates are -0.01% (Y), 0.16% (U) and -0.01% (V) with 101% and 98% encoding and decoding times, respectively.

# Introduction

In VTM 4.0, Intra and Inter predictions can be combined to generate a new prediction [1]. This method, referred to as Combined Inter and Intra Prediction (CIIP), allows a given Inter merge prediction to be combined with an Intra prediction of the same block. Any merge candidate prediction can be combined with one of 4 different Intra prediction modes (Planar, DC, Pure Vertical or Pure Horizontal). In VTM 4.0, when an Inter merge prediction is combined with Intra modes Planar and DC, the combined prediction is given by the average of the two sources of prediction. When Intra modes Pure Vertical and Pure Horizontal are used in the combination, a weighted average is used to combine the two sources of prediction and different weights are used for different parts of the block being predicted, according to the Intra mode used. Regarding signaling, whenever merge mode is selected, a flag is sent indicating if CIIP is used and in case it is, additional signaling specifies which of the 4 Intra prediction modes is used in the combination.

In test CE10-1.2, the Intra prediction used in CIIP is restricted to Planar mode. Additionally, a weighted combination of the two prediction sources is allowed. This is potentially advantageous as simply performing the average of each sample may not be ideal. Depending on the accuracy of either the Intra- or Inter-predicted candidate, there may be many cases in which one of the two predictions may be better suited to represent the content of the current block, but still the other prediction can provide some missing information which would improve the final prediction. In such cases, performing weighted prediction of the two candidates can result in a better combined prediction.

# Proposed method

In order to combine the Inter merge prediction with an Intra prediction (only Planar mode allowed), 3 possible pairs of weights can be used. Once the pair of weights to be used in a given block is determined, all samples within the block are then predicted using the same combined prediction obtained with such weights. The combined prediction Pcombi,j is given by

Pcombi,j=(WInter∙PInteri,j+WIntra∙PIntrai,j)≫3

where PInteri,j and PIntrai,j are the Inter and Intra prediction sources and WInter and WIntra are the weights associated with each prediction. The allowed pairs of weights {WInter, WIntra} are {4, 4}, {6, 2} and {2, 6}.

When CIIP is used, an additional binary flag is signalled to the decoder indicating if the weights used in the Inter + Intra combination are the same or not (unequalWeightFlag). In case the unequalWeightFlag is 0, no additional signaling is required and equal weights {4, 4} are used in the combination. If unequalWeightFlag is 1, an additional binary flag (UnequalWeight) is sent indicating which pair of weights is used in the combination. Table 1 summarises the extra signaling required by the proposed method.

Table 1- Summary of the signaling required by the proposed method.

| UnequalWeightFlag | UnequalWeight | WInter | WIntra |
| --- | --- | --- | --- |
| 0 | - | 4 | 4 |
| 1 | 0 | 6 | 2 |
| 1 | 1 | 2 | 6 |

It is important to emphasize that, since only Planar mode is allowed for the Intra prediction in CIIP, the ciip_luma_mpm_flag and the ciip_luma_mpm_idx syntax elements currently needed in VTM 4.0 are not required in the proposed method. Likewise, the derivation of an MPM list for CIIP modes is no longer needed. The proposed syntax changes to the relevant part of the draft specification text are highlighted below.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| ciip_unequal_weight_flag[ x0 ][ y0 ] | ae(v) |
| if(ciip_unequal_weight_flag[ x0 ][ y0 ] ) |  |
| ciip_unequal_weight[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Performance results

The performance of test CE10-1.2 is summarised in Table 2 and Table 3. The tests were performed using the JVET common test conditions [2] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 4.0, compiled with gcc 4.8.2. All experiments were run using AVX.

Table 2 - Performance of test CE10-1.2 for RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.11% | -0.11% | 0.01% | 100% | 98% |
| Class A2 | 0.00% | 0.09% | 0.12% | 100% | 99% |
| Class B | -0.04% | 0.09% | 0.02% | 100% | 99% |
| Class C | -0.01% | 0.06% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.04% | 0.05% | 100% | 99% |
| Class D | -0.01% | 0.13% | 0.05% | 101% | 97% |
| Class F | -0.10% | -0.12% | -0.13% | 100% | 100% |

Table 3 - Performance of test CE10-1.2 for LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.02% | -0.05% | 100% | 100% |
| Class C | 0.01% | 0.42% | 0.25% | 101% | 95% |
| Class E | -0.02% | 0.11% | -0.28% | 101% | 98% |
| Overall | -0.01% | 0.16% | -0.01% | 101% | 98% |
| Class D | -0.05% | 0.09% | -0.56% | 101% | 97% |
| Class F | -0.43% | -0.56% | -0.94% | 100% | 97% |

It is important to emphasize that the proposed method in test CE10-1.2 gives the encoder the flexibility to perform more or less extensive searches of the best weights to use in the combination of Inter and Intra predictions. For information purposes only, Table 4 shows extra performance results of the method in CE10-1.2 for RA using a different encoding algorithm that performs a more extensive search for the best weights to use in the proposed CIIP combination.

Table 4 – Extra RA performance results for test CE10-1.2 allowing higher encoding complexity.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.17% | -0.09% | -0.20% | 103% | 98% |
| Class A2 | -0.07% | -0.14% | -0.06% | 101% | 100% |
| Class B | -0.08% | -0.16% | -0.12% | 103% | 99% |
| Class C | -0.07% | -0.05% | -0.12% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.11% | -0.12% | 103% | 100% |
| Class D | -0.01% | -0.14% | -0.05% | 103% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

The extra results in Table 4 show that higher gains can be achieved with the method proposed in test CE10-1.2 if higher encoding complexity is allowed.

# Conclusions

In this contribution, the results of test CE10-1.2 are presented. In test CE10-1.2, a weighted combination of the Intra and Inter predictions used in CIIP is proposed. The proposed method limits the Intra mode used in CIIP to be predicted using Planar mode and allows 3 different weighting options to combine an Inter merge prediction with an Intra prediction.

# References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, in JVET-L0100, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video”, in JVET-L1010, October 2018.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).