The following spec is based on JVET-M0427  JVET_M0427_CE12-2_reshaping_delta_spec_v3_clean.docx.The added syntax is highlighted in yellow.

….

The variable OrgCW is set equal to (1 << BitDepthY ) / ( MaxBinIdx + 1).

–	If reshaper_model_min_bin_idx < = i <= reshaper_model_max_bin_idx

RspCW[ i ] = OrgCW + RspDeltaCW[ i ].

–	Otherwise, RspCW[ i ] = 0.

The variable Log2OrgCW is set equal to log2(OrgCW).

### 8.5.5.1.2	Picture reconstructon with mapping process for chroma sample value

Inputs to this process are:

–	an (nCurrSwx2)x(nCurrShx2) array mapped predMapSamples specifying the mapped luma predicted samples of the current block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

The output for this process is reconstructed chroma sample array recSamples.

The recSamples is derived as follows:

–	If ( !tile_group_reshaper_chroma_residual_scale_flag || ( (nCurrSw)x(nCurrSh) <= 4) )

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C ( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) 	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tile_group_reshaper_chroma_residual_scale_flag && ( (nCurrSw)x(nCurrSh) > 4)), the following applies:

The variable varScale is derived as follows:

- invAvgLuma = Clip1Y( ( ij predMapSamples[ (xCurr << 1 ) + i ][ (yCurr << 1) + j ]
 + nCurrSw * nCurrSh *2) / ( nCurrSw * nCurrSh *4 ) )

- The lumaVal is derived by involking the Picture inverse mapping process of luma sample values as specified in 8.5.6.1 with the input of invAvgLuma.

- idxYInv = lumaVal >> Log2OrgCW

- varScale = ChromaScaleCoef[ idxYInv ]

The recSamples is derived as follows:

–	 If tu_cbf_cIdx [ xCurr ][ yCurr ] equal to 1, the following applies:

shiftC = 11
recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1 ( predSamples[ i ][ j ] + Sign( resSamples[ i ][ j ] )
 * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << ( shiftC – 1 ) ) ) >> shiftC ) )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tu_cbf_cIdx[ xCurr ][ yCurr ] equal to 0)

recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1(predSamples[ i ][ j ] )	(8-xxx)