| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0299 |
| --- | --- |

| Title: | On Luma Dependent Chroma Residual Scaling of In-loop Reshaper |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In the reshaping tool of VTM4, Chroma residual scale factor is calculated from the average of luma block in reshaped domain. Specifically, the scale factor is found by the following three steps.

- Calculate average luma value of a block in reshape domain. If inter block, the block is mapped to reshaped domain first.

- Find index (idxS) where the average luma value belongs to in inverse PWL mapping. This requires an iterative operation.

- Find scale factor from the index

In method 1 of this contribution, it is proposed to switch the order of mapping and averaging in step 1. In this way, it only needs to perform mapping on one average value instead of a block. This reduces the number of mapping operations.

In method 2 of this contribution, it is proposed to calculate the residual scaling factor using luma average value in original domain. This eliminates the iterative operation to find the piece index (idxS) in step 2. idxS can now be found by simply shifting the average luma value. In addition, method 2) also reduces the number of mapping operations as in method 1.

It is asserted that proposed methods simplify the process to obtain Chroma residual scaling factor. Results show that there is no loss of performance. Method 1 and method 2 have near identical results. Their results are both are: 0.00%, -0.04%, 0.00% for Y, U, V in RA case, and xxx%, 0.xx%,0.xx% for Y, U,V respectively in LD case.

# Introduction

In Jan. 2019 JVET meeting, reshaping tool [1] is adopted into VTM4. Chroma residue scaling is part of the reshaping tool, and it is designed to compensate for luma signal interaction with the chroma signal.

Chroma residual scaling depends on the average value of the corresponding luma prediction block (for both intra- and inter-coded blocks). Denote avgY' as the average of the luma prediction block in reshaped domain. The Chroma residual scaling factor CScaleInv is computed in the following steps:

- Step 1: Calculate average luma value of a block in reshaped domain

- If intra mode, compute average of intra predicted luma values;

- if inter mode, compute average of forward reshaped inter predicted luma values. That is, the average luma value avgY'TU is computed in reshaped domain.

- Step 2: Find the piece index idxS where avgY’ belongs to in inverse PWL mapping. This is done by a for loop,

for(idxS = 0, idxFound = 0; idxS <= MaxBinIdx; idxS ++ ) {
	if( (avgY < ReshapePivot[ idxS + 1 ] ) {
		idxFound = 1
		break
	}
}

- Find scale factor from the index

CScaleInv  = cScaleInv[idxS] where cScaleInv[] is a pre-computed LUT.

CScaleInv is a constant value for the entire chroma block. With CScaleInv , chroma residual scaling is applied as follows:

Encoder side: CResScale=CRes*CScale=CRes/CScaleInv

Decoder side: CRes=CResScale/CScale=CResScale*CScaleInv

Where CRes is the original chroma residual signal and CResScale is the scaled chroma residual signal.

# Proposed method

This contribution aims at simplifying the procedure to get the Chroma residual scaling factor index idxS. Two methods are proposed.

## Method 1

In VTM4, in the first step of obtaining Chroma residual scaling factor, if inter block, the block is mapped to reshaped domain first, next the average is calculated. In this proposed method, it is proposed to switch the order of averaging and mapping if inter block. That is to perform the average of the block first, then do the mapping of the one average value only. This reduces the number of mapping operations.

In this method, the step 2 and 3 of obtaining Chroma residual scaling factor are not changed.

## Method 2

In this method, it proposes to find the idxS in original pixel domain. This method not only reduces the number of mapping operations, but also it removes the iterative loop to find idxS in step 2. The reason is that in original domain, the pixel value are linearly distributed, and the number of pieces are power of 2, so which piece a luma value belongs to can be easily found by a shift. In contrary, in forward reshaped domain, the pixel values are not linearly distributed, therefore to find the piece index where a pixel belongs to, it has to go through a for loop.

The side by side comparison of proposed method 2 vs. VTM4 are as below.

| Currently in VTM4 | Proposed Method 2 |
| --- | --- |
| - Find avgY’ in reshaped domain - If intra mode, compute average of intra predicted luma values - If inter mode, perform forward reshaping of the block, then compute average of forward reshaped inter predicted luma values -- avgY' - Find index idxS where avgY'_TU belongs to inverse mapping PWL. This is done by a for loop, for( idxS = 0, idxFound = 0; idxS <= MaxBinIdx; idxS++ ) { if( (S < ReshapePivot[ idxS + 1 ] ) { idxFound = 1 break } } - S = cScaleInv[idxS] | - Find avgY in original domain - If intra mode, compute average of intra predicted luma values, and the average value is mapped to original domain; - If inter mode, compute average of inter predicted luma values and obtain avgY. - The index idxS where avgY_TU belongs to can be easily found by idxS = avgY >> nShift, - S = cScaleInv[idxS] Note: nShift equals to log2 of the reshaper piece length. This is already known. For 10bit input and 16 pieces PWL mapping, nShift equals to 6 . |

The proposed method has several advantages:

- For step 2, it does not need an iterative process (a for loop) to find idxS where avgY belongs to. This is easily done by a shift.

- For the inverse mapping of intra block, it does the average of the block first, then only need to do mapping on one average pixel value instead of a block, this further reduces the number of mapping operations.

- For inter blocks, the proposed method does not need forward mapping. The inverse mapping is done to intra blocks instead. On average, there are less intra blocks or CIIP blocks in inter slices than pure inter blocks. So moving the mapping to intra blocks may save some operations on average.

Theoretically, it is equivalent to find idxS on linear mapping using average in original domain or find idxS on the inverse mapping using average in reshaped domain. Except the cases near pivot points, the two proposed methods should yield same index. Method 2 eliminates the for loop to find idxS, so it is simpler and more hardware friendly than method 1.

# Experimental Results

VTM4 software is modified to test proposed methods. Tests are done following the CTC [3]. Since Chroma residual scaling is disabled for separate tree operation, there is no change to All Intra test configuration, so only RA and LD are tested.  Experiment results show there is no noticeable coding loss with both methods but tiny gain. Method 1 and method 2 are almost the same. This is as expected that conceptually method 1 and method 2 are equivalent, but method 2 is more hardware friendly by removing the iterative loop to find idxS.

Table 3.1 Result of Proposed method 1 vs VTM4

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | -0.03% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | -0.07% | -0.02% | 100% | 99% |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.03% | 100% | 99% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | 0.00% | -0.02% | 0.00% | 100% | 101% | -0.03% | 0.29% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  | -0.01% | 0.12% | -0.38% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.00% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | 0.00% | -0.03% | -0.05% | 100% | 100% | -0.02% | 0.26% | -0.28% | 101% | 102% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 100% | #VALUE! | #VALUE! | #VALUE! |  |  |

Table 3.2 Result of Proposed method 2 vs VTM4

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | -0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 100% | 99% |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.03% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | 0.00% | -0.02% | 0.00% | 101% | 102% | -0.03% | 0.29% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  | -0.01% | 0.12% | -0.38% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 100% | 101% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | 0.00% | -0.03% | -0.05% | 101% | 102% | -0.02% | 0.26% | -0.28% | 101% | 102% |
| Class F | -0.02% | -0.01% | -0.01% | 101% | 102% | #VALUE! | #VALUE! | #VALUE! |  |  |

# Conclusion

In the reshaping tool of VTM4, Chroma residual scale factor is calculated from the average of luma block in reshaped domain. It requires an iterative process to find the piece index where the average luma value belongs to in the PWL mapping

Two methods are proposed in this document. Method 1 still calculates the luma average in reshaped domain, but reduces the number of mapping operations by switching the order of averaging and mapping. Method 2, calculates the luma average in original domain, it not only reduces the number of mapping operations, but also replaces the iterative loop to find idxS with a simple shift. Both method have near identical results, there are no loss. Method 2 is simpler and more hardware friendly than method 1. It is proposed to adopt method 2 into VVC draft.

# Reference

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team (JVET), JVET-M0427, Marrakech, MA, Jan 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-M0010, Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).