| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0299 |
| --- | --- |

| Title: | On Luma Dependent Chroma Residual Scaling of In-loop Reshaper |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In the reshaping tool of VTM4, Chroma residual scale factor is calculated from the average of luma block in reshaped domain. Specifically, the scale factor is found by the following three steps.

- Calculate average luma value of a block in reshaped domain. If inter block, the block is mapped to reshaped domain first.

- Find index (idxS) where the average luma value belongs to in inverse PWL mapping. This requires an iterative operation (a for loop).

- Find scale factor from the index

In method 1 of this contribution, it is proposed to switch the order of mapping and averaging in step 1. In this way, it only needs to perform mapping on one average value instead of a block. This reduces the number of mapping operations.

In method 2 of this contribution, it is proposed to calculate the residual scaling factor using luma value in original domain. This eliminates the iterative operation to find the piece index (idxS) in step 2. idxS can now be found by simply shifting the average luma value. There is at most one extra mapping operation which can be done by a LUT, while it eliminates the iterative for loop during piece index identification.

It is asserted that proposed methods simplify the process to obtain Chroma residual scaling factor. Results show that there is no loss of performance. Method 1 and method 2 have near identical results. Their results are both are: 0.00%, -0.04%, 0.00% for Y, U, V in RA case, and -0.02%, 0.14%, 0.01% for Y, U,V respectively in LD case.

In this revision, supplemental results of combineing proposed method 2 and simplification 1 of JVET-N0220 is provided. In this test, it uses the top left luma value in original domain to derive the chroma residual scaling factor. It has benefit of both approaches. Result is 0.00%, -0.04%, -0.06% for Y, U, V in RA case, and -0.03%, 0.14%, -0.06% for Y, U,V respectively in LD case. The results are very close to JVET-N0220 simplification 1 result.

# Introduction

In Jan. 2019 JVET meeting, reshaping tool [1] is adopted into VTM4. Chroma residue scaling is part of the reshaping tool, and it is designed to compensate for luma signal interaction with the chroma signal.

Chroma residual scaling depends on the average value of the corresponding luma prediction block (for both intra- and inter-coded blocks). Denote avgY' as the average of the luma prediction block in reshaped domain. The Chroma residual scaling factor CScaleInv is computed in the following steps:

- Step 1: Calculate average luma value of a block in reshaped domain

- If intra mode, compute average of intra predicted luma values;

- If inter mode, compute average of forward reshaped inter predicted luma values. That is, the average luma value avgY'TU is computed in reshaped domain.

- Step 2: Find the piece index idxS where avgY’ belongs to in inverse PWL mapping. This is done by a for loop,

for(idxS = 0, idxFound = 0; idxS <= MaxBinIdx; idxS ++ ) {
	if( (avgY < ReshapePivot[ idxS + 1 ] ) {
		idxFound = 1
		break
	}
}

- Find scale factor from the index

CScaleInv  = cScaleInv[idxS] where cScaleInv[] is a pre-computed LUT.

CScaleInv is a constant value for the entire chroma block. With CScaleInv , chroma residual scaling is applied as follows:

Encoder side: CResScale=CRes*CScale=CRes/CScaleInv

Decoder side: CRes=CResScale/CScale=CResScale*CScaleInv

Where CRes is the original chroma residual signal and CResScale is the scaled chroma residual signal.

# Proposed method

This contribution aims at simplifying the procedure to get the Chroma residual scaling factor index idxS. Two methods are proposed.

## Method 1

In VTM4, in the first step of obtaining Chroma residual scaling factor, if inter block, the block is mapped to reshaped domain first, next the average is calculated. In this proposed method, it is proposed to switch the order of averaging and mapping if inter block. That is to perform the average of the block first, then do the mapping of the one average value only. This reduces the number of mapping operations.

In this method, the step 2 and 3 of obtaining Chroma residual scaling factor are not changed.

## Method 2

In this method, it proposes to use luma value in original domain to find the idxS. This method not only reduces the number of mapping operations, but also removes the iterative loop to find idxS in step 2. The reason is that in original domain, the pixel value are linearly distributed, and the number of pieces are power of 2, so which piece a luma value belongs to can be easily found by a shift. In contrary, in reshaped domain, the pixel values are not linearly distributed, therefore to find the piece index where a pixel belongs to, it has to go through a for loop.

The side by side comparison of proposed method 2 vs. VTM4 are as below.

| Currently in VTM4 | Proposed Method 2 |
| --- | --- |
| - Find avgY’ in reshaped domain - If intra mode, compute average of intra predicted luma values - If inter mode, perform forward reshaping of the block, then compute average of forward reshaped inter predicted luma values -- avgY' - Find index idxS where avgY'_TU belongs to inverse mapping PWL. This is done by a for loop, for( idxS = 0, idxFound = 0; idxS <= MaxBinIdx; idxS++ ) { if( (S < ReshapePivot[ idxS + 1 ] ) { idxFound = 1 break } } - S = cScaleInv[idxS] | - Find avgY in original domain - If intra mode, compute average of intra predicted luma values, and map the one average luma value to original domain; - If inter mode, compute average of inter predicted luma values and obtain avgY. - The index idxS where avgY_TU belongs to can be easily found by idxS = avgY >> nShift, - S = cScaleInv[idxS] Note: nShift equals to log2 of the reshaper piece length. This is already known. For 10bit input and 16 pieces PWL mapping, nShift equals to 6 . |

The proposed method has several advantages:

- For step 2, it does not need an iterative process (a for loop) to find idxS where avgY belongs to. This is easily done by a shift.

- For the inverse mapping of intra block, it does the average of the block first, then only need to do mapping on one luma value instead of a block, this further reduces the number of mapping operations.

- For inter blocks, the proposed method does not need forward mapping. The inverse mapping is done to intra blocks instead. On average, there are less intra blocks or CIIP blocks in inter slices than pure inter blocks. So moving the mapping to intra blocks may save some operations on average. For intra block, comparing to existing approach in VTM4, it only adds mapping process to one luma value, while it saves the iterative for loop, so the overall complexity is still less for intra block.

Theoretically, it is equivalent to find idxS on linear mapping using luma value in original domain or find idxS on the inverse PWL mapping using luma value in reshaped domain. Except the cases near pivot points, the two proposed methods should yield same index. Method 2 eliminates the “for” loop to find idxS, so it is simpler and more hardware friendly than method 1.

Even if prediction samples are all in reshaped domain, the proposed method to identify the piece index is still simpler because there is at most one extra mapping operation which can be done by a LUT, while it eliminates the iterative for loop during piece index identification. Otherwise, if use lumaY in reshaped domain, the for loop cannot be avoided. This is shown in Fig 1.

- VTM4                          (b) proposed

Fig. 1   Identification of piece index in the PWL mapping

# Combined test of method 2 with JEVT-N0220 simplification 1

# In JEVT-N0220 simplification 1, luma value at the top left position of a block is used to derive the chroma luma scale factor. This luma value is in reshaped domain as in VTM4. It is asserted that this simplification reduces the pipeline delay.

In this combined test, luma value in original domain for Piecewise index identification, but the luma value is mapped from top left sample of a luma block instead of the block average. This has the benefit of JVET-N0220 simplification 1, as well as able to replace piecewise index identification process of Chroma scaling with a simple shift. The flow chart is same as what’s shown in Fig. (1), and now the input luma value is from the top left of a luma block instead of the average of the luma block.

# Experimental Results

VTM4 software is modified to test proposed methods. Tests are done following the CTC [3]. Since Chroma residual scaling is disabled for separate tree operation, there is no change to All Intra test configuration, so only RA and LD are tested.  Experiment results show there is no noticeable coding loss with both methods. Method 1 and method 2 have almost the same result, and results of many sequences are identical. This is as expected because theoretically method 1 and method 2 are equivalent, but method 2 is more hardware friendly by removing the iterative loop to find idxS.

Table 3.1 Result of Proposed method 1 vs VTM4

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | -0.03% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | -0.07% | -0.02% | 100% | 99% |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.03% | 100% | 99% | -0.01% | 0.03% | 0.14% | 100% | 99% |
| Class C | 0.00% | -0.02% | 0.00% | 100% | 101% | -0.03% | 0.29% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  | -0.01% | 0.12% | -0.38% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.00% | 100% | 100% | -0.02% | 0.14% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.05% | 100% | 100% | -0.02% | 0.26% | -0.28% | 101% | 102% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 100% | -0.05% | -0.15% | -0.09% | 100% | 102% |

Table 3.2 Result of Proposed method 2 vs VTM4

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | -0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.07% | -0.02% | 100% | 99% |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.03% | 100% | 100% | -0.01% | 0.03% | 0.14% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.00% | 101% | 102% | -0.03% | 0.29% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  | -0.01% | 0.12% | -0.38% | 100% | 100% |
| Overall | 0.00% | -0.04% | 0.00% | 100% | 101% | -0.02% | 0.14% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.05% | 101% | 102% | -0.02% | 0.26% | -0.28% | 101% | 102% |
| Class F | -0.02% | -0.01% | -0.01% | 101% | 102% | -0.05% | -0.15% | -0.09% | 100% | 102% |

# Table 3. Combined test result of method 2 and JVET-N0220 simplification 1

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.02% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.03% | -0.05% | -0.03% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | -0.07% | -0.02% | 100% | 101% | -0.01% | -0.07% | 0.22% | 100% | 100% |
| Class C | 0.02% | 0.01% | -0.19% | 101% | 100% | -0.03% | 0.51% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  | -0.05% | 0.01% | -0.63% | 101% | 101% |
| Overall | 0.00% | -0.04% | -0.06% | 100% | 100% | -0.03% | 0.14% | -0.06% | 100% | 100% |
| Class D | 0.01% | -0.02% | 0.15% | 101% | 100% | -0.02% | -0.31% | -0.49% | 101% | 101% |
| Class F | -0.01% | 0.07% | 0.01% | 101% | 100% | -0.13% | -0.35% | 0.01% | 100% | 100% |

# This supplemental result of comined test is very close to the JVET-N0220 simplification 1 result. For many rate points, the RD are identical. This shows that whether using luma value in reshaped domain or in original domain are equivalent. The one in original domain has lower complexity which allows to eliminate the iterative loop during piecewise index identification.

# Conclusion

In the reshaping tool of VTM4, Chroma residual scale factor is calculated from the average of luma block in reshaped domain. It requires an iterative process to find the piece index where the average luma value belongs to in the PWL mapping

Two methods are proposed in this document. Method 1 still calculates the luma average in reshaped domain, but reduces the number of mapping operations by switching the order of averaging and mapping. Method 2, calculates the luma average in original domain, it not only reduces the number of mapping operations, but also replaces the iterative loop to find idxS with a simple shift. Both method have near identical results, there are no loss. Method 2 is simpler and more hardware friendly than method 1.

Supplementary result of combining the proposed method 2 and JVET-N0220 simplification 1 is also provided, where luma value of top left of a block is mapped to original domain for Piecewise index identification. This shows that proposed method 2, specifically, the part of piecewise index identification in original domain, may be combined with other proposals and get joint benefits.

It is proposed to adopt method 2 or the piecewise index identification part of method 2 into VVC draft.

# Reference

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team (JVET), JVET-M0427, Marrakech, MA, Jan 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-M0010, Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).