#### 8.7.5.4 Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the transform block width,

- a variable nCurrSh specifying the transform block height,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

Output of this process is a reconstructed chroma picture sample array recSamples.

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If tile_group_chroma_residual_scale_flag is equal to 0 or  nCurrSw * nCurrSh is less than or euqal to 4, the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C ( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) 	(8-1063)

- Otherwise (tile_group_chroma_residual_scale_flag is equal to 1 and nCurrSw * nCurrSh is greater than 4), the following applies:

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

invAvgLuma = Clip1Y( ( k=02*nCurrSw-1l=02*nCurrSh-1PredMapSamples[ k ][ l ]	(8-1064)	 + nCurrSw * nCurrSh *2 ) / ( nCurrSw * nCurrSh *4 ) )

- The lumaVal is derived by involking the Picture inverse mapping process of luma sample values as specified in 8.7.5.3.1 with the input of invAvgLuma.

- idxYInv = lumaVal >> Log2(OrgCW)

- The variable varScale is derived as follows:

varScale = ChromaScaleCoeff[ idxYInv ]	(8-1065)

- The recSamples is derived as follows:

- If tu_cbf_cIdx [ xCurr ][ yCurr ] equal to 1, the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1( predSamples[ i ][ j ] + 	(8-1066)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tu_cbf_cIdx[ xCurr ][ yCurr ] equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1(predSamples[ i ][ j ] )	(8-1067)

