| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0300 |
| --- | --- |

| Title: | On High Bit Depth Signaling of In-loop Reshaper |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In luma mapping with chroma scaling (so-called reshaper), the absolute reshaper delta code word values are signaled for every piece-wise linear (PWL) piece. Considering that the absolute reshaper delta code word values depend on internal coding bit-depth, a generalized reshaping code word signalling method depending on coding bit-depth is proposed. In addition, the proposed method can reduce number of bits for signalling reshaper at high internal bit-depth where the absolute reshaper delta code word values are just scaled by 2*(internal_bit_depth - 10).

This change has no impact on CTC test condition. For coding with 12-bit internal bit depth, it has no change on PSNR while can save bits to signal reshaper. In terms of BD rate, there are a small gain of -0.01% in AI case, 0.0% and 0.0% for RA and LD cases.

# Introduction

In Jan. 2019 JVET meeting, reshaping tool is adopted into VTM4 [1]. The mapping function is a piecewise linear model with 16 equal pieces. The reshaper model is defined by pivot points and signaled as in Table 1.

Table 1. Syntax for tile_group_reshaper_model

| tile_group_reshaper_model () { | Descriptor |
| --- | --- |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | ue(v) |
| for ( i = reshaper_model_min_bin_idx; i <= reshaper_model_max_bin_idx; i++) { |  |
| reshape_model_bin_delta_abs_CW [ i ] | u(v) |
| if( reshaper_model_bin_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_bin_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| } |  |

Reshaper delta code word value RspDeltaCW[i] is signaled as an absolute value reshape_model_bin_delta_abs_CW[i] and a sign reshaper_model_bin_delta_sign_CW_flag[i] for the reshaper bins within min and max reshaper bin indices. The number of bits maxNbitsNeededDeltaCW needed to signal the reshaper delta code word value is also signalled in bitstream, and it equals reshaper_model_bin_delta_abs_cw_prec_minus1+1.

At encoder, the signaled reshaper delta code word value RspDeltaCW[i] is obtained by

RspDeltaCW[i]= ReshapePivot[ i + 1]- ReshapePivot[ i ] – OrgCW;

Where OrgCW =(1 << BitDepthY ) / 16,  and equals 64 for 10-bit video.

At decoder, the (i+1)th pivot of the forward reshaper are constructed by

ReshapePivot[ i + 1] = ReshapePivot[ i ] + RspCW[ i ]

Where RspCW[ i ] = OrgCW + RspDeltaCW[ i ];

# Problem Statement

The current reshaper delta code word signalled is mainly designed for internal coding bit depth 10. Even though it works for other coding bit depth, the signalling is not optimized. For higher coding bit depth, given the same MinBinIdx and MaxBinIdx, the OrgCW and RspCW[i] and signalled RspDeltaCW[i] value are all larger, and they are multiples of those at lower bit depth. An example is given for internal BitDepthY=10  and 12 when reshaperModelMinBinIdx is 1 and   reshaperModelMaxBinIdx is 14. This is obtained from log of actual coded bitstream.

|  | BitDepthY=10, OrgCW=64 |  |  |  | BitDepthY=12, OrgCW=256 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Idx | RspDeltaCW | RspCW | ReshapePivot |  | RspDeltaCW | RspCW | ReshapePivot |
| 1 | 2 | 66 | 66 |  | 8 | 264 | 264 |
| 2 | 2 | 66 | 132 |  | 8 | 264 | 528 |
| 3 | 2 | 66 | 198 |  | 8 | 264 | 792 |
| 4 | 2 | 66 | 264 |  | 8 | 264 | 1056 |
| 5 | 2 | 66 | 330 |  | 8 | 264 | 1320 |
| 6 | 5 | 69 | 399 |  | 20 | 276 | 1596 |
| 7 | 8 | 72 | 471 |  | 32 | 288 | 1884 |
| 8 | 5 | 69 | 540 |  | 20 | 276 | 2160 |
| 9 | 2 | 66 | 606 |  | 8 | 264 | 2424 |
| 10 | 2 | 66 | 672 |  | 8 | 264 | 2688 |
| 11 | 2 | 66 | 738 |  | 8 | 264 | 2952 |
| 12 | 2 | 66 | 804 |  | 8 | 264 | 3216 |
| 13 | 2 | 66 | 870 |  | 8 | 264 | 3480 |
| 14 | -1 | 63 | 933 |  | -4 | 252 | 3732 |

In the above example, for internal BitDepthY=10, maxNbitsNeededDeltaCW is 4; while for internal BitDepthY=12, maxNbitsNeededDeltaCW becomes 6. So roughly, signaling all RspDeltaCW[i] for BitDepthY=12 would take extra bits (ex: 28bits). However, it can be observed that the RspDeltaCW[i ] for BitDepthY=12 is multiple (i.e. 4 times) of the RspDeltaCW [ i ] at BitDepthY=10. Therefore current way of signaling RspDeltaCW[i] for higher internal bit depth is not optimal.

# Proposed method

It is proposed to modify the actual signalled RspDeltaCW[i] based on the internal luma bit depth.

At encoder, if bitdepthY > 10, RspDeltaCW[i] =  RspDeltaCW[i] >>(bitdepthY-10)

So the actual delta code word is scaled down by 2*(bitdepthY-10). Encoder need to ensure that the pivot points of reshaper are multiples of 2*(bitdepthY-10). This is already the case in VTM4.

Then at decoder, if bitdepthY > 10, RspDeltaCW[i] =  RspDeltaCW[i] <<(bitdepthY-10)

The maxNbitsNeededDeltaCW is determined based on the modified RspDeltaCW. With this method, it will be reduced by (bitdepthY-10) comparing to VTM4 when using non-modified RspDeltaCW.

The rest reshaper construction and mapping process remain the same.

# Experimental Results

VTM4 software is modified to allow testing higher bitdepth and it also incorporated the JEVT ticket #199 [3] on fixing reshaper higher bit depth bug. The proposed method is implemented on the top of the bug fixed VTM4.0. Both anchor and test are run with internal bit depth 12 (command line option “--InternalBitDepth=12”) , other conditions are following the CTC [4]. Experiments result show that there is no change of PSNR in all tests. There is small reduction of bits and this lead to 0.01% to 0.03% BD rate reduction.

Table 1 Result of Proposed method vs VTM4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.02% | -0.01% | -0.01% | 100% | 101% |
| Class E | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.03% | -0.02% | -0.02% | 100% | 101% |
| Class F | -0.01% | -0.01% | -0.01% | 100% | 101% |

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 100% | 100% | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.01% | -0.01% | -0.01% | 101% | 100% |

# Conclusion

This contribution proposes to modify the reshaper delta code word signalling to account for internal coding bit depth. This generalizes the reshaping code word signalling and reduces the unnecessary bits at high bit depth with no change on PSNR. There is no impact on CTC. It is proposed to adopt this modification into VVC draft.

# Reference

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team (JVET), JVET-M0427, Marrakech, MA, Jan 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4),” Document of Joint Video Experts Team (JVET), JVET-M1002, Marrakech, MA, Jan 2019.

- JVET bug tracker Ticket #199, “fix199: issue caused by using g_aucLog2 in 12b reshaping”, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/330

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-M0010, Marrakech, MA, Jan 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).