| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0302_v1 |
| --- | --- |

| Title: | CE10: CIIP with position-independent weights (Test CE10-1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | lphamvan@qti.qualcomm.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 10-1.1 for simplification of the combined inter-intra prediction (CIIP) mode. The number of intra modes allowed in CIIP is reduced from 4 to only 1, i.e., planar mode, and the CIIP MPM list construction is therefore also removed. In addition, an adaptive position-independent weight scheme based on the number of intra-coded neighbouring blocks is used. The average luma BD-rate gain is 0.05% and 0.03% for the RA and LDB configurations, respectively. The average encoding time (EncT 98%) is reportedly reduced without any average increase of decoding time (DecT 100%) under both the RA and LDB configurations.

# Introduction

The combined inter-intra prediction (CIIP) mode that improves the merge mode by combining one intra prediction and one merge indexed prediction was adopted in the 12th JVET meeting in Macao. In VTM4.0, candidates from a set of 4 intra modes, including planar, DC, horizontal and vertical predictions, are evaluated to select the best mode to combine with a merge indexed prediction. In the combining process, if planar or DC mode is selected, or the width or height (or both) of the block is smaller than 4, equal weights are used. Otherwise, the weights for intra and inter predicted samples (wIntra, wInter) are different based on the region of the samples within the coding block.

# Proposed Method

In this CE test (CE10-1.1), the CIIP mode is simplified by reducing the number of intra modes allowed from 4 to only 1, i.e., planar mode, and the CIIP MPM list construction is therefore also removed. The weight for intra and inter predicted samples are adaptively selected based on the number of neighbouring intra-coded blocks (Figure 1). The (wIntra, wInter) weights are adaptively set as follows. If both top and left neighbours are intra-coded, (wIntra, wInter) is set equal to (3, 1). Otherwise, if one of these blocks is intra-coded, these weights are identical, i.e., (2, 2), else the weights are set equal to (1, 3).

Figure 1 The position of the top and left neighbouring blocks that are checked to decide the weights for intra and inter predicted samples in blending.

# Experimental Results

The proposed technique has been implemented on top of VTM4.0. The experimental results under the RA and LDB configuration are summarized in Table 1.

It is reported that the proposed method has an average luma BD-rate gain of 0.05% and 0.03% for the RA and LDB configuration, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. The runtimes are as follows: (EncT, DecT) is (98%, 100%) under both the RA and LDB configurations. It should be noted that the gain is higher for Class A1 and Class F, i.e., 0.13% for Class A1 in RA and 0.11% and 0.45% for Class F in RA and LDB, respectively.

Table 1 Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.06% | -0.08% | 100% | 101% |
| Class A2 | 0.00% | -0.04% | 0.06% | 99% | 100% |
| Class B | -0.06% | -0.05% | 0.06% | 98% | 100% |
| Class C | -0.02% | -0.05% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | 0.02% | 98% | 100% |
| Class D | 0.02% | -0.07% | -0.08% | 98% | 100% |
| Class F | -0.11% | -0.09% | -0.06% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.07% | 0.04% | 98% | 100% |
| Class C | 0.03% | 0.67% | 0.24% | 98% | 100% |
| Class E | -0.11% | 0.00% | -0.10% | 98% | 100% |
| Overall | -0.03% | 0.20% | 0.07% | 98% | 100% |
| Class D | 0.03% | 0.80% | 0.19% | 97% | 98% |
| Class F | -0.45% | -0.27% | -0.55% | 99% | 103% |

# Conclusions

This contribution proposes to simplify the CIIP mode. The proposed technique leads to an average luma BD-rate gain of 0.05% and 0.03% for the RA and LDB configurations, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. The runtimes are as follows: (EncT, DecT) is (98%, 100%) under both the RA and LDB configurations. It is recommended to adopt this proposed technique into VVC.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).