| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0304-v2 |
| --- | --- |

| Title: | CE1-related: Combination of post-reconstruction filtering and bi-directional LIC mutually exclusive |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Fabrice Urban, Franck Galpin | Tel: Email: | +33.2.99.27.32.42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to combine LIC and post-reconstruction filtering in mutual exclusive way. Then the decoder pipeline worst case is unchanged compared to implementation of LIC or post-reconstruction filtering only.

It is reported the proposed combination of LIC with Bilateral post-reconstruction filtering implemented on top of CE1-5.4b and CE1-5.5b achieves the following gains (YUV):

- CE1-5.4b: -0.82%,-0.43%, -0.41% (RA) and -0.90%,0.18%, 0.01% (LDB)

- CE1-5.5b: -0.68%,-0.27%, -0.20% (RA) and -0.87%,0.38%, 0.06% (LDB)

It is reported the proposed combination of LIC with Hadamard post-reconstruction filtering implemented on top of CE1-5.4b and CE1-5.5b achieves the following gains (YUV):

- CE1-5.4b: -0.79%,-0.40%, -0.39% (RA) and -0.90%,0.47%, 0.07% (LDB)

- CE1-5.5b: -XX%,-XX%, -XX% (RA) and -XX%,-XX%, -XX% (LDB)

# Introduction

During the last JVET Meeting (Marrakech, MA, 9–18 Jan. 2019), to minimize implementation complexity of LIC and post-reconstruction filtering, it was recommended to follow some constraints when applying to current CU:

  - Not using reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

  - Disable for CIIP blocks

  - No temporal inheritance of LIC flag

  - No pruning based on LIC flag in merging candidate list generation

  - Disable LIC for all subblock modes

  - The VPDU issue should be solved (e.g., disabling for 128xN or by other method which avoids violating VPDU constraint).

  - Bi-Prediction issue should be solved (LIC should not have to be applied twice, i.e. for each L0/L1). The LIC is applied after the bi-predictive combination.

Additionally, to further alleviate the bi-prediction pipeline processing, it has been proposed in CE1 test-1.5.4b to disable LIC when BPWA has non-default weights. The rationale being that it may be sub-optimal cascading two successive weighting and clipping stages and applying additional BPWA non-default weights on top of LIC weights that were specially computed to compensate illumination variation may be counter-productive.

All these limitations have been applied to LIC. Similarly, some limitations have been applied to post-reconstruction filters so that no using reconstructed intra coded samples, not to burden the decoding of intra pipeline or delay inter pipeline.

# Proposal

This contribution proposes to combine LIC and post-reconstruction filtering in mutual exclusive way. Then the decoder pipeline worst case is unchanged compared to implementation of LIC or post-reconstruction filtering only.

Two post-reconstruction filters have been considered (bilateral and Hadamard), the combination of LIC and diffusion-filter being addressed in CE1-7.6. For CUs coded in Inter, the value of LIC flag is either coded, inferred (merge mode) or forced to false (subblocks modes, VDPU constraint or size inferior to 64 luma samples). In case LIC flag is false, then post-reconstruction filtering may apply if the conditions are met. If the LIC flag is true, then post-reconstruction filtering does not apply.

Figure 1: overview of the proposed pipeline in inter (top).

# Combining bi-prediction LIC and bilateral-filter

The code is based on CE1-5.4b or CE1-5.5b and CE1-1.1b.

|  | Combination LIC (CE1-5.4b) and bilateral filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.51% | -0.51% | 136% | 103% |  |  |  |  |  |
| Class A2 | -0.72% | -0.47% | -0.41% | 120% | 103% |  |  |  |  |  |
| Class B | -0.93% | -0.47% | -0.36% | 123% | 101% | -1.14% | -0.05% | -0.02% | 131% | 103% |
| Class C | -0.68% | -0.28% | -0.40% | 127% | 104% | -1.03% | 0.17% | -0.18% | 136% | 104% |
| Class E |  |  |  |  |  | -0.34% | 0.57% | 0.34% | 104% | 102% |
| Overall | -0.82% | -0.43% | -0.41% | 126% | 102% | -0.90% | 0.18% | 0.01% | 125% | 103% |
| Class D | -0.51% | -0.04% | -0.05% | 124% | 105% | -0.71% | 0.84% | 0.32% | 132% | 106% |
| Class F | -1.09% | -0.54% | -0.65% | 119% | 104% | -3.22% | -2.70% | -2.48% | 126% | 103% |

|  | Combination LIC (CE1-5.5b) and bilateral filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.77% | -0.32% | -0.28% | 116% | 104% |  |  |  |  |  |
| Class A2 | -0.64% | -0.49% | -0.30% | 113% | 104% |  |  |  |  |  |
| Class B | -0.81% | -0.22% | -0.13% | 113% | 101% | -1.03% | 0.11% | -0.21% | 114% | 103% |
| Class C | -0.49% | -0.13% | -0.14% | 112% | 104% | -0.87% | 0.41% | 0.12% | 115% | 105% |
| Class E |  |  |  |  |  | -0.59% | 0.76% | 0.41% | 110% | 103% |
| Overall | -0.68% | -0.27% | -0.20% | 113% | 103% | -0.87% | 0.38% | 0.06% | 113% | 104% |
| Class D | -0.30% | 0.08% | 0.14% | 111% | 104% | -0.57% | 0.93% | 0.26% | 115% | 106% |
| Class F | -1.12% | -0.56% | -0.61% | 109% | 104% | -3.32% | -2.60% | -2.24% | 112% | 104% |

# Combining bi-prediction LIC and Hadamard-filter

The code is based on CE1-5.4b or CE1-5.5b and CE1-1.2.1b (TEST=CE1_2_1b).

|  | Combination LIC (CE1-5.4b) and hadamard filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -0.36% | -0.54% | 137% | 100% |  |  |  |  |  |
| Class A2 | -0.72% | -0.59% | -0.41% | 123% | 102% |  |  |  |  |  |
| Class B | -0.87% | -0.35% | -0.30% | 125% | 101% | -1.15% | 0.33% | 0.12% | 138% | 107% |
| Class C | -0.64% | -0.34% | -0.38% | 127% | 100% | -1.11% | 0.27% | -0.17% | 139% | 102% |
| Class E |  |  |  |  |  | -0.21% | 0.96% | 0.32% | 112% | 100% |
| Overall | -0.79% | -0.40% | -0.39% | 127% | 101% | -0.90% | 0.47% | 0.07% | 131% | 103% |
| Class D | -0.38% | 0.05% | -0.13% | 123% | 102% | -0.78% | 0.58% | -0.45% | 131% | 105% |
| Class F | -1.14% | -0.54% | -0.60% | 117% | 102% | -3.26% | -2.52% | -2.69% | 126% | 100% |

|  | Combination LIC (CE1-5.5b) and hadamard filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.75% | -0.19% | -0.18% | 113% | 100% |  |  |  |  |  |
| Class C | -0.47% | -0.12% | -0.16% | 112% | 103% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.28% | 0.08% | 0.13% | 111% | 102% |  |  |  |  |  |
| Class F | -1.16% | -0.52% | -0.57% | 110% | 104% |  |  |  |  |  |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).