| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0305 |
| --- | --- |

| Title: | CE3-related: Luma intra mode coding with restricted MPM derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Andre Seixas Dias Gosala Kulupana 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Tel: Email: | saverio.blasi@bbc.co.uk andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

In the VVC WD, the process of decoding a mode that is not in the MPM list depends on the computation of the MPM list and sorting of the MPM modes. This contribution proposes to remove this dependency, allowing the process of decoding modes that are not in the MPM list to be performed with no need to evaluate the MPM list. An average BD-rate of 0.06% is reported for AI configuration with an encoder complexity of 98%. The reported decoder complexity is 95% on average.

In v2 of the contribution results are reported for additional encoder-only changes that improve the average luma BD rate performance.

# Proposed method

The process of decoding a mode that is not in the MPM list in the VVC WD [1] depends on the computation of the MPM list and sorting of the modes in the list. This contribution proposes to remove this dependency altogether. The luma intra mode IntraPredModeY[ xCb ][ yCb ] is directly set equal to intra_luma_mpm_remainder[ xCb ][ yCb ]. The maximum value of intra_luma_mpm_remainder[ ][ ] is changed from 60 to 66, namely all the 67 available modes.

While this change introduces additional redundancies in the luma intra mode coding, this modification removes completely the need at the decoder for computing the MPM list in case intra_luma_mpm_flag[ x0 ][ y0 ] = = 0, namely if the current mode is not in the list. The step of sorting the modes in the list at both encoder and decoder side is also removed. This reduces the complexity of the decoder, with very limited impact on the encoder compression efficiency. Moreover, additional flexibility is provided at the encoder side, as a bitstream could be produced which does not require any MPM list computation at all.

The following changes to Clause 8.4.2 are proposed:

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ] derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

Table 9-4 is modified as follows:

| intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 66 |
| --- | --- | --- |

# Results

Tests were performed using the JVET common test conditions [2] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 4.0, compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1: Performance of the proposed luma intra mode coding with no MPM derivation

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.07% | 0.03% | 96% | 93% | 0.02% | -0.01% | 0.07% | 100% | 97% |
| Class A2 | 0.04% | 0.11% | 0.04% | 98% | 92% | 0.00% | -0.03% | 0.01% | 100% | 98% |
| Class B | 0.05% | 0.02% | 0.05% | 98% | 95% | 0.03% | 0.07% | 0.00% | 100% | 99% |
| Class C | 0.09% | 0.04% | 0.02% | 99% | 97% | 0.07% | -0.06% | -0.22% | 99% | 99% |
| Class E | 0.08% | 0.08% | -0.03% | 99% | 96% |  |  |  |  |  |
| Overall | 0.06% | 0.06% | 0.02% | 98% | 95% | 0.03% | 0.00% | -0.04% | 100% | 99% |
| Class D | 0.10% | -0.03% | -0.20% | 99% | 94% | 0.04% | -0.33% | -0.35% | 100% | 96% |
| Class F | 0.09% | 0.04% | 0.12% | 98% | 97% | 0.03% | 0.02% | -0.01% | 100% | 100% |

An encoder-only change was also tested in which the size of the list of modes to test in full RD sense numModesForFullRD is decreased by one, while the number of MPM candidates that are pushed into the list is increased by one, namely:

const int numCand = PU::getIntraMPMs( pu, uiPreds ) + 1;

This keeps the number of modes to test in RD sense very similar to the CTC case, while encouraging more MPM modes to be tested in full RD. Results are presented in Table 2.

Table 2 - Performance of the proposed luma intra mode coding with no MPM derivation (additional encoder-only changes)

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.04% | -0.03% | 97% | 91% |  |  |  |  |  |
| Class A2 | -0.05% | 0.08% | -0.01% | 97% | 90% |  |  |  |  |  |
| Class B | -0.05% | -0.03% | 0.03% | 100% | 93% | 0.00% | -0.02% | -0.15% | 100% | 99% |
| Class C | 0.01% | -0.05% | 0.02% | 102% | 97% | 0.08% | 0.19% | -0.04% | 99% | 97% |
| Class E | -0.05% | 0.05% | 0.02% | 102% | 96% |  |  |  |  |  |
| Overall | -0.04% | 0.00% | 0.01% | 100% | 94% |  |  |  |  |  |
| Class D | 0.07% | 0.05% | -0.15% | 102% | 95% | 0.08% | 0.07% | -0.02% | 101% | 96% |
| Class F | 0.10% | -0.09% | 0.03% | 102% | 98% | 0.06% | -0.21% | -0.26% | 67% | 71% |

# Conclusions

This contribution proposes to decouple the derivation of luma intra modes that are not in the MPM list from the computation of the MPM list. The step of sorting the modes in the list at both encoder and decoder side is also removed. This reduces the complexity of the decoder and introduces more flexibility at the encoder side. Average 0.06% BD-rate losses are obtained in the AI configuration with encoder complexity savings of 98% and decoder complexity savings of 95% are obtained on average, with particularly large decoder complexity savings of 92% in larger resolution Class A2 sequences.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001-v5, January 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” in JVET-M1010, January 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).