| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0306-v1 |
| --- | --- |

| Title: | non-CE1: simplification of LIC parameters derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Fabrice Urban, Franck Galpin | Tel: Email: | +33.2.99.27.32.42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the derivation of the LIC parameters. The current method is based on LSM and quadratic sum terms operations whereas the proposal is based on linear sum terms operations. It is reported the proposed method reduce the number of operations.

# Introduction

In CE1, the LIC parameters (a;b) of (Equation 1) are derived through LSM method (Equation 2) with the reference and reconstructed samples of the L-shape around the current block. To avoid integer divisions, the number of samples should be a power of 2 so that the divisions are carried out with right shifting. The size of the L-shape is sizeL = 2 x min(cuWidth;cuHeight).

predcorr(s) = a . pred(s) + b

Equation 1: the LIC parameters (a;b) allow correcting the prediction.

a=refs×curr-ref(s)×cur(r)Ncurr²-ref(s)×ref(s)N

b=cur(r)N-a×ref(s)N

Equation 2: the LIC parameters (a;b) are derived with LSM method.

The Table 1 estimates the number of operations (sums, multiplications, divisions with right shifting and integer division) of the LIC parameters derivation based on the LSM method, where N=2k is the number of samples and d is the samples bit-depth.

Table 1: estimation of the number of operations in the LSM method.

|  | memory (bits) | sum | mult | Div (shift) | Div |
| --- | --- | --- | --- | --- | --- |
| cur(r) | d+k | N |  |  |  |
| ref(s) | d+k | N |  |  |  |
| refs×curr | 2d+k | N | N |  |  |
| curr² | 2d+k | N | N |  |  |
| a |  | 2 | 2 | 2 | 1 |
| b |  | 1 | 1 | 1 |  |
| Total | 6d+4k | 4N + 3 | 2N+3 | 3 | 1 |

# Proposal

It is proposed to simplify the derivation of the LIC parameters and reduce the number of operations. The parameter “a” is computed as the ratio of reconstructed and reference mean absolute deviations (Equation 3). The parameter “b” is computed in the same way as the LSM method.

a=abs curr- cur(r)N abs refs- ref(s)N

b=cur(r)N-a×ref(s)N

Equation 3: the proposed LIC parameters (a;b) derivation equations.

As in LSM method, an additional regularization process is applied to avoid extremum values. The proposed regularization stage is the following:

if  ( b < -bmax && a > 1+damax ) 		a = 1+damax and b computed with Equation 3

else if ( b > bmax && a < 1-damin ) 	a = 1-damin and b computed with Equation 3

where:	damax=0.25 and bmax=100

The Table 2 estimates the number of operations of the proposed method. The number of sums is slightly increased, but all the other number of operations are reduced. The number of multiplications is significantly reduced.

Table 2: estimation of the number of operations in the LSM method.

|  | memory | sum | mult | Div (shift) | Div |
| --- | --- | --- | --- | --- | --- |
| cur(r)N | d+k | N |  | 1 |  |
| ref(s)N | d+k | N |  | 1 |  |
| abscurr-… | d+k | 2N |  |  |  |
| absrefs-… | d+k | 2N |  |  |  |
| a |  |  |  |  | 1 |
| b |  | 1 | 1 |  |  |
| Total | 4d+4k | 6N + 1 | 1 | 2 | 1 |

# Results

The proposal is implemented based on CE1-5.1b (uni-dir LIC).

Table 3: proposal implementation based on CE1-5.1b .

|  | Proposal based on (CE1-5.1b) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.18% | -0.33% | 119% | 97% |  |  |  |  |  |
| Class A2 | -0.22% | -0.38% | -0.26% | 109% | 100% |  |  |  |  |  |
| Class B | -0.29% | -0.20% | -0.20% | 114% | 103% |  |  |  |  |  |
| Class C | -0.25% | -0.30% | -0.32% | 117% | 102% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.31% | -0.26% | -0.27% | 115% | 101% |  |  |  |  |  |
| Class D | -0.14% | -0.10% | -0.29% | 115% | 102% |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 2: proposal difference with CE1-5.1b .

|  | Proposal difference with (CE1-5.1b) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.02% | -0.03% |  |  |  |  |  |  |  |
| Class C | 0.04% | 0.13% | 0.09% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.01% | 0.27% | -0.09% |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).