#### Local illumination compensation process

##### General

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a luma motion vector mvLX given in 1/16-luma-sample units,

- a chroma motion vector mvCLX given in 1/32-chroma-sample units,

- the selected reference picture sample array refPicLXL and the arrays refPicLXCb and refPicLXCr,

- an (cbWidth)x(cbHeight) array predSamplesLXL of prediction luma sample values,

- two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesLXCb and predSamplesLXCr of prediction chroma sample values.

Outputs of this process are:

- a modified (cbWidth)x(cbHeight) array predSamplesLXL of prediction luma sample values,

- two modified (cbWidth / 2)x(cbHeight / 2) arrays predSamplesLXCb and predSamplesLXCr of prediction chroma sample values.

The luma block width of neighboring reference samples cbLicWidth and the luma block height of neighboring reference samples reference cbLicHeight are derived as follows:

cbLicWidth = Min( cbWidth, 16 )

cbLicHeight = Min( cbHeight, 16 )

The derivation process for neighboring samples of current and reference coding unit specified in clause 8.5.7.5.2  is invoked with the luma location ( xCb, yCb ), the luma block width of neighboring reference samples cbLicWidth, the luma block height of neighboring reference samples cbLicHeight, the luma motion vector mvLX, the selected luma reference picture sample array refPicLXL, and the colour component index cIdx set equal to 0 as inputs, and the outputs are assigned to the sum of luma neighboring samples of a reference block sumRL, the sum of luma neighboring samples of the current block sumCL, the sum of multiplied luma neighboring samples of a reference block sumRRL, the sum of multiplied luma neighboring samples of the current block and a reference block sumRCL and the base-2 logarithm of the number of neighboring samples cntShiftL as outputs.

The derivation process for linear model for illumination changes specified in clause 8.5.7.5.3 is invoked with the sum of luma neighboring samples of a reference block sumRL, the sum of luma neighboring samples of the current block sumCL, the sum of multiplied luma neighboring samples of a reference block sumRRL, the sum of multiplied luma neighboring samples of the current block and a reference block sumRCL, the base-2 logarithm of the number of neighboring samples cntShiftL and the colour component index cIdx set equal to 0 as inputs, and the outputs are assigned to the luma scaling factor aL and the luma offset bL.

The luma prediction samples predSamplesLXL[ x ][ y ] with x = 0..cbWidth − 1, y = 0..cbHeight − 1 are modified as follows:

predSamplesLXL[ x ][ y ] = Clip1Y( ( ( aL * predSamplesLXL[ x ][ y ] ) >> 5 ) + bL )

The chroma sample location ( xCbC, yCbC ) specifying the top-left sample of the current chroma coding block relative to the top-left sample of the picture, the chroma block width cbWidthC, the chroma block height cbHeightC, the chroma block width of neighboring reference samples cbLicWidthC and the chroma block height of neighboring reference samples cbLicHeightC are derived as follows:

( xCbC, yCbC ) = ( xCb >> 1, yCb >> 1)

cbWidthC = cbWidth >> 1

cbHeightC = cbHeight >> 1

cbLicWidthC = Min( cbWidthC, 8 )

cbLicHeightC = Min( cbHeightC, 8 )

The derivation process for neighboring samples of current and reference coding unit specified in clause 8.5.7.5.2  is invoked with the chroma location ( xCbC, yCbC ), the chroma block width of neighboring reference samples cbLicWidthC, the chroma block height of neighboring reference samples cbLicHeightC, the chroma motion vector mvCLX, the selected chroma reference picture sample array refPicLXCb, and the colour component index cIdx set equal to 1 as inputs, and the outputs are assigned to the sum of chroma neighboring samples of a reference block sumRCb, the sum of chroma neighboring samples of the current block sumCCb, the sum of multiplied chroma neighboring samples of a reference block sumRRCb, the sum of multiplied chroma neighboring samples of the current block and a reference block sumRCCb and the base-2 logarithm of the number of neighboring samples cntShiftCb as outputs.

The derivation process for linear model for illumination changes specified in clause 8.5.7.5.3 is invoked with the sum of chroma neighboring samples of a reference block sumRCb, the sum of chroma neighboring samples of the current block sumCCb, the sum of multiplied chroma neighboring samples of a reference block sumRRCb, the sum of multiplied chroma neighboring samples of the current block and a reference block sumRCCb, the base-2 logarithm of the number of neighboring samples cntShiftCb and the colour component index cIdx set equal to 1 as inputs, and the outputs are assigned to the chroma scaling factor aCb and the chroma offset bCb.

The chroma prediction samples predSamplesLXCb[ x ][ y ] with x = 0..cbWidth / 2 − 1, y = 0..cbHeight / 2 − 1 are modified as follows:

predSamplesLXCb[ x ][ y ] = Clip1C( ( ( aCb * predSamplesLXCb[ x ][ y ] ) >> 5 ) + bCb )

The derivation process for neighboring samples of current and reference coding unit specified in clause 8.5.7.5.2  is invoked with the chroma location ( xCbC, yCbC ), the chroma block width of neighboring reference samples cbLicWidthC, the chroma block height of neighboring reference samples cbLicHeightC, the chroma motion vector mvCLX, the selected chroma reference picture sample array refPicLXCr, and the colour component index cIdx set equal to 2 as inputs, and the outputs are assigned to the sum of chroma neighboring samples of a reference block sumRCr, the sum of chroma neighboring samples of the current block sumCCr, the sum of multiplied chroma neighboring samples of a reference block sumRRCr, the sum of multiplied chroma neighboring samples of the current block and a reference block sumRCCr and the base-2 logarithm of the number of neighboring samples cntShiftCr as outputs.

The derivation process for linear model for illumination changes specified in clause 8.5.7.5.3 is invoked with the sum of chroma neighboring samples of a reference block sumRCr, the sum of chroma neighboring samples of the current block sumCCr, the sum of multiplied chroma neighboring samples of a reference block sumRRCr, the sum of multiplied chroma neighboring samples of the current block and a reference block sumRCCr, the base-2 logarithm of the number of neighboring samples cntShiftCr and the colour component index cIdx set equal to 2 as inputs, and the outputs are assigned to the chroma scaling factor aCr and the chroma offset bCr.

The chroma prediction samples predSamplesLXCr[ x ][ y ] with x = 0..cbWidth / 2 − 1, y = 0..cbHeight / 2 − 1 are modified as follows:

predSamplesLXCr[ x ][ y ] = Clip1C( ( ( aCr * predSamplesLXCr[ x ][ y ] ) >> 5 ) + bCr )

##### Derivation process for neighboring samples of current and reference coding unit

Inputs to this process are:

- A sample location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable cbLicWidth specifying the block width of neighboring reference samples,

- a variable cbLicHeight specifying the block height of neighboring reference samples,

- a motion vector mvLX,

- the selected reference picture sample array refPicLX,

- a variable cIdx specifying the colour component index.

Outputs of this process are:

- an average of neighboring samples of a reference block avgR,

- an average of neighboring samples of the current block avgC,

- a sum of mean absolute deviations of neighboring samples of a reference block sumAR,

- a sum of mean absolute deviations of neighboring samples of the current block sumAC,

- the base-2 logarithm of the number of neighboring samples cntShift.

The integer motion vector intMv is derived as follows:

intMv[ 0 ] = ( cIdx  = =  0 )  ?  ( ( mvLX[ 0 ] + ( 1 << 3 ) ) >> 4 )  :  ( ( mvLX[ 0 ] + ( 1 << 4 ) ) >> 5 )

intMv[ 1 ] = ( cIdx  = =  0 )  ?  ( ( mvLX[ 1 ] + ( 1 << 3 ) ) >> 4 )  :  ( ( mvLX[ 1 ] + ( 1 << 4 ) ) >> 5 )

The samples locations ( xInt[ x ], yInt[ y ] ) are derived as follows for x = −1.. cbLicWidth – 1 and y = − 1.. cbLicHeight − 1:

xInt[ x ] = Clip3( 0, pic_width_in_luma_samples − 1, xCb + intMv[ 0 ] + x )

yInt[ y ] = Clip3( 0, pic_height_in_luma_samples − 1, yCb + intMv[ 1 ] + y )

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC

The variables precShift, minDim and numSteps are derived as follows:

precShift = Max( 0, bitDepth - 12 )

minDim = Min( cbLicWidth, cbLicHeight )

numSteps = ( minDim > 8 )  ?  ( minDim >> 1 )  :  minDim

The current luma location ( xCbY, yCbY ) is derived as follows:

( xCbY, yCbY )  =  ( cIdx  = =  0 )  ?  ( xCb, yCb )  :  ( xCb << 1, yCb << 1 )

The variables availL and availT are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCbY, yCbY ) and the neighbouring luma location ( xCbY − 1, yCbY ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCbY, yCbY ) and the neighbouring luma location ( xCbY, yCbY − 1 ) as inputs, and the output is assigned to availT.

The variable currPic specifies the current picture.

The variables sumR, sumC, sumRR, sumRC and cntShift are derived as follows:

- If both availL and availT are equal to FALSE, cntShift is set equal to 0, sumR is set equal to 0, sumC is set equal to 0, sumRR is set equal to 0 and sumRC is set equal to 0.

- Otherwise, the following applies:

- The neighbouring samples C[ x ][ y ] are derived as follows:

    - When availL is equal to TRUE, the neighbouring left samples C[ x ][ y ] with x = −1, y = 0.. cbLicHeight − 1, are set equal to ( currPic[ xCb + x ][ yCb + y ]  >>  precShift ).

    - When availT is equal to TRUE, the neighbouring top samples C[ x ][ y ] with x = 0.. cbLicWidth − 1, y = −1, are set equal to ( currPic[ xCb + x ][ yCb + y ]  >>  precShift ).

- The neighboring samples of the reference block R[ x ][ y ] are derived as follows:

    - When availL is equal to TRUE, the neighbouring left samples of the reference block R[ x ][ y ] with x = −1, y = 0.. cbLicHeight − 1, are set equal to ( refPicLX[ xInt[ x ] ][ yInt[ y ] ]  >>  precShift ).

    - When availT is equal to TRUE, the neighbouring top samples of the reference block R[ x ][ y ] with x = 0.. cbLicWidth − 1, y = −1, are set equal to ( refPicLX[ xInt[ x ] ][ yInt[ y ] ]  >>  precShift ).

- The variables sL[ k ] and sT[ k ] are derived as follows for k = 0..numSteps – 1:

sL[ k ] = ( k * cbLicHeight ) >> Log2( numSteps )

sT[ k ] = ( k * cbLicWidth ) >> Log2( numSteps )

- The variables sumR, sumC, sumRR and sumRC are derived as follows:

sumR = ( availL  ?  k=0numSteps-1R -1 sL[k]   :  0 )  +  ( availT  ?  k=0numSteps-1R sT[k] -1  :  0 )

sumC = ( availL  ?  k=0numSteps-1C -1 sL[k]   :  0 )  +  ( availT  ?  k=0numSteps-1C sT[k] -1  :  0 )

- The variable cntShift is derived as follows:

cntShift = ( availL  &&  availT )  ?  ( Log2( numSteps ) + 1 )  :  Log2( numSteps )

- The variables shift, off, avgR and avgC are derived as follows:

precDC = ( cntShift >= 2 ) ? 2 : 0

shift = cntShift – precDC

off = shift ? (1 << (shift - 1)) : 0

avgR = ( sumR + off ) >> shift

avgC = ( sumC + off ) >> shift

- The variables sumAR and sumAC are derived as follows:

sumAR = ( availL  ?  k=0numSteps-1abs((R -1 sL[k] -precDC)-avgR)  :  0 )  +  ( availT  ?  k=0numSteps-1abs(R sTk -1≪preDC-avgR)  :  0 )

sumAC = ( availL  ?  k=0numSteps-1abs((C -1 sL[k] -precDC)-avgC)  :  0 )  +  ( availT  ?  k=0numSteps-1abs(C sTk -1≪preDC-avgC)  :  0 )

##### Derivation process for linear model for illumination changes

Inputs to this process are:

- an average of neighboring samples of a reference block avgR,

- an average of neighboring samples of the current block avgC,

- a sum of mean absolute deviations of neighboring samples of a reference block sumAR,

- a sum of mean absolute deviations of neighboring samples of the current block sumAC,

- the base-2 logarithm of the number of neighboring samples cntShift,

- a variable cIdx specifying the colour component index.

Outputs to this process are:

- a scaling factor a,

- an offset b.

The array LICDivTable[ k ] with k=0..63 is derived as follows:

LICDivTable[ 0 ] = 0

LICDivTable[ k ] = ( ( 1 << 15 ) + ( k >> 1 ) ) / k

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC

The variables precShift and cropShift are derived as follows:

precShift = Max( 0, bitDepth - 12 )

cropShift = Max( 0, bitDepth – precShift + cntShift - 15 )

The variables a and b are derived as follows:

- If cntShift is equal to 0, a is set equal to 32 and b is set equal to 0.

- Otherwise, the following applies:

- The variables a1 and a2 are derived as follows:

a1 = sumAC

a2 = sumAR

- The variables shifta1, shifta2 and shifta are derived as follows:

shifta1 = Max( 0, Ceil( Log2( Abs( a2 ) ) ) – 18 )

shifta2 = Max( 0, Ceil( Log2( Abs( a2 ) ) ) – 6 )

shifta = shifta2 + 10 – shifta1

- The variables a1 and a2 are modified as follows:

a1 = a1 >> shifta1

a2 = Clip3( 0, 63, a2 >> shifta2 )

- The variable a is derived as follows:

a = Clip3( 0, 128, ( a1 * LICDivTable[ a2 ] ) >> shifta )

- The variable b is derived as follows:

shift = 5 + precDC

b = Clip3( minb, maxb, ( (avgC<<precDC) – ( ( a * avgR ) >> 5 ) + ( 1 << ( shift – 1 ) ) >> shift )

- The variables a and b are modified as follows:

- if  b is less than (- 100) and a is greater than 40:

  - a = 40

  - b = Clip3( minb, maxb, ( (avgC<<precDC) – ( ( a * avgR ) >> 5 ) + ( 1 << ( shift – 1 ) ) >> shift )

- else if  b is greater than (100) and a is less than 24:

  - a = 24

  - b = Clip3( minb, maxb, ( (avgC<<precDC) – ( ( a * avgR ) >> 5 ) + ( 1 << ( shift – 1 ) ) >> shift )

- else a and b are not modified.