| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0309-v2 |
| --- | --- |

| Title: | Non-CE4: Switched half-pel interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anastasia Henkel, Benjamin Bross, Martin Winken, Paul Keydel, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49-30-31002-562 anastasia.henkel@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Version 2 of the document with the following changes:

# Updated results (software which was used for previous results contained a bug, low-delay Class B sequences are still not finished)

# Added new test 3 results (i.e., two alternative 6-tap interpolation filters)

# Abstract

This contribution proposes a switched luma interpolation filter for the half-pel position.  Results for three variants thereof are presented:  In a first test, one alternative 6-tap half-pel interpolation filter can be used.  In a second test, there are two alternative 8-tap half-pel interpolation filters.  In a third test, there are two alternative 6-tap interpolation filters.  The switching between the used luma half-pel interpolation filters is done at CU level.  In order to reduce the signaling overhead, the switching is dependent on the used motion vector accuracy.  For that purpose, the Adaptive Motion Vector Resolution (AMVR) scheme is extended to also support half-pel luma motion vector accuracy.  Only in case of this half-pel motion vector accuracy mode, an alternative half-pel interpolation filter can be used and it is indicated by an additional syntax element which interpolation filter is used.  In skip or merge mode with a spatial merging candidate, the value of this syntax element is also inherited from the neighbouring block.

For test 1 (i.e., one alternative 6-tap half-pel interpolation filter), the following results are reported relative to VTM-4.0:

RA: 	-0.30% (Y), -0.67% (Cb), -0.67% (Cr)	110% enc.time		99% dec.time

For test 2 (i.e., two alternative 8-tap half-pel interpolation filters), the following results are reported relative to VTM-4.0:

RA:	-0.36% (Y), -0.73% (Cb), -0.73% (Cr)	114% enc.time		98% dec.time

For test 3 (i.e., two alternative 6-tap half-pel interpolation filters), the following results are reported relative to VTM-4.0:

RA:	-0.36% (Y), -0.79% (Cb), -0.73% (Cr)	114% enc.time		98% dec.time

# Introduction

According to the current VVC draft [1] and VTM [2], motion vector differences of translational inter-predicted blocks can be encoded in 3 different accuracies (i.e., quarter-pel, full-pel, 4-pel).  The interpolation filters that are used for each fractional position are fixed, however.  In this contribution, it is proposed to allow usage of one or two alternative luma interpolation filters for the half-pel position.  The switching between the used luma interpolation filters can be done at CU level.  In order to reduce the signaling overhead, the switching is dependent on the used motion vector accuracy.  For that purpose, the Adaptive Motion Vector Resolution (AMVR) scheme is extended to also support half-pel luma motion vector accuracy.  Only in case of this half-pel motion vector accuracy mode, an alternative half-pel interpolation filter can be used and it is indicated by an additional syntax element which interpolation filter is used.  In skip or merge mode with a spatial merging candidate, the value of this syntax element is also inherited from the neighbouring block.

# Switched half-pel interpolation filter

The switching of the half-pel luma interpolation filter is done depending on the motion vector accuracy.  In addition to the existing quarter-pel, full-pel, and 4-pel AMVR modes, a new half-pel accuracy AMVR mode is introduced.  Only in case of half-pel motion vector accuracy, an alternative half-pel luma interpolation filter can be selected.

## Half-pel AMVR mode

An additional AMVR mode for non-affine non-merge inter-coded CUs is proposed which allows signaling of motion vector differences at half-pel accuracy.  The existing AMVR scheme of the current VVC draft [1] is extended straightforward in the following way: Directly following the syntax element amvr_flag, if amvr_flag == 1, there is a new context-modeled binary syntax element hpel_amvr_flag which indicates usage of the new half-pel AMVR mode if hpel_amvr_flag == 1. Otherwise, i.e. if hpel_amvr_flag == 0, the selection between full-pel and 4-pel AMVR mode is indicated by the syntax element amvr_precision_flag as in the current VVC draft.

## Alternative luma half-pel interpolation filters

For a non-affine non-merge inter-coded CU which uses half-pel motion vector accuracy (i.e., the half-pel AMVR mode), a switching between the HEVC/VVC half-pel luma interpolation filter and one or more alternative half-pel interpolation is made based on the value of a new syntax element if_idx.  The syntax element if_idx is only signaled in case of half-pel AMVR mode.  In case of skip/merge mode using a spatial merging candidate, the value of the syntax element if_idx is inherited from the neighbouring block.

### Test 1: One alternative 6-tap half-pel interpolation filter

In this test case, there is one 6-tap interpolation filter as an alternative to the ordinary HEVC/VVC half-pel interpolation filter.  The following table shows the mapping between the value of the syntax element if_idx and the selected half-pel luma interpolation filter:

| if_idx | Binarization | Filter | Interpolation filter coefficients |
| --- | --- | --- | --- |
| 0 | 0 | Gauss (6-tap) | [0, 3, 9, 20, 20, 9, 3, 0] |
| 1 | 1 | HEVC/VVC (8-tap) | [-1, 4, -11, 40, 40, -11, 4, -1] |

### Test 2: Two alternative 8-tap half-pel interpolation filters

In this test case, there are two 8-tap interpolation filters as an alternative to the ordinary HEVC/VVC half-pel interpolation filter. The following table shows the mapping between the value of the syntax element if_idx and the selected half-pel luma interpolation filter:

| if_idx | Binarization | Filter | Interpolation filter coefficients |
| --- | --- | --- | --- |
| 0 | 0 | Filter 1 (8-tap) | [3, 6, 10, 13, 13, 10, 6, 3] |
| 1 | 10 | Filter 2 (8-tap) | [-1, -1, 9, 25, 25, 9, -1, -1] |
| 2 | 11 | HEVC/VVC (8-tap) | [-1, 4, -11, 40, 40, -11, 4, -1] |

### Test 3: Two alternative 6-tap half-pel interpolation filters

In this test case, there are two 6-tap interpolation filters as an alternative to the ordinary HEVC/VVC half-pel interpolation filter. The following table shows the mapping between the value of the syntax element if_idx and the selected half-pel luma interpolation filter:

| if_idx | Binarization | Filter | Interpolation filter coefficients |
| --- | --- | --- | --- |
| 0 | 0 | Gauss | [0, 3, 9, 20, 20, 9, 3, 0] |
| 1 | 10 | Flat top | [0, -3, 4, 31, 31, 4, -3, 0] |
| 2 | 11 | HEVC/VVC (8-tap) | [-1, 4, -11, 40, 40, -11, 4, -1] |

# Simulation results

According to the common test conditions in JVET-M1010 [3], simulations were performed for random-access (RA) and low-delay B (LB) configuration with VTM-4.0 software.  The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

Note that for the RA configuration, the half-pel AMVR mode as well as the switched half-pel interpolation filter are disabled for the non-reference pictures (i.e., pictures having temporal_id == 4).

## Results for Test 1 (one alternative 6-tap half-pel luma interpolation filter)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.47% | -0.39% | 109% | 99% |  |
| Class A2 | -0.36% | -1.15% | -0.79% | 110% | 99% |  |
| Class B | -0.36% | -0.80% | -0.86% | 110% | 97% |  |
| Class C | -0.24% | -0.30% | -0.57% | 110% | 101% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.30% | -0.67% | -0.67% | 110% | 99% |  |
| Class D | -0.20% | -0.38% | -0.51% | 110% | 99% |  |
| Class F | -0.09% | -0.07% | -0.18% | 106% | 99% |  |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | #VALUE! | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |
| Class C |  | -0.20% | -0.20% |  | -0.27% |  | 116% |  | 98% |
| Class E |  | -0.17% | 0.66% |  | -0.65% |  | 115% |  | 101% |
| Overall |  | #VALUE! | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |
| Class D | -0.17% | 0.00% | -0.73% | 118% | 101% |
| Class F | -0.25% | -0.22% | 0.96% | 112% | 97% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.80% | -1.18% |  | -1.38% |  | 112% |  | 97% |  |
| Class C | -0.56% | -0.79% |  | -0.72% |  | 113% |  | 100% |  |
| Class E | -0.89% | -0.40% |  | -0.61% |  | 112% |  | 99% |  |
| Overall | -0.74% | -0.85% |  | -0.97% |  | 112% |  | 99% |  |
| Class D | -0.41% | -0.37% | -0.18% | 114% | 101% |
| Class F | -0.23% | 0.05% | -0.47% | 109% | 96% |

## Results for Test 2 (two alternative 8-tap half-pel luma interpolation filters)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.46% | -0.28% | 113% | 97% |  |
| Class A2 | -0.39% | -1.23% | -0.81% | 115% | 99% |  |
| Class B | -0.46% | -0.84% | -1.00% | 114% | 97% |  |
| Class C | -0.26% | -0.43% | -0.68% | 115% | 100% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.36% | -0.73% | -0.73% | 114% | 98% |  |
| Class D | -0.23% | -0.44% | -0.43% | 115% | 99% |  |
| Class F | -0.10% | -0.09% | -0.13% | 109% | 100% |  |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.22% | 0.06% | -0.27% | 123% | 98% |
| Class E | -0.28% | 0.15% | -0.25% | 117% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | 0.10% | -1.25% | 124% | 101% |
| Class F | -0.17% | 0.10% | -0.07% | 116% | 96% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.61% | -0.69% | -0.58% | 118% | 99% |
| Class E | -1.02% | -0.49% | -0.81% | 114% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.41% | -0.42% | 0.21% | 120% | 101% |
| Class F | -0.26% | -0.13% | 0.30% | 112% | 96% |

## Results for Test 3 (two alternative 6-tap half-pel luma interpolation filters)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.68% | -0.40% | 113% | 98% |
| Class A2 | -0.42% | -1.26% | -0.86% | 115% | 98% |
| Class B | -0.45% | -0.86% | -0.93% | 114% | 96% |
| Class C | -0.26% | -0.42% | -0.62% | 114% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.79% | -0.73% | 114% | 98% |
| Class D | -0.26% | -0.47% | -0.47% | 114% | 98% |
| Class F | -0.12% | -0.13% | -0.19% | 108% | 101% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.21% | -0.10% | -0.02% | 121% | 99% |
| Class E | -0.14% | 0.14% | -0.59% | 117% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.13% | 0.18% | -0.54% | 123% | 101% |
| Class F | -0.23% | -0.19% | -0.12% | 116% | 97% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.92% | -1.15% | -1.70% | 115% | 97% |
| Class C | -0.60% | -0.90% | -0.75% | 117% | 100% |
| Class E | -0.92% | -0.23% | -0.50% | 114% | 96% |
| Overall | -0.82% | -0.83% | -1.08% | 115% | 98% |
| Class D | -0.43% | -0.33% | -0.45% | 117% | 101% |
| Class F | -0.25% | -0.16% | 0.05% | 113% | 98% |

## Results for half-pel AMVR mode only (no additional interpolation filter)

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | 0.08% | 105% | 100% |
| Class A2 | -0.16% | -0.22% | -0.09% | 105% | 99% |
| Class B | -0.02% | -0.04% | -0.04% | 106% | 98% |
| Class C | -0.08% | -0.05% | -0.16% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.08% | -0.06% | 106% | 100% |
| Class D | -0.14% | -0.04% | -0.16% | 106% | 101% |
| Class F | -0.03% | -0.04% | -0.09% | 104% | 101% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.10% | 0.12% | -0.14% | 109% | 94% |
| Class E | -0.04% | 0.43% | 0.18% | 109% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | -0.18% | -0.42% | 111% | 101% |
| Class F | -0.18% | -0.45% | 0.08% | 108% | 97% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor_VTM_4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.38% | -0.46% | 107% | 98% |
| Class E | -0.03% | -0.48% | 0.09% | 107% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.19% | -0.10% | 108% | 101% |
| Class F | 0.03% | 0.12% | 0.07% | 107% | 98% |

# Conclusion

This contribution proposes a switched luma interpolation filter for the half-pel position.  Results for three variants thereof are presented:  In a first test, one alternative 6-tap half-pel interpolation filter can be used.  In a second test, there are two alternative 8-tap half-pel interpolation filters.  In a third test, there are two alternative 6-tap interpolation filters.  The switching between the used luma half-pel interpolation filters is done at CU level.  In order to reduce the signaling overhead, the switching is dependent on the used motion vector accuracy.  For that purpose, the Adaptive Motion Vector Resolution (AMVR) scheme is extended to also support half-pel luma motion vector accuracy.  Only in case of this half-pel motion vector accuracy mode, an alternative half-pel interpolation filter can be used and it is indicated by an additional syntax element which interpolation filter is used.  In skip or merge mode with a spatial merging candidate, the value of this syntax element is also inherited from the neighbouring block.

For test 1 (i.e., one alternative 6-tap half-pel interpolation filter), the following results are reported relative to VTM-4.0:

RA: 	-0.30% (Y), -0.67% (Cb), -0.67% (Cr)	110% enc.time		99% dec.time

For test 2 (i.e., two alternative 8-tap half-pel interpolation filters), the following results are reported relative to VTM-4.0:

RA:	-0.36% (Y), -0.73% (Cb), -0.73% (Cr)	114% enc.time		98% dec.time

For test 3 (i.e., two alternative 6-tap half-pel interpolation filters), the following results are reported relative to VTM-4.0:

RA:	-0.36% (Y), -0.79% (Cb), -0.73% (Cr)	114% enc.time		98% dec.time

- References

- B. Bross, J. Chen, S. Liu (editors), “Versatile Video Coding (Draft 4),“ Joint Video Experts Team, doc. JVET-M1001, Marrakech, MA, Jan. 2019.

- J. Chen, Y. Ye, S. Kim (editors), “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4),” Joint Video Experts Team, doc. JVET-M1002, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors), “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-M1010, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).