| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0310 |
| --- | --- |

| Title: | AHG14: On gradual decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar, Alireza Aminlou Nokia Technologies Hatanpaan valtatie 30 33100 Tampere Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes gradual decoding refresh based on the sub-picture-based picture partitioning of JVET-N0046.

# Introduction

The following inherent features of sub-pictures (as originally proposed in JVET-M0261) motivated us to consider how they could be applied for gradual decoding refresh:

- Loop filtering is inherently turned off over sub-picture boundaries. This is needed to keep refreshed region (sub-picture) independent of the remaining of the picture.

- Describing how motion vectors over sub-picture boundaries was easy to achieve as described originally in JVET-M0261 and as refined in JVET-N0046. This is needed for improving compression efficiency for refreshed regions.

However, the sub-picture design in JVET-M0261 or JVET-N0046 was found to be insufficient for gradual decoding refresh due to the following reasons:

- The sub-pictures were required to have the same size and location to enable inter prediction between sub-pictures. However, in gradual decoding refresh, the size and position of sub-pictures evolves.

- It should be able to use a complete decoded picture as a reference for predicting the "dirty" sub-picture to improve compression.

This contribution modifies the sub-picture design of JVET-N0046 to overcome the above-mentioned problems.

# Overview of the proposal

Figure 1 provides an example illustrating the proposal. Refresh sub-pictures are indicated with the diagonally striped rectangles, while the remaining picture area forms another sub-picture, which is casually called "dirty" sub-picture.

Figure 1. Example of the proposed sub-picture based gradual decoding refresh

The proposal is summarized as follows:

- Pictures from the random access point (inclusive) to the recovery point (exclusive) have two sub-pictures, a refresh sub-picture and a "dirty" sub-picture. The size of the refresh sub-picture increases from the random access point towards the recovery point.

- full_pic_ref_flag is included in the PPS. When equal to 0, the sub-picture with the index (in decoding order, within the referenced coded picture) that has the same index as the current sub-picture in the current picture is extracted and used as reference for inter prediction. When equal to 1, the entire reference picture is used as reference for inter prediction of the current sub-picture.

- full_pic_ref_flag is equal to 0 in the active PPS for the refresh sub-picture. Thus, only the previous refresh sub-pictures are used as reference for prediction of the current refresh sub-picture.

- full_pic_ref_flag is equal to 1 in the active PPS for the "dirty" sub-picture. Thus, a "dirty" sub-picture can use samples from both "dirty" sub-picture and refresh sub-pictures for deriving prediction blocks of inter prediction.

- A process for generating unavailable reference pictures like in HEVC is proposed. The process generates reference pictures for those active reference pictures that are not present in the DPB when CvsStartFlag is equal to 1.

- Sub-pictures that use unavailable reference pictures for prediction are marked incorrect in the decoder output. This is to be specified as part of HRD.

In addition to the proposals in this contribution, indicating a random access point for GDR should be enabled. Either a specific NAL unit type or an SEI message similar to the recovery point SEI message of HEVC could be used for this purpose.

# Proposed specification text

## Picture parameter set syntax and semantics

The following gray-highlighted changes are proposed while the yellow-highlighted changes are from JVET-N0046.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| multiple_subpics_enabled_flag | u(1) |
| if( multiple_subpics_enabled_flag ) { |  |
| subpic_width_in_ctus_minus1 | ue(v) |
| subpic_height_in_ctus_minus1 | ue(v) |
| full_pic_ref_flag | u(1) |
| } |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| … // no changes for the rest of the syntax |  |

full_pic_ref_flag equal to 0 specifies that when this PPS is active for the i-th sub-picture of the current picture, the i-th sub-picture of each active reference picture is extracted and used as reference in the inter prediction process and it is a requirement for bitstream conformance that there is an i-th sub-picture present in each active reference picture. full_pic_ref_flag equal to 1 specifies that the entire reference pictures is used as reference in the inter prediction process.

## Decoding process

The following gray-highlighted and red-strikethrough changes are proposed into clause 8.1.3 while the yellow-highlighted changes are from JVET-N0046.

When the current picture is an IRAP picture or the first picture of a CVS, the following applies:

–	If the current picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the current picture, the variable HandleCraAsCvsStartFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, the variable HandleCraAsCvsStartFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

…

When multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture is equal to 1, each CTU location in the decoded picture is marked as unoccupied.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the tile group header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first tile group of a picture.

–	At the beginning of the decoding process for each tile group of a non-IDR picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first tile group of a picture.

–	When NoRaslOutputFlag is equal to 1 and CurrPic is not an IDR picture, the decoding process for generating unavailable reference pictures specified in clause 8.3.X is invoked, which needs to be invoked only for the first tile group of a picture.

- If multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture is equal to 0, the following applies:

–	[Ed. (YK): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.]

Otherwise, the following applies for each sub-picture of CurrPic in decoding order:

–	The location of the sub-picture is the next such unoccupied location in CTU raster scan order within the decoded picture that is large enough to fit the sub-picture within the picture boundaries.

–	It is a requirement of bitstream conformance that the following constraints apply:

–	The value of multiple_subpics_enabled_flag in the PPS(s) that were active for decoding the picture corresponding to each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be equal to the value of multiple_subpics_enabled_flag in the PPS(s) that are active for the current picture.

–	Each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have a sub-picture at the same location and of the same size as the current sub-picture.

–	If full_pic_ref_flag is equal to 0, the following applies:

–	For decoding the i-th sub-picture of CurrPic, the decoding processes of [Ed. (MH): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.] are invoked with "picture" being replaced by "sub-picture" and the reference pictures corresponding to the active entries in RefPicList[ 0 ] or RefPicList[ 1 ] being generated by extracting the i-th sub-picture of the reference picturethe area collocating with the current sub-picture. The decoded sub-picture resulting from the decoding processes occupies the rectangular area wihin the decoded picture whose top-left corner is the location of the sub-picture as derived above and whose width and height in CTUs are equal to PicWidthInCtbsY and PicHeightInCtbsY, respectively, derived from the active PPS for the sub-picture.

–	Otherwise (full_pic_ref_flag is equal to 1), the following applies:

–	The decoding processes of [Ed. (MH): Add herein the invocation of the decoding processes for coding tree units, scaling, transform, in-loop filtering, etc.] are invoked. The decoded sub-picture resulting from the decoding processes occupies the rectangular area wihin the decoded picture whose top-left corner is the location of the sub-picture as derived above and whose width and height in CTUs are equal to PicWidthInCtbsY and PicHeightInCtbsY, respectively, derived from the active PPS for the sub-picture.

- After all tile groups of the current picture have been decoded, the current decoded picture is marked as "used for short-term reference".

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).