| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0311 |
| --- | --- |

| Title: | Context Modeling Simplification and Reduction in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen F. Le Léannec T. Poirier | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the context modelling and reduce the related contexts for CABAC in VVC. In this contribution, the CABAC context modelling for the regular coded bins sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, rem_abs_gt2_flag, last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, affine_flag, amvr_flag, triangle_flag and qt_split_cu_flag are investigated. With 24, 54, 66 and maximum 83 contexts reduction, the BD-Rate increase is reported as 0.03%, 0.04%, 0,06% and 0.11% for RA, respectively.

# Introduction

In VTM-4.0, there’re around 387 contexts used for regular coded syntax elements.

## Transform coefficients coding

CABAC context modelling for regular coded bins sig_coeff_flag, par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag is based on the sum of absolute values of partially reconstructed coefficients in a local neighborhood of current coefficient, as recalled here.

Figure 1: local neighborhood (x0,…,x4) of a current transform coefficient (X), as used in VTM-4.0 and division of luma and chroma transform blocks into region based on the diagonal (diag=x+y) index.

More precisely, the context for each regular coded bin is determined according to the following (JVET-K0072):

- significance flag (90 = 54 + 36 models): ctxIdSig = f( diag, sumAbs1, state )

- parity flag (32 = 21 + 11 models): ctxIdPar = f( diag, sumAbs1 − numSig, last )

- greater1 flag (32 = 21 + 11 models): ctxIdGt1 = ctxIdPar

- greater2 flag (32 = 21 + 11 models): ctxIdGt2 = ctxIdPar

where:

- diag is defined as the diagonal index of the coefficient at position (x,y): diag=x+y

- sumAbs  =  sum of absolute values in local neighborhood (template)

- sumAbs1 = sum of partially reconstructed absolute levels after first pass

- numSig  = number of transform coefficients with sig_coeff_flag == 1

The local neighborhood is used for both luma and chroma transform blocks. In particular, the chroma TBs are divided into 2 regions, respectively corresponding to coefficients for which diag=0 and diag>0.

In VTM-4.0, there are 186 contexts (90 for sig_coeff_flag and 96 for Chroma) for both The context model index for sig_coeff_flag, par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag in VVC is listed in Table 1 and 2.

Table 1. The context model index of sig_coeff_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d<2 | 12, 13,14, 15, 16, 17 |
| d<5 | 6, 7, 8, 9,10, 11 |
| d>=5 | 0, 1, 2, 3, 4, 5 |
| Chroma |  |
| Diagonal position | Context model index |
| d<2 | 6, 7, 8, 9, 10, 11 |
| d>=2 | 0, 1, 2, 3, 4, 5 |

Table 2. The context model index of par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 16, 17,18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |
| Chroma |  |
| Diagonal position | Context model index |
| d=0 | 6, 7, 8, 9, 10 |
| d>0 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

## Last significant coefficient coordinate coding

In VTM-4.0, the position of the last significant coefficient in a block is coded by explicitly signaling its (X, Y) coordinates. Coordinate X indicates the column number and Y the row number. The coordinates are binarized in two parts, a prefix and a suffix. The prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are both regular coded using truncated Rice binarization with cMax = (log2TbSize << 1) – 1. Here, if the syntax element to be parsed is last_sig_coeff_x_prefix, log2TbSize is set equal to log2TbWidth and otherwise, log2TbSize is set equal to log2TbHeight. That is, the maximum possible magnitude is determined by the transform block width or height. The maximum length of the number of regular coded bins for one coordinate is 3, 5, 7 and 9, for block sizes of 4, 8, 16 and 32 respectively.

Due to the limitation of the coefficient scans to non-zero out region of transform units with greater than 32 width or height. The scans within the non-zero out region are identical to corresponding non-zero width by non-zero height TUs and resulting last coefficient position’s coordinate coding binarization also follows that of corresponding non-zero width x non-zero height TU. Table 3 shows the binarization for W = 64 or H = 64 in VTM-4.0.

Table 3. Binarization for W = 64 or H = 64 in VTM-4.0

| Magnitude of last position component | last_sig_coeff_x_prefix or last_sig_coeff_y_prefix |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4-5 | 11110 |
| 6-7 | 111110 |
| 8-11 | 1111110 |
| 12-15 | 11111110 |
| 16-23 | 111111110 |
| 24-31 | 111111111 |
| 32-47 | N/A |
| 48-63 | N/A |

In VTM-4.0, there are 23 contexts (20 for Luma and 3 for Chroma) for both prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix. Table 4 shows the context assignment for different bins for a given coordinate across all block sizes T, Luma, and Chroma components.

Table 4. Context index for each truncated unary code bin and the block size T in VTM-4.0

| BinIndex | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | Luma |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 1 | 2 |  |  |  |  |  |  |  |  |
| 8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |  |  |
| 16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |  |  |
| 32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |  |  |
| 64 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | N/A | N/A |
| T | Chroma |  |  |  |  |  |  |  |  |  |  |
| 4 | 21 | 22 | 23 |  |  |  |  |  |  |  |  |
| 8 | 21 | 21 | 22 | 22 | 23 |  |  |  |  |  |  |
| 16 | 21 | 21 | 21 | 21 | 22 | 22 | 22 |  |  |  |  |
| 32 | 21 | 21 | 21 | 21 | 22 | 22 | 22 | 22 | 23 |  |  |

## affine_flag, amvr_flag, triangle_flag and qt_split_cu_flag coding

In VTM-4.0, the context derivation process of some syntax elements uses left and above neighboring syntax elements. For example, affine_flag, which is signaled to indicate whether the affine model based motion compensation is used to generate the prediction samples of the current CU or not, is regular coded with 3 context models and the context model is derived with the sum of the affine_flag of the left block L and above block A as depicted in Figure 2.

Figure 2: neighboring blocks used in context model derivation for affine_flag

And the CABAC context ctx derivation process is formulated as the follow:

ctx = (L is available && L is affine) + (A is available && A is affine)

CABAC context of other syntax elements amvr_flag (to signal AMVR mode), triangle_flag (to signal triangular prediction mode) and qt_split_cu_flag (to signal quadtree partitioning) are also derived in quite the similar way.

# Proposed Solution

## Transform coefficients coding

Method 1: local neighborhood removed in both chroma TB region (x+y>0 and x+y=0)

In this contribution, which was firstly proposed in JVET-M0107[1], the context modelling process for par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag is modified for complexity reduction. As shown in Table 5, the local neighborhood is replaced by a single CABAC context for each of these bins, in each chroma TB region. This leads to 24 contexts less than in VTM-4.0.

Table 5. The proposed context model index of par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 16, 17,18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |
| Chroma |  |
| Diagonal position | Context model index |
| d=0 | 2 |
| d>0 | 1 |
| Last non-zero coefficient | 0 |

Method 2:  share contexts among the transform coefficients

In this contribution, method 1 can also combined with another contribution, which was proposed in JVET-M0489[2]. It is proposed to share the context models among the transform coefficients.

More specifically, when coding sig_coeff_flag, as shown in Table 6, the context models that are labeled in yellow are combined, which can save 30 contexts more. When coding par_level_flag, rem_abs_gt1_flag or rem_abs_gt2_flag, for Luma component, as shown Table 7, the context models that are labeled in yellow are combined, which can save 12 contexts more. Therefore, it can save 54 or 66 context models in total for sig_coeff_flag, par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag.

Table 6. The proposed context model index of sig_coeff_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d<2 | 12, 13, 14, 15, 16, 17 |
| d<5 | 6, 7, 8, 9, 10, 11 |
| d>=5 | 0, 1, 2, 3, 4, 5 |
| Chroma |  |
| Diagonal position | Context model index |
| d<2 | 6, 7, 8, 9, 10, 11 |
| d>=2 | 0, 1, 2, 3, 4, 5 |

Table 7. The proposed context model index of par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 16, 17,18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |
| Chroma |  |
| Diagonal position | Context model index |
| d=0 | 2 |
| d>0 | 1 |
| Last non-zero coefficient | 0 |

## Last significant coefficient coordinate coding

Method 1: share contexts among the binarization of the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for block size 64

In this contribution, the context modelling of the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for block width (height) equal to 64 in binarization is reduced. One context is assigned to four bins in the bin string instead of two bins, which can save 4 contexts. Table 8 shows the context assignment of the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for block size equal to 64.

Table 8. The proposed context index for each truncated unary code bin of the block size 64

| BinIndex | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | Luma |  |  |  |  |  |  |  |  |  |  |
| 64 | 15 | 15 | 15 | 15 | 16 | 16 | 16 | 16 | 17 | N/A | N/A |

Method 2: share contexts among the binarization of the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for block size 4 and 8

In this contribution, the context modelling sharing among the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for the block width (height) equal to 4 and 8 in binarization is also proposed. Table 9 shows the context assignment of the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for block size equal to 4 and 8. In total, 6 contexts can be reduced.

Table 9. Context index for each truncated unary code bin and the block size T in VTM-4.0

| BinIndex | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | Luma |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 1 | 2 |  |  |  |  |  |  |  |  |
| 8 | 0 | 0 | 1 | 1 | 2 |  |  |  |  |  |  |
| 16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |  |  |
| 32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |  |  |
| 64 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | N/A | N/A |

## affine_flag, amvr_flag, triangle_flag and qt_split_cu_flag coding

Method 1: derive context model with the OR information of the left and above syntax element for affine_flag and qt_split_cu_flag

In this contribution, the context modelling derivation of affine_flag is modified as the follow:

ctx = (L is available && L is affine) ∥ (A is available && A is affine)

with 1 context reduction.

And the context modeling derivation of qt_split_cu_flag is modified as the follow:

ctx = (L is available && cqDepth_L > cqDepth) ∥ (A is available && cqDepth_A > cqDepth)

+ (cqDepth < 2 ? 0: 2)

with 2 contexts reduction.

Method 2: derive context model without using the left and above syntax element for amvr_flag and triangle_flag

In this contribution, it is also proposed to use only one context model for coding amvr_flag and triangle_flag to avoid using the left and above syntax elements. This modification can reduce the line buffer size as well as the parsing complexity, since the syntax elements of the neighbouring blocks is not used in the context derivation process. And in total, 4 contexts can be reduced for amvr_flag and triangle_flag.

# Simulation results

The proposed method is implemented on the top of VTM-4.0. The common test conditions defined in [3] is used in the simulation. The tables below show the BD-Rate of the proposed methods of transform coefficients coding relative to VTM-4.0. Please note that the timing is not reliable.

Table 10 gives the performances of the method 1 of transform coefficients coding compared to the VTM-4.0 anchor.

Table 10: method 1 of transform coefficient performances over VTM-4.0.

Next table provides the results of method 1 over VTM-4.0, when combined only the context reduction of sig_coeff_flag in method 2 of transform coefficients coding.

Table 11: method 1 and sig_coeff_flag of method 2 of transform coefficient performances over VTM-4.0.

Table 12 provides the results of method 1 over VTM-4.0, when combined method 2 of transform coefficients coding.

Table 12: method 1 and method 2 of transform coefficient performances over VTM-4.0.

Table 13 provides all the context simplification and reduction proposals in this contribution over VTM-4.0.

Table 13: proposed methods in this contribution performances over VTM-4.0.

# Conclusions

This contribution proposes some ways to reduce the complexity of the regular coded bins entropy coding and the number of CABAC contexts in the same time. This is achieved through the removal of the contexts for the sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, rem_abs_gt2_flag, last_sig_coeff_x_prefix, last_sig_coeff_y_prefix, affine_flag, amvr_flag, triangle_flag and qt_split_cu_flag. The results of different combinations of the proposed methods in this contribution are given by the following table.

As can be seen, a significant reduction of the context used in VTM-4.0 can be achieved without major penalty in terms of coding efficiency.

Table 14: summary of the performances and CABAC context reduction of the proposed methods

| Proposed method | Nb contexts/VTM-4.0 | RA (Y/Cb/Cr) |
| --- | --- | --- |
| method 1 of transform coefficients coding | -24 | 0.03% 0.11% 0.17% |
| method 1 and sig_coeff_flag of method 2 of transform coefficient | -54 | 0.04% 0.22% 0.09% |
| method 1 and method 2 of transform coefficient | -66 | 0.06% 0.16% 0.15% |
| all proposed methods in this contribution | -83 | 0.11% 0.21% 0.07% |

# Reference

[1] F. Le Léannec, Y. Chen, JVET-M0107, “CE7-related: reduced local neighborhood usage for transform coefficients coding”, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[2] M. Gao, X. Li, X. Zhao and S. Liu, JVET-M0489, “CE7-related: Reduced context models for transform coefficients coding”, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[3] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).