| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0312-v1 |
| --- | --- |

| Title: | CE9-related: A SIMD-friendly simplification for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

Optical flow estimation in BDOF for a 4x4 block of luma samples is performed using the sample values and sample gradients over a 6x6 block of predicted sample positions. The boundary handling for sample gradients and sample values at CU boundaries are not very SIMD-friendly. In this proposal, it is proposed to use a 4x6 block of predicted sample positions for computing the optical flow. The impact of not having extended samples to the left and right of the central 4x4 block on the BDRATE are presented. The results indicate that there is a 0.03% impact on BDRATE in RA. The proposal improves the SIMD-friendliness and improves the worst-case complexity of BDOF. It is requested that this simplification be considered for further study in a core experiment.

# Introduction

BDOF requires computation of horizontal and vertical sample gradients at 6x6 predicted sample positions for obtaining the optical flow estimate and the central 4x4 sample gradients are used for applying a correction to conventional bi-prediction using the computed optical flow and the sample gradients. The computation of sample gradients within the 6x6 sample positions requires sample values at 8x8 sample positions. The sample gradients for the positions falling on the boundary of a coding unit require sample values outside the coding unit and integer grid sample values closest to the desired sample position are used in this case. For positions outside the CU, the closest boundary sample gradients and sample values are used. This process requires column-wise access for sample positions to the left or right of a coding unit. These operations need to happen element by element are hence not SIMD-friendly. The accumulation of the product terms involving the gradient sums and the sample differences happens on a 6x6 block and these accumulations also are not SIMD-friendly.

In this proposal, the optical flow is computed for a 4x4 block of luma samples using the sample values and sample gradients over a 4x6 block of predicted sample positions. This makes the method SIMD-friendly for the most part, except for the computation of sample gradients of boundary sample positions using integer grid samples.

# Proposed Method

The primary change proposed to make BDOF more SIMD-friendly is to reduce the number of predicted sample positions that are used for solving the over-constrained set of equations to obtain the optical flow estimate for a given 4x4 block within a coding unit.

Specifically, the 6x6 block of sample positions used for solving the equations is replaced by a 4x6 block of predicted sample positions as shown in Fig.1. The sample gradient computation over the 4x6 sample positions still requires accessing sample positions outside the coding unit. Based on the method adopted from CE9.1.1 at the last meeting, the predicted sample values for fractional sample positions falling outside the dimensions of the coding unit are taken as the reference sample value on the integer grid that is closest to the fractional sample position.

Figure 1: Proposed 4x6 set of predicted sample positions for optical flow computation

Central 4x4 set of predicted sample positions

Making the block size as 4x6 eliminates the need for obtaining any sample values or sample gradient value at positions that fall to the left or right of the central 4x4 block of sample positions. Obtaining the sample values and sample gradient values at positions that fall above or below the central 4x4 block of sample SIMD-friendly.

With the change to 4x6 block of sample positions, the accumulation of product terms becomes SIMD-friendly. It becomes possible to implement the SIMD lane-wise accumulation for multiple sub-blocks using a common SIMD load. The proposed method also allows 2 line buffers to be remembered from a previous row of 4x4 sub-blocks and re-used for the next row of 4x4 sub-blocks. Also, the proposed change reduces the worst-case complexity of BDOF for the case of all 8x8 coding units by ~20% as the optical flow equations of only 80 sample positions are used instead of 100 sample positions earlier while solving for the optical flow estimate.

# Results

Table 3.1 presents the results over VTM4.0 in RA for the proposed method that uses 4x6 block of sample positions for computing the flow. The proposed simplification results in a 0.03% drop in average luma BDRATE with small gains in average chroma BDRATEs. There is a small reduction in the average decoding time. More optimal SIMD implementations can further reduce the average decoding time.

Table 3.1 Results with 4x6 block of sample positions based optical flow estimation

# Conclusions

A SIMD-friendly simplification for BDOF is proposed wherein a 4x6 block of predicted sample positions are used for computing the optical flow instead of the the current 6x6 block of positions. The simplifications reduces the worst-case operation count for BDOF by 20%. It also reduces a significant number of element by element column accesses for CU boundary handling. There is a small impact (~0.03%) on average luma BDRATE in RA. It is requested that the proposed method be considered for further study in core experiments.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).