| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0313-v2 |
| --- | --- |

| Title: | CE3-related: Low latency intra sub-partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rahul Vanam, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4800 rahul.vanam@interdigital.com yuwen.he@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Intra sub-partitions (ISP) predicts successive sub-partitions using reconstructed samples from its previous adjoining sub-partitions. In this proposal, a low latency ISP scheme is presented that predicts a sub-partition using the prediction samples from its previous adjoining sub-partition.  Applying this scheme to 8×4 and 4×8 CUs in the VTM-4.0 anchor yields a small luma BD-rate loss of 0.01% for AI configuration and no loss for RA configuration, with no impact on both the encoding and decoding time.  Applying this scheme to all CU sizes permitted by ISP yields an average luma BD-rate loss of 0.15% and 0.08% for AI and RA, respectively, without impact on the encoding and decoding time.

# Introduction

The intra sub-partitions (ISP) is an intra coding tool that was adopted in VVC during the 13th JVET meeting.  ISP splits a luma intra-predicted block vertically or horizontally into two or four sub-partitions depending upon the block size.  Block size of 4×8 and 8×4 are always split into two sub-partitions, while larger blocks are split into four sub-partitions.  The residual is added to the prediction samples of the first sub-partition to yield reconstructed samples, which are then used to predict the next sub-partition. The process of prediction and reconstruction is repeated on successive sub-partitions. Figure 1(a) illustrates the prediction of a sub-partition ‘B’ using bottom row samples from reconstructed sub-partition ‘A’.  The associated pipeline stages are illustrated in Figure 2(a).  Since a sub-partition cannot be predicted until its previous sub-partition has been reconstructed, it introduces latency for processing successive sub-partitions.  This proposal presents a scheme to reduce this latency in ISP.

Figure 1. (a) Sub-partition prediction in ISP. (b) Sub-partition prediction in low latency ISP.

# Proposed Low Latency ISP

In this proposal, each successive sub-partition is predicted using prediction samples from a previous sub-partition. For example, in Figure 1(b), sub-partition A is predicted as in ISP, and the bottom row prediction samples of A are used to predict sub-partition B.  Since the prediction of sub-partition B does not depend upon reconstruction of A, the proposed approach reduces latency introduced by ISP.  Pipeline stages for both ISP and our proposed approach are illustrated in Figures 2(a) and (b), respectively, and they illustrate the latency reduced in our proposed approach.

Figure 2. (a) Pipeline in ISP. (b) Pipeline in low latency ISP.

# Experimental results

VTM-4.0 anchor is used as the software basis in our experiments, and we use test sequences, configurations, and test conditions specified by JVET-M1010 [1]. Following two tests are considered:

- : Low latency ISP applied only to 8×4 and 4×8 CUs, and regular ISP applied to larger CUs.

- : Low latency ISP applied to all CU sizes permitted by ISP.

Tests are conducted for both the AI and RA configurations as specified by [2].  AI and RA results for Test (1) are provided in Table 1 and Table 2, respectively, and results for Test (2) are provided in Table 3 and Table 4.  Detailed results are provided in the accompanying Excel sheets.

Table 1. AI results for Test (1) where Low latency ISP is applied only to 8×4 and 4×8 CUs.

|  | All Intra Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 100% | 95% |
| Class A2 | 0.00% | 0.10% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.05% | 100% | 101% |
| Class C | 0.04% | -0.15% | -0.05% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.01% | -0.02% | 0.00% | 100% | 99% |
| Class D | 0.02% | 0.02% | -0.06% | 100% | 100% |
| Class F | 0.01% | 0.03% | -0.18% | 100% | 101% |

Table 2. RA results for Test (1) where Low latency ISP is applied only to 8×4 and 4×8 CUs.

|  | Random access Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.15% | -0.04% | 101% | 103% |
| Class A2 | -0.03% | 0.06% | -0.02% | 102% | 96% |
| Class B | 0.01% | -0.06% | -0.08% | 101% | 101% |
| Class C | 0.03% | 0.02% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | -0.04% | 101% | 100% |
| Class D | -0.03% | -0.18% | -0.21% | 101% | 100% |
| Class F | 0.02% | -0.16% | -0.20% | 100% | 99% |

Table 3. AI results for Test (2) where Low latency ISP is applied to all CU sizes permitted by ISP.

|  | All Intra Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.09% | 0.08% | 99% | 96% |
| Class A2 | 0.10% | 0.23% | 0.11% | 100% | 100% |
| Class B | 0.13% | 0.15% | 0.16% | 100% | 99% |
| Class C | 0.22% | 0.21% | 0.24% | 97% | 99% |
| Class E | 0.19% | 0.29% | 0.13% | 98% | 98% |
| Overall | 0.15% | 0.19% | 0.15% | 99% | 98% |
| Class D | 0.12% | 0.25% | 0.08% | 98% | 101% |
| Class F | 0.17% | 0.17% | 0.12% | 100% | 97% |

Table 4. RA results for Test (2) where Low latency ISP is applied to all CU sizes permitted by ISP.

|  | Random access Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.01% | 0.11% | 100% | 100% |
| Class A2 | 0.06% | 0.26% | 0.11% | 101% | 98% |
| Class B | 0.09% | 0.18% | 0.03% | 101% | 101% |
| Class C | 0.11% | 0.15% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.15% | 0.07% | 100% | 100% |
| Class D | 0.06% | -0.05% | -0.12% | 101% | 101% |
| Class F | 0.05% | 0.07% | -0.06% | 100% | 98% |

# Conclusion

This contribution proposes a low latency ISP scheme where a sub-partition is predicted using the prediction samples from a previous sub-partition.  Since the sub-partition prediction no longer depends on the reconstructed samples from its previous sub-partition, the proposed approach reduces latency introduced by ISP.  Applying this scheme to 8×4 and 4×8 CUs yields a small luma BD-rate loss of 0.01% for AI, and no loss for RA, with no impact on the encoding and decoding time.  Applying this scheme to all CU sizes permitted by ISP yields a luma BD-rate loss of 0.15% and 0.08%, for AI and RA, respectively, with no impact on the encoding and decoding time.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Jan. 2019, Marrakech, MA.

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, 13th meeting, Jan. 2019, Marrakech, MA.

# Additional Results

In this section, another low latency ISP scheme is presented called the CU-based low latency ISP.  The above reference row and left reference column adjoining a CU are first used to predict samples adjoining each sub-partition.  For example, in Figure 3, the bottom row of sub-partitions A, B, and C are predicted using the reference samples adjoining the CU.  Each successive sub-partition is then predicted using the adjoining predicted samples, and CU reference samples.  For example, in Figure 3, bottom row predicted samples from A are used for predicting B.

The same two tests listed in Section 3 are performed for the CU-based low latency ISP for both AI and RA configurations, and the results are summarized in Tables 5, 6, 7, and 8.  Applying the CU-based low latency ISP to 8×4 and 4×8 CUs, and regular ISP applied to larger CUs, yields small luma average BD-rate loss of 0.01% for AI, and no loss for RA.  Applying the CU-based low latency ISP to all CU sizes permitted by ISP yields average luma BD-rate loss of 0.18% and 0.09% for AI and RA, respectively.  For both tests, there is no impact on average encoding and decoding time.

Figure 3. CU-based low latency ISP.

Table 5. AI results for Test (1) where CU-based low latency ISP is applied only to 8×4 and 4×8 CUs.

|  | All Intra Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | -0.01% | 100% | 94% |
| Class A2 | 0.00% | 0.09% | -0.01% | 101% | 96% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class C | 0.05% | 0.08% | 0.15% | 100% | 100% |
| Class E | 0.01% | 0.00% | -0.03% | 100% | 98% |
| Overall | 0.01% | 0.05% | 0.03% | 100% | 98% |
| Class D | 0.02% | 0.07% | 0.01% | 100% | 99% |
| Class F | 0.05% | 0.04% | -0.01% | 99% | 99% |

Table 6. RA results for Test (1) where CU-based low latency ISP is applied only to 8×4 and 4×8 CUs.

|  | Random access Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.26% | -0.05% | 100% | 95% |
| Class A2 | -0.01% | 0.05% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.02% | -0.02% | 101% | 99% |
| Class C | 0.01% | -0.22% | -0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.08% | 100% | 98% |
| Class D | 0.02% | -0.01% | -0.05% | 99% | 100% |
| Class F | 0.01% | -0.19% | -0.29% | 100% | 100% |

Table 7. AI results for Test (2) where CU-based low latency ISP is applied to all CU sizes permitted by ISP.

|  | All Intra Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.05% | 0.08% | 100% | 92% |
| Class A2 | 0.12% | 0.22% | 0.11% | 101% | 96% |
| Class B | 0.15% | 0.16% | 0.21% | 100% | 99% |
| Class C | 0.26% | 0.23% | 0.30% | 99% | 99% |
| Class E | 0.26% | 0.26% | 0.18% | 100% | 98% |
| Overall | 0.18% | 0.18% | 0.19% | 100% | 97% |
| Class D | 0.15% | 0.21% | 0.10% | 99% | 99% |
| Class F | 0.21% | 0.19% | 0.03% | 100% | 99% |

Table 8. RA results for Test (2) where CU-based low latency ISP is applied to all CU sizes permitted by ISP.

|  | Random access Main10 (Over VTM-4.0) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.24% | 0.06% | 100% | 96% |
| Class A2 | 0.07% | 0.21% | 0.04% | 100% | 98% |
| Class B | 0.09% | 0.11% | 0.02% | 100% | 98% |
| Class C | 0.11% | 0.16% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.17% | 0.07% | 100% | 98% |
| Class D | 0.04% | 0.32% | -0.37% | 100% | 100% |
| Class F | 0.06% | 0.10% | 0.02% | 100% | 99% |

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).