| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1001-v6 |
| --- | --- |

| Title: | Versatile Video Coding (Draft 4) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text of video coding specification |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Shan Liu | Email: | firstname.lastname@hhi.fraunhofer.de jianle.chen@huawei.com shanl@tencent.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

Since this is an early draft, topics outside of the specific aspects that have been established by recorded meeting agreements are not included in the specification. Such aspects are to be determined by further development of the VVC project in JVET. The high-level syntax for the standard is yet to be developed. The aspects of high-level syntax in this early draft are provided only to show how certain features are likely to be controlled by some high-level syntax that may have a sequence level, a picture level, and a tile group level (a picture spatial region level that includes a subset of the CTUs of the picture).

Draft 1 of Versatile Video Coding.

### Decoding process for intra blocks

#### General decoding process for intra blocks

Inputs to this process are:

- a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable predModeIntra specifying the intra prediction mode,

- a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before in-loop filtering.

The maximum transform block size maxTbSize is derived as follows:

maxTbSize = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / 2	(8-95)

The luma sample location is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * 2, yTb0 * 2 )	(8-96)

Depending on maxTbSize, the following applies:

- If IntraSubPartSplitType is equal to NO_ISP_SPLIT and nTbW is greater than maxTbSize or nTbH is greater than maxTbSize, the following ordered steps apply.

- The variables newTbW and newTbH are derived as follows:

newTbW = ( nTbW  >  maxTbSize ) ? ( nTbW / 2 ) : nTbW	(8-97)

newTbH = ( nTbH   >  maxTbSize ) ? ( nTbH / 2 ) :  nTbH	(8-98)

- The general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbW is greater than maxTbSize, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbH is greater than maxTbSize, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbW is greater than maxTbSize and nTbH is greater than maxTbSize, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- Otherwise, the following ordered steps apply:

- The variables nW, nH, numPartsX and numPartsY are derived as follows:

nW = IntraSubPartitionsSplitType = = ISP_VER_SPLIT  ?  nTbW / NumIntraSubPartitions  :  nTbW	(8-99)

nH = IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  ?  nTbH / NumIntraSubPartitions  :  nTbH	(8-100)

numPartsX = IntraSubPartitionsSplitType = = ISP_VER_SPLIT  ?  NumIntraSubPartitions  :  1	(8-101)

numPartsY = IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  ?  NumIntraSubPartitions  :  1	(8-102)

- For xPartIdx = 0..numPartsX − 1 and yPartIdx = 0..numPartsY − 1, the following applies:

- Variable flag ispLLFlag is derived as follows

ispLLFlag = (NumIntraSubPartitions  = = 2) and (xPartIdx > 0 or yPartIdx > 0) and (cIdx = = 0) ? 1 : 0  (8-103)

- The general intra sample prediction process as specified in clause 8.4.4.2.1 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH set equal to nW and nH, the coding block width nCbW and height nCbH set equal to nTbW and nTbH, the low latency ISP flag ispLLFlag, and the variable cIdx as inputs, and the output is an (nTbW)x(nTbH) array predSamples.

- The scaling and transformation process as specified in clause 8.7.2 is invoked with the luma location ( xTbY, yTbY ) set equal to ( xTbY + nW * xPartIdx, yTbY + nH * yPartIdx ), the variable cIdx, the transform width nTbW and the transform height nTbH set equal to nW and nH as inputs, and the output is an (nTbW)x(nTbH) array resSamples.

- The picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the transform block location ( xTbComp, yTbComp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the transform block width nTbW, the transform block height nTbH set equal to nW and nH, the variable cIdx, the (nTbW)x(nTbH) array predSamples, and the (nTbW)x(nTbH) array resSamples as inputs, and the output is a modified reconstructed picture before in-loop filtering.

#### Intra sample prediction

##### General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable ispLLFlag specifying the use of low latency ISP approach

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

Some of the predicted samples are stored in a list predArray[x], with x = 0...Max(nTbW − 1, nTbH − 1)

The variables refW and refH are derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

refW = nTbW * 2		(8-103)

refH = nTbH * 2		(8-104)

- Otherwise ( IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

refW = nCbW * 2		(8-105)

refH = nCbH * 2		(8-106)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 )  ?  IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ]  :  0	(8-107)

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.4.4.2.2 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the low latency ISP flag ispLLFLag, the list containing predicted samples predArray, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.4.2.3 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.4.4.2.4 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.4.4.2.5 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.4.4.2.6 is invoked with the transform block width nTbW, the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.4.4.2.8 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.4.4.2.7 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the coding block width nCbW and height nCbH, the colour component index cIdx, and the reference sample array p as inputs, and the modified intra prediction mode predModeIntra and the predicted sample array predSamples as outputs.

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

if IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  and cIdx = = 0, assign predSamples[x][nTbH-1] to predArray[x], with x = 0..nTbW-1

Otherwise, if IntraSubPartitionsSplitType = = ISP_VER_SPLIT  and cIdx = = 0, assign predSamples[nTbW-1][y] to predArray[y], with y = 0..nTbH-1

##### Reference sample availability marking process

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable refIdx specifying the intra prediction reference line index,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable ispLLFlag specifying the low latency ISP flag,

- a list predArray containing predicted samples,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx for intra sample prediction.

The refW + refH + 1 + ( 2 * refIdx ) neighbouring samples refUnfilt[ x ][ y ] that are constructed samples prior to the in-loop filter process, with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, are derived as follows:

- The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(8-108)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp << 1, yTbCmp << 1 )	(8-109)

( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp << 1, yNbCmp << 1 )	(8-110)

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

- Each sample refUnfilt[ x ][ y ] is derived as follows:

- If availableN is equal to FALSE, the sample refUnfilt[ x ][ y ] is marked as "not available for intra prediction".

- Otherwise, if ispLLFlag = = 1 and (x != −1 – refIdx or y != −1 – refIdx), the sample refUnfilt[ x ][ y ] is marked as "available for intra prediction", and the following applies:

  - If IntraSubPartitionsSplitType = = ISP_HOR_SPLIT and y = = −1 – refIdx, assign predArray[x] to refUnfilt[ x ][ y ]

  - Otherwise, if IntraSubPartitionsSplitType = = ISP_VER_SPLIT and x = = −1 – refIdx, assign predArray[y] to refUnfilt[ x ][ y ]

- Otherwise (ispLLFlag = = 0 or (x = = −1 – refIdx and y = = −1 – refIdx) ), the sample refUnfilt[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to refUnfilt[ x ][ y ].

