| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0314-v2 |
| --- | --- |

| Title: | CE9-related: Results for use of DMVR refined MVs from top and top-left CTUs for spatial MV prediction and CU boundary de-blocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

Use of refined MVs in a CTU-level pipeline friendly manner was first proposed in JVET-K0041 and was studied in multiple core experiments including CE9-2.1 in JVET-M0147. Due to concerns around hardware pipeline stage re-design for the spatial MV prediction stage and the boundary strength calculation stage for de-blocking, use of refined MVs only for temporal prediction was adopted at the last meeting. In this contribution, in Test-1, the possible BDRATE gains from using refined MVs of top and top-left CTUs in spatial MV prediction are presented. The results indicate that 0.11% BDRATE gain is possible in RA over VTM4.0. Also, the results for a Test-2 that uses refined MVs at coding unit boundaries for computing boundary strengths for de-blocking on top of Test-1 are also presented. With this method, it becomes possible to update the refined MVs immediately without waiting for de-blocking boundary strength computation to complete. It is requested that the benefits offered by the use of refined MVs be further studied in core experiments.

# Introduction

Dependency on the refined motion vectors for reconstructing motion vectors to perform reference sample pre-fetch was posing a significant issue for DMVR. A method for using refined MVs of neighbor sub-blocks that fall within top and top-left CTUs was first proposed in JVET-K0041 to address this dependency issue. Following that, this method was studied in core experiments for 2 cycles. At the last JVET meeting, the aspect of using refined MVs in this manner for spatial MV prediction and as centers of refinement for a current CU was not adopted. The specific reasons for this were that in certain current designs, the motion vector prediction and reconstruction happens in a pipeline stage that precedes any pixel processing pipeline stage. Similarly, it was possible to compute the boundary strengths for de-blocking as soon as the bitstream is parsed and the motion vectors are reconstructed. Hence, in some designs, the boundary strengths were also computed in pipeline stages that precede any pixel processing pipeline stage. The use of refined MVs for spatial MV prediction offers some compression gain. In this contribution, the results for using refined MVs of top and top-left CTU neighbors for spatial MV prediction and as centers of refinement for a current CU are presented for RA against VTM4.0 as the reference. As described in core experiment summaries earlier, this method allows refined MV line buffer to be used across CTU boundaries with only a requirement for storing a single unrefined MV per CTU corresponding to the top-right CTU neighbor access.

Also, the adopted DMVR method requires the unrefined MVs and the delta motion vectors to be maintained till deblocking boundary strengths have been computed. If deblocking can use the refined MVs to obtain boundary strengths at the coding unit boundary aligned edges, the need for maintaining the unrefined MV will be localized to the MV prediction and reconstruction stage. The results for such a change are presented on top of the use of refined MVs for spatial MV prediction.

# Results

Table 3.1 summarizes the results for Test-1 that uses the refined MVs of top and top-left CTU neighbors for spatial MV prediction and as centers of refinement for current coding units. It can be seen that there is an average luma BDRATE gain of 0.11% in RA with similar chroma BDRATE gains. The gain increases with increasing resolution and it can be seen that class A2 shows a luma average BDRATE gain of 0.26%. The average decoding time increases by 2% due to a higher % of coding units applying DMVR indicating many close mode decisions.

Table 3.1 Results for Test-1

The preliminary results for using the refined MVs for computing boundary strength on sub-block edges aligned with coding unit boundaries (but not the internal sub-block edges) indicate that there is a small average BDRATE gain of 0.01%. Table 3.2 shows the result for Test-2 that performs de-blocking boundary strength calculations for CU boundaries using refined MVs in addition to Test-1.

Table 3.2: Results for Test-2

# Conclusions

This contribution presents the results for using refined MVs in a reference sample pre-fetch friendly pipeline manner for spatial MV prediction and as centers of refinement for current CUs. The internal memory benefits are summarized again. The results indicate an average BDRATE gain of 0.11% with class A2 reaching as high as 0.26% against VTM4.0. In addition, use of refined MVs to derive boundary strength for sub-block edges aligned with CU boundaries is also proposed. This method also offers a small BDRATE gain. Given these advantages, it is requested that the specific drawbacks, if any, around re-design of hardware pipeline stages for MV prediction/reconstruction and boundary strength determination to be dependent on the refined MV be studied further.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).