| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0316 |
| --- | --- |

| Title: | CE8-related: Default Processing for IBC Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a default processing method for IBC mode in order to prevent un-defined decoder behavior. There are 3 encoder constraints in VTM4.0 which are heavily relied on the quality of the encoder. First, the IBC merge list may not be fully filled. Therefore, encoder can only signal merge index which does not exceed the valid size of IBC merge list. Second, the encoder can only signal valid block vector whether it is skip/merge mode or AMVP mode. Third, the encoder can only signal chroma IBC mode when collocated luma blocks are all coded with IBC mode and the block vectors are all valid. In this contribution, a default processing method for IBC mode is designed which uses a default value as the prediction samples to prevent these un-defined decoder behaviors. The simulation result of “Test 1: Apply default processing method to IBC merge/skip mode” shows 0.00% gain in AI case, 0.03% gain in RA case and 0.04% gain in LB case in screen content coding materials. “Test 2: Test 1 + Apply default processing method to IBC chroma mode” shows 0.02% gain in AI case, 0.04% gain in RA case and 0.06% loss in LB case. “Test 3: Test 1 + Apply default processing method to CU based IBC chroma mode” shows 0.18% gain in AI case, 0.02% loss in RA case and 0.04% loss in LB case.

# Introduction

In the 13th JVET meeting, IBC mode as an independent mode was adopted into VTM4.0. In the current design of IBC mode, there are 3 encoder constraints as below:

- The IBC merge list may not be fully filled. Therefore, encoder can only signal merge index which does not exceed the valid size of IBC merge list.

- The encoder can only signal valid block vector whether it is skip/merge mode or AMVP mode.

- The encoder can only signal chroma IBC mode when collocated luma blocks are all coded with IBC mode and the block vectors are all valid.

These encoder constraints are heavily relied on the quality of the encoder. It is desirable to design a default processing method for IBC mode in order to prevent un-defined decoder behavior.

# Technical description

In this method, a default processing method for IBC mode is designed which uses a default value of 1 << (internalBitDepth - 1) as the prediction samples.

For the constraint of invalid IBC merge index, in this method, the size of IBC merge list is set to be fixed to maximum merge candidate number, which is 6 for VTM4.0 CTC. If the available IBC merge candidates less than 6, fill the merge list with zero block vectors. At the encoder, zero block vector candidates will not be tested. Zero block vectors and other invalid block vectors may be signaled to the decoder. The decoder first examines the validity of the signal block vectors. If a block vector is not valid, the default value, 1 << (internalBitDepth - 1), is used to fill the prediction values.

In VTM4.0, chroma IBC mode is sub-block based, each 2x2 chroma block inherits block vector from its collocated luma block. The encoder must check all of the prediction mode and block vectors of the collocated luma blocks. Only when collocated luma blocks are all coded with IBC mode and the block vectors are all valid, the chroma block can coded in IBC mode. In this contribution, CU based chroma IBC mode is presented to reduce the complexity. In this method, the whole chroma CU uses one block vector which is inherited from the collocated luma block using the center position of the chroma CU. If this block vector is invalid, the default value, 1 << (internalBitDepth - 1), is used to fill the prediction values.

# Simulation results

The simulation is based on VTM4.0 software. The anchor is generated by using VTM4.0 default configurations with IBC mode. Common test conditions for Class F are followed. Results are shown in the following tables:

## Test 1: Apply default processing method to IBC merge/skip mode

In this test, if the number of IBC merge candidates less than 6, add zero block vector candidates. When checking the first invalid block vector candidate, the default value 1 << (internalBitDepth - 1) is used as prediction. The other invalid block vector candidates and zero block vector candidates won’t be check in the encoder side.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | 0.01% | -0.07% | 99% | 95% |
| Class SCC 1080p | 0.00% | -0.04% | -0.04% | 81% | 94% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.04% | -0.05% | -0.13% | 97% | 96% |
| Class SCC 1080p | -0.03% | 0.03% | 0.01% | 78% | 80% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.60% | 0.13% | 96% | 94% |
| Class SCC 1080p | -0.04% | -0.10% | -0.11% | 78% | 78% |

## Test 2: Test 1 + Apply default processing method to IBC chroma mode

Based on test 1, in this test, for IBC chroma mode, if the inherited block vector of the chroma sub-block is invalid, use the default value 1 << (internalBitDepth - 1) as prediction.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | -0.01% | -0.11% | 99% | 96% |
| Class SCC 1080p | -0.02% | -0.04% | -0.05% | 81% | 94% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.04% | -0.08% | -0.14% | 96% | 95% |
| Class SCC 1080p | -0.04% | 0.00% | -0.01% | 78% | 79% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.19% | -0.69% | 0.53% | 82% | 93% |
| Class SCC 1080p | 0.06% | 0.14% | 0.12% | 82% | 92% |

## Test 3: Test 1 + Apply default processing method to CU based IBC chroma mode

Based on test 1, in this test, for IBC chroma mode, the whole chroma CU use one block vector which is inherited from the collocated luma block using the center position of the chroma CU.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.11% | -0.21% | 82% | 91% |
| Class SCC 1080p | -0.18% | -0.21% | -0.31% | 85% | 91% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.20% | -0.21% | 82% | 95% |
| Class SCC 1080p | 0.02% | -0.02% | -0.07% | 82% | 93% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.18% | 0.03% | 0.46% | 81% | 92% |
| Class SCC 1080p | 0.04% | -0.02% | -0.07% | 82% | 90% |

# Conclusions

This contribution presents a default processing method for IBC mode in order to prevent un-defined decoder behavior. In this contribution, a default processing method for IBC mode is designed which uses a default value as the prediction samples to prevent these un-defined decoder behaviors. The simulation result of “Test 1: Apply default processing method to IBC merge/skip mode” shows 0.00% gain in AI case, 0.03% gain in RA case and 0.04% gain in LB case in screen content coding materials. “Test 2: Test 1 + Apply default processing method to IBC chroma mode” shows 0.02% gain in AI case, 0.04% gain in RA case and 0.06% loss in LB case. “Test 3: Test 1 + Apply default processing method to CU based IBC chroma mode” shows 0.18% gain in AI case, 0.02% loss in RA case and 0.04% loss in LB case.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).