| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0317 |
| --- | --- |

| Title: | CE8-related: Simplification on IBC Merge/Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a simplification method for IBC merge/skip mode by reducing the number of potential IBC merge candidates. On top of VTM4.0, the simulation results show 0.02% loss in AI case, 0.00% loss in RA case and 0.00% loss in LB case in screen content coding materials.

# Introduction

In the 13th JVET meeting, IBC mode as an independent mode was adopted into VTM4.0. In the VTM4.0, IBC merge/skip mode and inter merge/skip mode use two independent merge lists. Sharing list for inter merge mode and IBC mode was introduced to VTM4.0 and VTM4.0.1 in order to parallelly process small merge/skip CUs. However, in the worst case, two merge list may still be generated for 4x4 CUs in a shared listed region, one for inter merge/skip mode and one for IBC merge/skip mode. Since these two merge lists can be generated parallelly, the cycle budget for a shared list region can still be meet. However, it is still desired to have a simplified merge list generation scheme to reduce the area size for hardware implementation and reduce cycles for software implementation. In this contribution, IBC merge list simplification method is proposed so that the complexity of IBC merge list generation is reduced in terms of number of potential candidates (from 11 to 7) and pruning (from 9 to 5).

# Technical description

## IBC Merge list in VTM4.0

In VTM4.0, the maximum number of IBC merge candidates is 6, the order to generate an IBC merge list is as below:

- Spatial candidates for blocks A1, B1, B0 and A0 (3 pruning required).

- If number of candidates less than 4, add B2 (2 pruning required).

- 5 HMVP candidates (4 pruning required).

- Pairwise candidate.

## Modification on IBC merge list

In this contribution, the spatial candidates of B0, A0, B2 and pairwise candidate are removed.

The order to generate an IBC merge candidate list is modified as below:

- Spatial candidates for blocks A1, B1 (1 pruning required)

- 5 HMVP candidates (4 pruning required).

## IBC merge/skip mode sharing list

In order to enable parallel processing small IBC merge/skip CUs, sharing list method is integrated in VTM4.0.1. In this method, the 32 and below pixels region shares the same IBC merge list. The IBC HMVP table is reset at the beginning of each CTU line.

The tickets number of these two fixes are ticket 193 and ticket 211.

# Simulation results

The simulation is based on VTM4.0 software. The anchor is generated by using VTM4.0 default configurations with IBC mode. Common test conditions for Class F are followed. Results are shown in the following tables:

## Test 1: IBC merge list simplification

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.03% | 0.09% | 0.03% | 99% | 99% |
| Class SCC 1080p | 0.02% | 0.01% | 0.00% | 99% | 100% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.12% | -0.09% | 100% | 99% |
| Class SCC 1080p | 0.00% | 0.00% | -0.07% | 100% | 99% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.46% | -0.14% | 100% | 97% |
| Class SCC 1080p | 0.00% | 0.01% | 0.01% | 100% | 100% |

## Test 2: fix for ticket 193 and ticket 211

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.06% | -0.01% | -0.15% | 99% | 100% |
| Class SCC 1080p | 0.06% | 0.01% | 0.01% | 82% | 99% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | 0.02% | -0.17% | 100% | 100% |
| Class SCC 1080p | -0.03% | 0.04% | 0.00% | 81% | 84% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.10% | -1.10% | 100% | 101% |
| Class SCC 1080p | 0.04% | 0.17% | 0.08% | 82% | 86% |

## Test 3: Test 1 + Test 2

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.08% | 0.06% | -0.09% | 98% | 101% |
| Class SCC 1080p | 0.08% | 0.05% | 0.03% | 81% | 99% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | -0.02% | 99% | 100% |
| Class SCC 1080p | 0.00% | 0.14% | 0.09% | 81% | 84% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.30% | -0.46% | 100% | 102% |
| Class SCC 1080p | 0.23% | 0.17% | 0.16% | 82% | 87% |

# Conclusions

This contribution presents a simplification method for IBC merge/skip modeby reducing the number of potential IBC merge candidates. On top of VTM4.0, the simulation results show 0.02% loss in AI case, 0.00% loss in RA case and 0.00% loss in LB case in screen content coding materials.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft text changes

Based on version 6 of JVTE-M1001, the newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough.

## Test 1: IBC merge list simplification

#### 8.6.2.3 Derivation process for spatial merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are as follows:

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvA0, mvA1, mvB0, mvB1 and mvB2 of the neighbouring coding units,

For the derivation of availableFlagA1 and mvA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- The variables availableFlagA1 and mvLA1 are derived as follows:

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0 and both components of  mvA1 are set equal to 0.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

mvA1 = MvL0[ xNbA1 ][ yNbA1 ]		(8-979)

For the derivation of availableFlagB1 and mvB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1.

- The variables availableFlagB1 and mvB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0 and both components of  mvB1 are set equal to 0:

  - availableB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

mvB1 = MvL0[ xNbB1 ][ yNbB1 ]		(8-980)

For the derivation of availableFlagB0 and mvB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0 and mvB0 are derived as follows:

  - If one or more of the following conditions are true, availableFlagB0 is set equal to 0 and both components of   mvB0 are set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvB0 = MvL0[ xNbB0 ][ yNbB0 ]		(8-981)

For the derivation of availableFlagA0 and mvA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0 and mvA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0 and both components of  mvA0 are set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvA0 = MvL0[ xNbA0 ][ yNbA0 ]		(8-982)

For the derivation of availableFlagB2 and mvB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2 and mvB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0 and both components of  mvB2 are set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB0 is equal to TRUE and the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableA0 is equal to TRUE and the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvB2 = MvL0[ xNbB2 ][ yNbB2 ]		(8-983)

#### 8.6.2.4 Derivation process for pairwise average merging candidate

Inputs to this process are:

- a merging motion vector candidate list mergeMvCandList,

- the number of elements numCurrMergeCand within mergeMvCandList.

Outputs of this process are:

- the merging motion vector candidate list mergeMvCandList,

- the number of elements numCurrMergeCand within mergeCandList.

The candidate mvAvgCand is derived as follows and numCurrMergeCand is incremented by 1:

mvAvgCand[ 0 ] = ( ( mergeMvCandList[ 0 ][ 0 ] + mergeMvCandList[ 1 ][ 0 ] ) / 32 ) << 4	(8-984)

mvAvgCand[ 1 ] = ( ( mergeMvCandList[ 0 ][ 1 ] + mergeMvCandList[ 1 ][ 1 ] ) / 32 ) << 4	(8-985)

The candidate mvAvgCand is added at the end of mergeMvCandList and numCurrMergeCand is incremented by 1 as follows:

mergeMvCandList[ numCurrMergeCand++ ] = mvAvgCand	(8-986)