| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0321 |
| --- | --- |

| Title: | CE3: Simplification of LM Mode (CE3-1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Hong-Jheng Jhu Xianglin Wang | Email: | yiwenchen@kwai.com jhuhong-jheng@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the results of CE3-1.6 on the simplification of LM modes. There are six sub-tests under this CE test. In the CE3-1.6.1, a modification to the cross-component linear model (CCLM) deriving method is presented. To determine the α and β for the CCLM model, currently in VVC the minimum and maximum luma samples are searched among the down-sampled top and left neighbouring reconstructed luma samples. α and β are then determined by the linear model formed from the minimum/maximum down-sampled luma samples and the corresponding chroma samples. In this proposal, it is proposed to search the minimum and maximum luma samples among the top and left neighbouring reconstructed luma samples without the down-sampling process. After the minimum and maximum luma samples are identified, their corresponding down-sampled luma samples are then generated to derive the α and β. Simulation results reportedly show BD-rates on Y, Cb and Cr components are 0.02%, 0.19%, and 0.22% for AI configuration over VTM-4.0, with 100% encoding time and 100% decoding time. The combination of the proposed simplification and the other CE tests which reduce the number of samples used to derive the LM model are also tested in CE3-1.6.2 to CE3-1.6.6. The simulation results reportedly show that the proposed simplification has the similar behavior in the combination tests in terms of coding efficiency.

# Introduction

In the current CCLM scheme, the minimum (min) and maximum (Max) luma values are searched among the neighboring down-sampled luma samples as depicted in the right part of Figure 1. After the min and Max luma samples are determined, the linear model parameters α and β are obtained according to the values of the min/Max luma samples(LumaMax,Lumamin) and the corresponding chorma samples (ChromaMax,Chromamin) using the following equation:

α=ChromaMax-ChromaminLumaMax-Lumamin, β=Chromamin-αLumamin

Figure 1: Down-sampled reconstructed luma samples (Rec’L) and the reconstructed chorma samples (RecC) used to derive α and β in VTM-4.0.

# Proposed Implementations for CE tests

In current CCLM, a six-tap filter is applied to derive down-sampled luma samples. To reduce the computational complexity, we propose to use the corresponding reconstructed luma samples directly (without down-sample filtering) to find the min/Max luma samples as shown in the right part of Figure 2. After the min/Max luma samples are identified, their corresponding down-sampled luma samples are then generated to derive the α and β.

Figure 2: Reconstructed luma samples (RecL) and the reconstructed chorma samples (RecC) that are used directly to derive α and β in the proposed scheme.

In this case, the down-sampling operations are greatly reduced to only two, with one for the minimum luma sample and one for the maximum luma sample. The number of operations for α and β derivation in the VTM-4.0 and the proposed schemes are illustrated in Table 1. As shown in this table, the numbers of “multiplications”, “sums” and “shift” are reduced from “2N+1” to 5, “6N+3” to 15 and “N” to 2, respectively. It is noted that N is at least 4 (2x2 chroma block) and the proposed modification could reduce the number of operations by around 97% for a 32x32 chroma block.

Table 1 The comparisons of number of operations in deriving α and β (N >=4 in VTM-3.0)

| Operations | Number of operations for α and β derivation |  |
| --- | --- | --- |
|  | VTM-3.0 | Proposed |
| Multiplications | 2N+1 | 4+1 |
| Sums | 6N+3 | 12+3 |
| Shift | N | 2 |
| “Division” | 1 | 1 |
| Comparison | 2N | 2N |

The proposed method is tested as CE3-1.6.1 and the combination tests of the proposed method with the other methods to reduce the number of samples are also tested in CE3-1.6.2 to CE3-1.6.6 as illustrated below. It is noted that in the CE3-1.6.6, no down-sampling process is performed at all because all the four points are required to derive the LM parameters and there is no point to perform down-sampling on the four points in this combined scheme.

CE3-1.6.1) the proposed method in JVET-M0229[1].

CE3-1.6.2) combination of CE3-1.6.1 and CE3-1.1.

CE3-1.6.3) combination of CE3-1.6.1 and CE3-1.2.

CE3-1.6.4) combination of CE3-1.6.1 and CE3-1.3.

CE3-1.6.5) combination of CE3-1.6.1 and CE3-1.4.

CE3-1.6.6) combination of CE3-1.6.1 and CE3-1.5.

# Results

The proposed modification (CE3-1.6.1) is integrated into VTM-4.0 [2] and the results of the proposed implementation under CTC [3] against VTM4.0 are shown in Table 1. The summary of all the sub-tests under CE3-1.6 is illustrated in Table 2. Moreover, the proposed modification (CE3-1.6.1) is also tested in the combination with the CE tests of MMLM (CE3-1.7 and CE3-1.8) and the results are also illustrated in Table 3. When compared to the results reported in CE3-.1.7 and CE3-1.8, the results also show that the proposed simplification introduces around 0.03%, 0.2% and 0.2% BD-rate increase for Y, Cb and Cr components under AI with significantly reduced computation complexity of MMLM.

Table 1. Performance of CE3-1.6.1

Table 2. Summary of the coding performance of CE3-1.6.1 to CE3-1.6.6

| # Tests | # AI | # RA | # LDB | # Enc. Time | # Dec. Time |
| --- | --- | --- | --- | --- | --- |
| # 3-1.6.1 | # 0.02%/0.19%/0.22% | # 0.02%/0.22%/0.14% | # -0.01%/0.56%/0.26% | # 100% | # 100% |
| # 3-1.6.2 | # 0.05%/0.32%/0.33% | # 0.02%/0.16%/0.16% | # 0.03%/0.33%/0.18% | # 100% | # 100% |
| # 3-1.6.3 | # -0.02%/-0.07%/-0.13% | # 0.00%/-0.18%/-0.27% | # 0.03%/0.33%/-0.30% | # 100% | # 100% |
| # 3-1.6.4 | # -0.02%/0.08%/-0.05% | # 0.00%/-0.02%/-0.13% | # 0.01%/0.02%/-0.12% | # 100% | # 100% |
| # 3-1.6.5 | # 0.01%/0.18%/0.13% | # 0.02%/0.11%/0.08% | # -0.01%/0.22%/0.22% | # 100% | # 100% |
| # 3-1.6.6 | # 0.00%/0.20%/0.08% | # 0.00%/0.20%/-0.01% | # -0.01%/0.44%/-0.02% | # 100% | # 100% |
| # Combined with 3-1.7 | # -0.23%/-1.29%/-1.82% | # -0.11%/-0.97%/-1.39% | # -0.03%/-0.16%/-0.16% | # 100% | # 100% |
| # Combined with 3-1.8.2 | # -0.22%/-0.37%/-0.73% | # -0.09%/-0.18%/-0.351% | # -0.02%/0.13%/-0.13% | # 100% | # 100% |

# Conclusion

In this proposal, a simplification on CCLM mode is proposed to reduce computational complexity. The modification provides around 97% reduction of down-sample filtering operations needed for a 32x32 chroma block, with 0.02%, 0.19% and 0.22 BD-rate increase for Y, Cb and Cr components respectively under AI condition. It is recommended to consider adoption of the proposed simplification into VVC for CCLM mode.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

- “ CE3-related: Simplification of LM Mode”, Y.-W. Chen, X. Wang, doc. JVET-M0229, January 2019.

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).