#### Derivation process for luma motion vectors for merge triangle mode

This process is only invoked when merge_triangle_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors mvLXA, and mvLXB

- the reference indices refIdxLXA and refIdxLXB

- the prediction list flags predFlagLXA and predFlagLXB

with X being 0 and 1 respectively.

The motion vectors mvLA and mvLB, the reference indices refIdxLA and refIdxLB and the prediction flags predFlagLA and predFlagLB are derived by the following ordered steps:

- The derivation process for merging candidates as specified in clause 8.5.2.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the refIdxLXN, predFlagLXN, mvLXN[ 0 ], mvLXN[ 1 ], and gbiIdxN from the mergeCandList with X being 0 or 1 and N being 0 to 5.

- The variables m and n, being the candidates in the merging candidate list mergeCandList, are derived using merge_triangle_idx0[ xCb ][ yCb ] and as merge_triangle_idx1[ xCb ][ yCb ] follows:

m = merge_triangle_idx0[ xCb ][ yCb ]		(8-491)

n = merge_triangle_idx1[ xCb ][ yCb ] + ( merge_triangle_idx1[ xCb ][ yCb ] >= m ) ? 1 : 0	(8-492)

- Set mvLXA, mvLXB to 0, set refIdxLXA, refIdxLXB to -1, set predFlagLXA and predFlagLXB to 0 with X being 0 and 1 respectivley.

- Set an index variable idx equal to 0;

If backward prediction is used,

for (int32_t i = 0; i < maxNumMergeCand; i++)

If predFlagL0 of mergeCandList[ i ] is equal to 1

If(idx==m), set mvL0A = MvL0mergeCandList[i], refIdxL0A = refIdxL0mergeCandList[i], predFlagL0A = 1;

If(idx==n), set mvL0B = MvL0mergeCandList[i], refIdxL0B = refIdxL0mergeCandList[i], predFlagL0B = 1;

Idx = idx + 1;

Else if predFlagL1 of mergeCandList[ i ] is equal to 1

If(idx==m), set mvL1A = MvL1mergeCandList[i], refIdxL1A = refIdxL1mergeCandList[i], predFlagL1A = 1;

If(idx==n), set mvL1B = MvL1mergeCandList[i], refIdxL1B = refIdxL1mergeCandList[i], predFlagL1B = 1;

idx = idx + 1;

Else (i.e. backward prediction not used),

for (int32_t i = 0; i < maxNumMergeCand; i++)

If predFlagL0 of mergeCandList[ i ] is equal to 1

If(idx==m), set mvL0A = MvL0mergeCandList[i], refIdxL0A = refIdxL0mergeCandList[i], predFlagL0A = 1;

If(idx==n), set mvL0B = MvL0mergeCandList[i], refIdxL0B = refIdxL0mergeCandList[i], predFlagL0B = 1;

idx = idx + 1;

for (int32_t i = 0; i < maxNumMergeCand; i++)

if predFlagL1 of mergeCandList[ i ] is equal to 1

If(idx==m), set mvL1A = MvL1mergeCandList[i], refIdxL1A = refIdxL1mergeCandList[i], predFlagL1A = 1;

If(idx==n), set mvL1B = MvL1mergeCandList[i], refIdxL1B = refIdxL1mergeCandList[i], predFlagL1B = 1;

idx = idx + 1;

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-